# CHAPTER 3: METHODOLOGY

# 3.1 System Description

This chapter presents the mathematical models and simulation methodology developed for the Solar-Assisted Heat Pump Dryer (SAHPD) system. The SAHPD is designed for drying apple slices in the climatic conditions of Nepal, with the objective of minimizing electrical energy consumption while maintaining consistent product quality. The system integrates three thermal energy sources: a vapor compression heat pump (R134a refrigerant), flat-plate solar air collectors, and a sensible heat recovery exchanger (HRX). Ten distinct system configurations are evaluated, each representing a different integration strategy for these components.

The dryer operates as a through-flow system with 10 trays arranged in series. Conditioned air enters at the top tray (Tray 1) and exits from the bottom tray (Tray 10), progressively picking up moisture from the product. The drying chamber geometry is determined by the product loading and target air velocity. The simulation advances in discrete timesteps of 60 seconds, updating air conditions, product moisture, and energy flows at each step until all trays reach the target moisture content.

## 3.1.1 Design Parameters

Table 3.1: Design parameters for the SAHPD system

| Parameter | Symbol | Value | Unit |
| --- | --- | --- | --- |
| Target drying temperature | T_set | 45 | °C |
| Air velocity (through trays) | v | 1.1 | m/s |
| Dry mass of product | m_p,dry | 3.0 | kg |
| Initial moisture content (d.b.) | X_0 | 6.5 | kg/kg |
| Final moisture content (d.b.) | X_f | 0.10 | kg/kg |
| Product (apple slices) | thickness | 6 | mm |
| Number of trays | N | 10 | - |
| Loading density | ρ_load | 3.6 | kg_fresh/m² |
| Timestep | Δt | 60 | s |
| Maximum simulation time | t_max | 72 | h |
| Refrigerant | - | R134a | - |

## 3.1.2 Drying Chamber Geometry

The drying chamber dimensions and air mass flow rate are derived from the product specifications rather than being specified independently. This ensures physical consistency between the product loading, air velocity, and mass flow rate.

The fresh mass of product loaded per tray is:

m_fresh = m_p,dry × (1 + X_0) / N    (3.1)

The tray area is determined from the loading density:

A_tray = m_fresh,tray / ρ_load    (3.2)

For the through-flow configuration, air flows horizontally through the gap between trays. The cross-sectional area for air flow is:

A_cross = √A_tray × δ_gap    (3.3)

where δ_gap = 0.12 m is the vertical gap between trays. The dry air mass flow rate is then:

ṁ_da = ρ_air × v × A_cross    (3.4)

The air density ρ_air is computed from the ideal gas law at the drying temperature and local atmospheric pressure:

ρ_air = P_atm / (R_da × T_set,K)    (3.5)

where P_atm is obtained from the standard atmosphere model based on site elevation:

P_atm = 101325 × (1 − 0.0065 × z / 288.15)^5.2561    (3.6)

# 3.2 System Configurations

Ten configurations are investigated, grouped into four families based on their thermal energy integration strategy. All configurations share the same drying chamber, product loading, and kinetics model. They differ only in how the inlet air is conditioned before entering the chamber.

## 3.2.1 Configuration A: Heat Pump Convective Dryer (Baseline)

Configuration A serves as the baseline heat pump convective dryer (HPCD). In the open-loop mode (r = 0), ambient air is drawn through the heat pump condenser, heated to the target temperature, passed through the drying chamber, and exhausted to the atmosphere. The heat pump evaporator draws heat from a separate ambient air stream.

Air flow path (open-loop):

Ambient → HP Condenser (→45°C) → Chamber → Exhaust

In this mode, the evaporator and condenser air streams are independent. The heat pump is sized to deliver the required condenser heat duty:

Q_cond = ṁ_da × (h_out − h_in)    (3.7)

where h_out and h_in are the moist air enthalpies at the condenser outlet (T_set) and inlet (T_amb), respectively.

## 3.2.2 Configuration B: Solar + Heat Pump Series

Configuration B introduces a flat-plate solar air collector in series before the heat pump condenser. The solar collector preheats ambient air, reducing the thermal lift required from the heat pump. During periods of high solar irradiance, the collector may heat the air to or above the target temperature, allowing the compressor to operate at reduced capacity or shut off entirely.

Air flow path:

Ambient → Solar Collector → HP Condenser (→45°C) → Chamber → Exhaust

The condenser heat duty is reduced to:

Q_cond = ṁ_da × c_p × max(T_set − T_solar,out, 0)    (3.8)

## 3.2.3 Configurations C1 and C2: Solar-Assisted Evaporator

Configurations C1 and C2 direct solar-heated air to the heat pump evaporator rather than the condenser. By raising the evaporator source temperature, these configurations increase the heat pump COP, reducing compressor work for the same condenser heat delivery. C1 and C2 differ in the mixing point of recirculated and ambient air relative to the solar collector:

Config C1: ambient air is mixed with recirculated air before the solar collector.

Config C2: ambient air passes through the solar collector first, then mixes with recirculated air before the evaporator.

## 3.2.4 Configurations D1, D2, and D3: Heat Recovery Exchanger + HP

The D-family configurations introduce a sensible heat recovery exchanger (HRX) between the warm, humid chamber exhaust and the cold, dry incoming ambient air. The HRX pre-warms the inlet air using waste heat from the exhaust, reducing the thermal load on the heat pump condenser. All D configurations operate in open-loop mode (r = 0) with no solar collector.

Config D1: Ambient air is heated by HRX, then by the HP condenser. Evaporator uses separate ambient air.

Config D2: Same as D1, but the evaporator air stream includes a dynamic ambient supplement derived from the exhaust composition.

Config D3: Air streams are swapped — exhaust passes through the HRX hot side then to the condenser; ambient passes through HRX cold side then to the evaporator.

Air flow path (D1):

Ambient → HRX(cold) → HP Condenser → Chamber → HRX(hot) → Exhaust

## 3.2.5 Configurations E1, E2, and E3: HRX + Solar + HP

The E-family configurations combine all three thermal recovery strategies: heat recovery, solar preheating, and heat pump conditioning. These represent the most thermally integrated designs.

Config E1: Ambient → HRX → Solar → HP Condenser → Chamber. Evaporator uses separate ambient air.

Config E2: Same condenser path as E1, but the evaporator air stream is supplemented with ambient air via an iterative sizing procedure.

Config E3: Solar collector heats the evaporator stream instead of the condenser stream. Ambient → HRX → HP Condenser → Chamber; Exhaust+Ambient → Solar → Evaporator.

Table 3.2: Summary of SAHPD configurations

| Config | Solar | HRX | HP | Key Mechanism |
| --- | --- | --- | --- | --- |
| A | No | No | Yes | HP-only baseline (open-loop) |
| B | Yes | No | Yes | Solar preheats condenser inlet air |
| C1 | Yes | No | Yes | Solar preheats evaporator air (mix before solar) |
| C2 | Yes | No | Yes | Solar preheats evaporator air (mix after solar) |
| D1 | No | Yes | Yes | HRX pre-warms condenser inlet; evap: ambient |
| D2 | No | Yes | Yes | HRX pre-warms condenser inlet; evap: dynamic mix |
| D3 | No | Yes | Yes | HRX streams swapped (exhaust to condenser) |
| E1 | Yes | Yes | Yes | HRX + Solar on condenser stream; evap: ambient |
| E2 | Yes | Yes | Yes | HRX + Solar on condenser stream; evap: dynamic mix |
| E3 | Yes | Yes | Yes | HRX + Solar on evaporator stream |

# 3.3 Mathematical Models

## 3.3.1 Psychrometric Model

Air properties at every point in the system are described by three interdependent state variables: dry-bulb temperature T (°C), humidity ratio ω (kg water per kg dry air), and relative humidity RH. All psychrometric calculations follow ASHRAE standards (ASHRAE, 2021).

Saturation vapor pressure (Tetens correlation, valid −40 to +50°C):

p_sat(T) = 610.94 × exp(17.625 × T / (T + 243.04))  [Pa]    (3.9)

Humidity ratio from temperature and relative humidity:

ω = 0.62198 × RH × p_sat(T) / (P_atm − RH × p_sat(T))    (3.10)

Moist air specific enthalpy (per kg dry air):

h = c_p,da × T + ω × (h_fg,0 + c_p,v × T)  [kJ/kg_da]    (3.11)

where c_p,da = 1.006 kJ/(kg·K) is the specific heat of dry air, c_p,v = 1.86 kJ/(kg·K) is the specific heat of water vapor, and h_fg,0 = 2501 kJ/kg is the latent heat of vaporization at 0°C.

Relative humidity from temperature and humidity ratio (inverse calculation):

RH = (ω × P_atm) / ((0.62198 + ω) × p_sat(T))    (3.12)

## 3.3.2 Solar Collector Model

The flat-plate solar air collector is modeled using the Hottel–Whillier–Bliss equation (Duffie and Beckman, 2013). The useful heat gain is:

Q_u = A_c × F_R × [η_opt × G − U_L × (T_in − T_amb)]    (3.13)

where the heat removal factor F_R is:

F_R = (ṁ_da × c_p) / (A_c × U_L) × [1 − exp(−F' × U_L × A_c / (ṁ_da × c_p))]    (3.14)

The collector outlet temperature is:

T_out = T_in + Q_u / (ṁ_da × c_p)    (3.15)

Table 3.3: Solar collector parameters

| Parameter | Symbol | Value | Unit |
| --- | --- | --- | --- |
| Optical efficiency | η_opt | 0.75 | - |
| Overall loss coefficient | U_L | 5.0 | W/(m²·K) |
| Collector efficiency factor | F' | 0.90 | - |
| Collector area | A_c | 0–20 | m² |
| Incidence angle modifier | K_θ | 1.0 | - |

## 3.3.3 Heat Pump Thermodynamic Cycle

The heat pump operates on a standard vapor compression cycle with R134a refrigerant. Thermodynamic properties at each state point are computed using the CoolProp library (Bell et al., 2014), which provides NIST-quality equations of state. The cycle consists of four processes:

(1) Evaporation with superheat: the refrigerant absorbs heat from the air stream (or ambient environment) at low pressure, exiting as superheated vapor at T_1 = T_evap + ΔT_sh.

(2) Compression: the superheated vapor is compressed to condenser pressure. The actual enthalpy rise is related to the isentropic enthalpy rise by the compressor isentropic efficiency:

h_2 = h_1 + (h_2s − h_1) / η_is    (3.16)

(3) Condensation with subcooling: the high-pressure refrigerant rejects heat to the drying air, exiting as subcooled liquid at T_3 = T_cond − ΔT_sc.

(4) Expansion: isenthalpic throttling through the expansion valve (h_4 = h_3).

The refrigerant mass flow rate is determined from the condenser heat duty:

ṁ_ref = Q_cond / (h_2 − h_3)    (3.17)

Energy flows and performance:

Q_evap = ṁ_ref × (h_1 − h_4)    (3.18)

W_comp = ṁ_ref × (h_2 − h_1) / η_mech    (3.19)

COP = Q_cond / W_comp    (3.20)

First law of thermodynamics (energy conservation):

Q_cond = Q_evap + W_comp × η_mech    (3.21)

Table 3.4: Heat pump parameters

| Parameter | Symbol | Value | Unit |
| --- | --- | --- | --- |
| Refrigerant | - | R134a | - |
| Compressor isentropic efficiency | η_is | 0.75 | - |
| Mechanical efficiency | η_mech | 0.95 | - |
| Superheat | ΔT_sh | 5 | K |
| Subcooling | ΔT_sc | 5 | K |
| Evaporator effectiveness | ε_evap | 0.85 | - |
| Condenser effectiveness | ε_cond | 0.85 | - |
| Evaporator coil approach | ΔT_approach | 3 | K |
| Min. evaporator temperature | T_evap,min | −5 | °C |
| Max. condenser temperature | T_cond,max | 70 | °C |
| Max. pressure ratio | PR_max | 10 | - |

### 3.3.3.1 Heat Pump Sizing: Open-Loop vs. Closed-Loop

Two distinct heat pump sizing approaches are used depending on whether the evaporator and condenser process the same or different air streams.

Open-loop sizing (Configs A(r=0), B, C1, C2, D, E):

In open-loop configurations, the condenser and evaporator air streams are independent. The heat pump is sized to deliver the required air heating duty:

Q_cond = ṁ_da × (h(T_set, ω_in) − h(T_in, ω_in))    (3.22)

The evaporator temperature is set based on the ambient (or source) air temperature with a fixed approach:

T_evap = T_source − ΔT_approach,evap    (3.23)

Closed-loop sizing (Config A with r > 0):

When exhaust air is recirculated, the same air stream passes through both the evaporator and condenser. The first law of thermodynamics (Eq. 3.21) creates a direct coupling: the condenser heat output is constrained by the evaporator heat absorption plus compressor work. The evaporator extracts heat from the warm, humid mixed air (recirculated exhaust + fresh ambient):

Q_evap,air = ṁ_da × (h_before,evap − h_after,evap)    (3.24)

Q_cond,1st law = Q_evap,air × COP / (COP − η_mech)    (3.25)

The actual condenser output is the minimum of the first-law limit and the heating requirement. When Q_cond,1st law is less than the heating requirement, the chamber inlet temperature floats below T_set.

### 3.3.3.2 Evaporator Air-Side Model

The evaporator uses an effectiveness-based model equivalent to the contact factor method of Chou and Chua (1994). The air outlet temperature is:

T_air,out = T_air,in − ε_evap × (T_air,in − T_coil)    (3.26)

where the coil surface temperature is:

T_coil = T_evap,sat + ΔT_approach    (3.27)

Dehumidification occurs when the air is cooled below its dewpoint. In this case, the outlet air exits at saturation:

If T_air,out < T_dp:  ω_out = ω_sat(T_air,out)  (saturated)    (3.28)

If T_air,out ≥ T_dp:  ω_out = ω_in  (no condensation)    (3.29)

### 3.3.3.3 Condenser Air-Side Model

The condenser performs sensible heating only (no moisture change). The air outlet temperature is limited by the condenser effectiveness:

T_air,out = T_air,in + ε_cond × (T_cond,sat − T_air,in)    (3.30)

where T_cond,sat = T_set + 10 K is the condenser saturation temperature. The air outlet temperature is clamped to T_set to prevent overheating. The humidity ratio is unchanged through the condenser (ω_out = ω_in).

## 3.3.4 Heat Recovery Exchanger (HRX)

The HRX is modeled as a counter-flow sensible heat exchanger that transfers heat from the warm chamber exhaust (hot side) to the cold incoming ambient air (cold side). Only sensible heat is exchanged through the solid wall; no moisture transfer occurs on the cold side. This is preferred over an enthalpy recovery ventilator (ERV) because moisture transfer from the humid exhaust back to the inlet air would be counterproductive for drying (Fadhel et al., 2011).

Outlet temperatures:

T_amb,heated = T_amb + ε_HRX × (T_exhaust − T_amb)    (3.31)

T_exh,cooled = T_exhaust − ε_HRX × (T_exhaust − T_amb)    (3.32)

If the exhaust stream is cooled below its dewpoint on the hot side, condensation occurs and the exhaust exits at saturation conditions. The HRX effectiveness is set to ε_HRX = 0.70, consistent with values reported for plate-type heat exchangers in drying applications (Jouhara et al., 2017).

## 3.3.5 VPD-Based Exhaust Bypass Strategy

For configurations D and E, a vapor pressure deficit (VPD) exhaust bypass strategy is implemented to improve energy efficiency during the late drying phase. When the product approaches equilibrium moisture, the exhaust air leaves the chamber relatively dry and warm. In this condition, the HRX transfers heat from exhaust air that has substantial drying potential remaining. The VPD bypass routes exhaust air directly to exhaust (bypassing the HRX) when drying potential is low, preventing unnecessary heat recovery from air that should be expelled.

The VPD utilization fraction is defined as:

VPD_util = (ω_exhaust − ω_inlet) / ω_inlet    (3.33)

The bypass engages when VPD_util falls below a threshold (typically 0.05), with a 3× hysteresis band and a 600-second dwell time to prevent oscillation.

# 3.4 Drying Kinetics Model

The drying kinetics are modeled using a parametric effective drying coefficient K_eff fitted from experimental data. The moisture ratio (MR) is defined as:

MR = (X − X_eq) / (X_0 − X_eq)    (3.34)

where X is the current moisture content (kg water/kg dry solid), X_0 = 6.5 is the initial moisture content, and X_eq is the dynamic equilibrium moisture content from the GAB isotherm.

## 3.4.1 GAB Sorption Isotherm

The equilibrium moisture content is computed using the temperature-dependent GAB (Guggenheim–Anderson–de Boer) model (Kaymak-Ertekin and Gedik, 2004):

X_eq = (X_m × C × K × a_w) / ((1 − K·a_w) × (1 − K·a_w + C·K·a_w))    (3.35)

where a_w = RH (water activity equals relative humidity at equilibrium), and X_m, C, K are temperature-dependent parameters with Arrhenius-type dependence. Typical values at drying conditions (45°C, 10–40% RH) yield X_eq ≈ 0.04–0.08 kg/kg d.b.

## 3.4.2 Parametric Effective Drying Coefficient

The effective drying coefficient K_eff is modeled as a log-linear function of temperature, relative humidity, and air velocity, fitted by ordinary least squares regression to 13 experimental drying curves from Phase 2:

ln(K_eff) = b_0 + b_1·T + b_2·RH + b_3·v + b_4·(T×RH)    (3.36)

Table 3.5: Parametric K_eff model coefficients (R² = 0.90)

| Coefficient | Value | Description |
| --- | --- | --- |
| b_0 | −11.58 | Intercept |
| b_1 | 0.0524 | Temperature effect [°C⁻¹] |
| b_2 | −1.876 | Relative humidity effect |
| b_3 | 0.611 | Air velocity effect [s/m] |
| b_4 | 0.0132 | T×RH interaction |

This parametric model provides smooth, continuous K_eff values across the operating envelope, naturally handling interpolation between experimental points.

## 3.4.3 Moisture Content Update

The moisture content of each tray is updated at each timestep using a first-order finite difference scheme:

X_{k+1} = X_k − K_eff(T, RH, v) × (X_k − X_eq) × Δt    (3.37)

subject to the constraint X_{k+1} ≥ X_eq. The actual water removal rate from each tray is the minimum of three physical limits:

(1) Kinetic limit: dṁ_w,kinetic = K_eff × (X − X_eq) × m_dry / Δt

(2) Air capacity limit: dṁ_w,air = ṁ_da × Δω_max  (prevents RH_out > 95%)

(3) Removable moisture: m_removable = (X − X_eq) × m_dry

# 3.5 Multi-Tray Chamber Model

The drying chamber consists of 10 trays arranged in a series cascade. Air enters at Tray 1 (hottest, driest air) and exits from Tray 10 (coolest, most humid). At each timestep, air conditions are updated sequentially through the tray stack:

Tray j: (T_j,in, ω_j,in) → drying → (T_j,out, ω_j,out) = (T_{j+1},in, ω_{j+1},in)

For each tray, the enthalpy balance accounts for the latent heat of evaporation:

h_out = h_in − (dṁ_w / ṁ_da) × h_fg    (3.38)

ω_out = ω_in + dṁ_w / (ṁ_da × Δt)    (3.39)

where h_fg = 2400 kJ/kg is the latent heat of vaporization at drying conditions. The outlet temperature is recovered from the enthalpy and humidity ratio using the inverse psychrometric relation (Eq. 3.11). The simulation terminates when all 10 trays satisfy X ≤ X_f = 0.10 kg/kg d.b.

Due to the cascade arrangement, Tray 1 dries fastest (~4–5 hours) while Tray 10 dries slowest (~15–17 hours). The total drying time is determined by the slowest tray.

# 3.6 Fan Power Model

Fan electrical consumption is computed from the air-side pressure drop through the drying chamber using the Darcy–Weisbach equation for each tray channel:

ΔP = f × (L/D_h) × (ρ × v² / 2) + K_minor × (ρ × v² / 2)    (3.40)

Fan electrical power:

W_fan = (ṁ_da × ΔP_total) / (ρ_air × η_fan)    (3.41)

where η_fan = 0.60 is the overall fan efficiency (motor + impeller). The fan power is typically 2–4% of the total electrical consumption.

# 3.7 Weather Data and Study Locations

Hourly weather data for each location is obtained from the PVGIS Typical Meteorological Year (TMY) database (European Commission Joint Research Centre), which provides representative hourly profiles based on ERA5 reanalysis data. Each TMY file contains 8,760 hourly records of ambient temperature, relative humidity, global horizontal irradiance (GHI), wind speed, and atmospheric pressure.

Table 3.6: Study locations and climatic characteristics

| Location | Elevation (m) | P_atm (Pa) | Climate Zone | Mean T (°C) | Mean RH (%) |
| --- | --- | --- | --- | --- | --- |
| Kathmandu | 1,350 | 86,120 | Subtropical highland | 18.4 | 65 |
| Biratnagar | 72 | 100,460 | Subtropical lowland | 24.5 | 78 |
| Taplejung | 1,820 | 81,600 | Temperate highland | 14.5 | 72 |

## 3.7.1 Seasonal Analysis

To assess system performance under varying climatic conditions, simulations are conducted for three representative seasons aligned with the Nepalese agricultural calendar:

Sharad/Autumn (October–November): Post-monsoon apple harvest and primary drying window. Moderate temperatures, declining humidity.

Hemanta/Winter (December–January): Cold, dry conditions with low solar irradiance. Stress test for heat pump performance.

Basanta/Spring (March–April): Warm, dry, high solar irradiance. Best-case scenario for solar-assisted configurations.

Seasonal weather files are extracted from the annual TMY data and re-indexed from hour 0 for simulation compatibility.

# 3.8 Performance Metrics

The following metrics are used to compare configurations:

Specific energy consumption (electrical):

SEC = (W_comp + W_fan) / m_w,removed  [kWh/kg]    (3.42)

Coefficient of performance:

COP = Q_cond / W_comp    (3.43)

Solar fraction:

SF = Q_solar / (Q_solar + Q_cond)  [−]    (3.44)

Energy savings relative to baseline:

ES = (SEC_A − SEC_x) / SEC_A × 100  [%]    (3.45)

Second-law (Carnot) efficiency:

η_II = COP / COP_Carnot    (3.46)

COP_Carnot = T_cond,K / (T_cond,K − T_evap,K)    (3.47)

# 3.9 Model Validation

The simulation model is validated through four independent consistency checks applied to all configurations and locations:

First law:  The first law of thermodynamics is verified at every timestep: Q_cond = Q_evap + W_comp × η_mech. The maximum observed error across all configurations is less than 10⁻⁶.

Water mass balance:  The cumulative water removed from all trays equals the sum of individual tray moisture decrements, with zero discrepancy.

Psychrometric consistency:  Temperature, humidity ratio, and relative humidity are verified for mutual consistency at every air state point (error < 4 × 10⁻⁶).

COP validation:  COP values range from 3.5 to 4.8, with Carnot efficiency η_II = 0.61–0.62, consistent with published values for R134a heat pump dryers (Chua et al., 2002).

# 3.10 Simulation Procedure

The simulation follows a quasi-steady-state approach where each 60-second timestep is solved as a sequence of steady-state component calculations. The procedure for each timestep is:

1. Read ambient weather conditions (T_amb, RH_amb, GHI) from the TMY file.

2. Compute inlet air conditions through the thermal conditioning chain (HRX, solar collector, HP components) according to the configuration.

3. Pass conditioned air through the 10-tray cascade, updating moisture content and air state at each tray.

4. Compute energy flows (Q_cond, Q_evap, W_comp, Q_solar, W_fan) and cumulative totals.

5. Check termination criterion (all trays at X ≤ 0.10).

6. Advance to next timestep or terminate.

The simulation is implemented in Python 3.11 using NumPy for numerical operations, CoolProp for refrigerant thermodynamic properties, and Pandas for data management. Hourly weather data is interpolated linearly between timesteps. All simulations are deterministic and reproducible.