# SAHPD Model: Air-HP Interaction & Recirculation Analysis

# 1. Air Property Calculations (Psychrometrics)

All air properties are computed from first principles using ASHRAE psychrometric relations. The key state variables at every point in the system are Temperature (T), Humidity Ratio (omega, kg water/kg dry air), and Relative Humidity (RH).

## 1.1 Saturation Vapor Pressure

The saturation vapor pressure of water at temperature T (in C) is computed using the Buck equation:

p_sat(T) = 611.21 * exp((18.678 - T/234.5) * T / (257.14 + T))  [Pa]

## 1.2 Humidity Ratio

From temperature and relative humidity:

omega = 0.622 * RH * p_sat(T) / (P_atm - RH * p_sat(T))  [kg/kg_da]

P_atm depends on elevation: Kathmandu (1350m) = 86,120 Pa; Biratnagar (72m) = 100,460 Pa.

## 1.3 Moist Air Enthalpy

Enthalpy per kg of dry air (ASHRAE):

h = 1.006*T + omega*(2501 + 1.86*T)  [kJ/kg_da]

This captures both sensible heat (1.006*T) and latent heat (omega*2501) of the moisture in air. The latent component is critical: humid air carries significantly more energy than dry air at the same temperature.

## 1.4 Dewpoint Temperature

The dewpoint is computed by inverting the saturation pressure equation:

p_v = omega * P_atm / (0.622 + omega)

T_dp = f_inverse(p_v)  [C]

If air is cooled below T_dp, moisture condenses out. This is how the evaporator dehumidifies.

# 2. Heat Pump Component Calculations

## 2.1 Vapor Compression Cycle (R134a)

The HP uses a standard 4-point vapor compression cycle computed with CoolProp (NIST-quality thermodynamic properties):

### State 1: Evaporator Outlet (Superheated Vapor)

T_1 = T_evap + DT_superheat  (5 + 5 = 10 C)

P_1 = P_sat(T_evap)  [from CoolProp]

h_1, s_1 = CoolProp(T_1, P_1)  [kJ/kg]

### State 2: Compressor Outlet (Superheated Vapor)

P_2 = P_sat(T_cond)  = P_sat(55 C)  [condenser pressure]

h_2s = CoolProp(P_2, s_1)  [isentropic compression]

h_2 = h_1 + (h_2s - h_1) / eta_isentropic  [actual, eta=0.72]

W_comp_specific = (h_2 - h_1) / eta_mechanical  [kJ/kg_ref, eta=0.90]

### State 3: Condenser Outlet (Subcooled Liquid)

T_3 = T_cond - DT_subcooling  (55 - 5 = 50 C)

h_3 = CoolProp(T_3, P_cond)  [kJ/kg]

Q_cond_specific = h_2 - h_3  [kJ/kg_ref]

### State 4: Expansion Valve Outlet (Two-Phase)

h_4 = h_3  [isenthalpic expansion]

Q_evap_specific = h_1 - h_4  [kJ/kg_ref]

## 2.2 Sizing the Refrigerant Flow

The refrigerant mass flow rate is sized to deliver the required Q_cond:

m_ref = Q_cond_target / (h_2 - h_3)  [kg/s]

Then all energy flows scale with m_ref:

Q_evap = m_ref * (h_1 - h_4)  [kW]

W_comp = m_ref * (h_2 - h_1) / eta_mech  [kW]

Q_cond = m_ref * (h_2 - h_3)  [kW]

COP = Q_cond / W_comp

The compressor is modeled as infinitely variable (no maximum displacement). Feasibility flags warn when Q_cond > 4 kW (1-ton AC limit).

## 2.3 First Law Enforcement

The HP first law requires: Q_cond = Q_evap + W_comp. In closed-loop mode (r > 0), the SAME air stream passes through the evaporator and then the condenser. This means Q_cond is constrained by how much heat the evaporator can extract from the air:

Q_evap_air = m_da * (h_before_evap - h_after_evap)  [kW]

Q_cond_1st_law = Q_evap_air * COP / (COP - eta_mech)

Q_cond_actual = min(Q_cond_1st_law, Q_cond_air_requirement, Q_cond_effectiveness)

When Q_cond_1st_law < Q_cond_air_requirement, the condenser CANNOT heat air to T_set (45 C). T_to_chamber floats below T_set. This is the physical root of the recirculation problem.

## 2.4 Evaporator Air-Side Model

The evaporator uses an effectiveness-based model (equivalent to the contact factor method of Chou & Chua, 1994):

T_out = T_air_in - epsilon_evap * (T_air_in - T_coil)

T_coil = T_evap_sat + DT_approach  (5 + 3 = 8 C default)

Dehumidification: if T_out < T_dewpoint of incoming air, the air exits saturated at T_out:

if T_out < T_dp:  omega_out = omega_sat(T_out)  [saturated]

else:             omega_out = omega_in          [no condensation]

epsilon_evap = 0.85 (fixed). T_evap_sat = 5 C (fixed target), modulated down to -5 C when T_air_in approaches T_coil (minimum 5K temperature difference maintained).

## 2.5 Condenser Air-Side Model

The condenser effectiveness (epsilon_cond = 0.85) limits the maximum air outlet temperature:

T_out_max = T_air_in + epsilon_cond * (T_cond_sat - T_air_in)

With T_cond_sat = 55 C and epsilon = 0.85, the condenser can reach 45 C whenever T_air_in < 37 C. The condenser does not add or remove moisture (sensible heating only, omega unchanged).

# 3. Config A: HP-Only Dryer

## 3.1 Open-Loop Air Path (r = 0)

Air flows in a single pass with no recirculation:

Ambient (8.8 C) --> CONDENSER --> 45 C --> CHAMBER --> Exhaust (discarded)

^

Separate ambient --> EVAPORATOR --> (heat absorbed, air discarded)

The evaporator and condenser process DIFFERENT air streams. There is no first-law coupling between them on the air side. The HP is sized freely to heat ambient air to T_set.

Q_cond = m_da * (h(45 C, omega_amb) - h(T_amb, omega_amb))

= 0.098 * (h(45) - h(8.8)) = 3.63 kW

T_evap = T_amb - 10 = -1.2 C  (external heat source)

T_to_chamber = 45 C  (ALWAYS reaches T_set)

## 3.2 Closed-Loop Air Path (r > 0)

A fraction r of exhaust is recirculated and mixed with (1-r) fresh ambient air. The SAME air stream passes through evaporator then condenser:

r*Exhaust + (1-r)*Ambient --> MIX --> EVAPORATOR --> CONDENSER --> CHAMBER

^                                                              |

|______________________________________________________________|

Step-by-step calculation (example: r=0.1, Kathmandu, t=1h):

1. MIXING:

T_mix = 0.1*35 + 0.9*8.8 = 11.4 C

omega_mix = 0.1*omega_exh + 0.9*omega_amb = 0.0063 kg/kg

2. EVAPORATOR:

T_coil = 5 + 3 = 8 C, but T_mix - T_coil = 3.4K < 5K minimum

--> Modulate: T_coil = 11.4 - 5 = 6.4 C, T_evap = 3.4 C

T_out = 11.4 - 0.85*(11.4 - 6.4) = 7.2 C

T_dp(omega=0.0063) = 3 C --> T_out > T_dp --> NO dehumidification

Q_evap = 0.098 * 1.006 * (11.4 - 7.2) = 0.41 kW  (sensible only)

3. FIRST LAW CONSTRAINT:

COP = 4.0 (at T_evap=3.4 C, T_cond=55 C)

Q_cond = Q_evap * COP/(COP - eta_m) = 0.41 * 4.0/3.1 = 0.53 kW

vs Q_cond needed for 45 C = 3.73 kW

Q_cond_actual = min(0.53, 3.73) = 0.53 kW  <-- FIRST LAW LIMITS

4. CONDENSER:

T_to_chamber = 7.2 + 0.53/(0.098*1.006) = 7.2 + 5.4 = 12.6 C

(vs 45 C at r=0)

## 3.3 The Negative Feedback Loop

At r=0.1 in cold climate (T_amb=8.8 C), a vicious cycle develops:

Low T_to_chamber (13 C)

--> Slow drying

--> Low T_exhaust (10 C)

--> Low T_mix (9 C)

--> Tiny Q_evap (0.4 kW)

--> Tiny Q_cond (0.5 kW)

--> Low T_to_chamber ...

The system gets stuck at 10-15 C. Drying takes 72+ hours and never completes. This is the "valley of death" at intermediate r values (0.1-0.7) in cold climates.

## 3.4 Why r >= 0.9 Works

At r=0.9, the mix is dominated by warm exhaust:

T_mix = 0.9*38 + 0.1*8.8 = 35.1 C

DeltaT = 35.1 - 8.0 = 27.1 K  --> large Q_evap

Also: omega_mix is high --> T_dp > T_coil --> DEHUMIDIFICATION occurs

--> Q_evap includes LATENT heat (much larger than sensible alone)

--> Q_cond is large --> T_to_chamber reaches 45 C

# 4. Config B: Solar + HP Series

## 4.1 Open-Loop Air Path (r = 0)

Ambient --> SOLAR COLLECTOR --> T_after_solar --> CONDENSER --> CHAMBER

^

Separate ambient --> EVAPORATOR (heat source)

Solar preheats the air before the HP condenser. If T_after_solar >= 45 C, the HP shuts off (W_comp = 0, free solar drying). Otherwise, the HP only boosts from T_after_solar to 45 C (smaller delta-T = less compressor work).

## 4.2 Closed-Loop Air Path (r > 0)

r*Exhaust + (1-r)*Ambient --> MIX --> EVAPORATOR --> SOLAR --> CONDENSER --> CHAMBER

^                                                                       |

|_______________________________________________________________________|

Key difference from Config A: the SOLAR COLLECTOR sits between evaporator and condenser. After the evaporator cools air to ~6 C, the solar collector reheats it (to 15-25 C during daytime) before it enters the condenser.

The first-law Q_evap is computed from the mix-to-after-evaporator enthalpy drop (same as Config A). But Q_cond_required is smaller because the condenser inlet is now T_after_solar (warmer). When Q_cond_required < Q_cond_1st_law, the HP does less work than the first law allows.

## 4.3 Why Config B Solves the Valley of Death

At intermediate r (e.g., r=0.2, Kathmandu):

Config A:  Mix(11 C) --> Evap(6 C) --> Cond --> 13 C  (STUCK)

Config B:  Mix(11 C) --> Evap(6 C) --> Solar(20 C) --> Cond --> 45 C  (WORKS)

The solar collector provides the "missing" thermal energy that the HP first law cannot deliver. During daytime, solar adds 10-30 C to the air temperature. At night, Config B behaves like Config A (slow drying), but daytime solar progress carries the batch to completion.

Result: ALL recirculation ratios converge in Config B (22/22 runs completed).

# 5. Recirculation Ratio Results

## 5.1 Config A (HP-Only)

Kathmandu (1350m, T_amb = 8.8 C):

| r | Time (h) | W_comp (kWh) | SEC (kWh/kg) | Converged |
| --- | --- | --- | --- | --- |
| 0.0 | 13.8 | 13.52 | 0.717 | Yes |
| 0.1 | 72.0 | 15.55 | 0.974 | No |
| 0.2 | 72.0 | 15.07 | 0.951 | No |
| 0.3 | 72.0 | 14.67 | 0.931 | No |
| 0.4 | 72.0 | 14.35 | 0.915 | No |
| 0.5 | 72.0 | 14.08 | 0.901 | No |
| 0.6 | 72.0 | 13.89 | 0.890 | No |
| 0.7 | 72.0 | 14.02 | 0.892 | No |
| 0.8 | 31.4 | 15.33 | 0.849 | Yes |
| 0.9 | 14.9 | 12.35 | 0.669 | Yes |
| 1.0 | 15.2 | 12.45 | 0.674 | Yes |

Biratnagar (72m, T_amb = 25 C):

| r | Time (h) | W_comp (kWh) | SEC (kWh/kg) | Converged |
| --- | --- | --- | --- | --- |
| 0.0 | 14.4 | 10.13 | 0.543 | Yes |
| 0.1 | 72.0 | 28.26 | 1.602 | No |
| 0.2 | 72.0 | 28.49 | 1.609 | No |
| 0.3 | 72.0 | 29.33 | 1.647 | No |
| 0.4 | 72.0 | 30.84 | 1.716 | No |
| 0.5 | 33.5 | 17.85 | 0.995 | Yes |
| 0.6 | 29.8 | 17.99 | 0.994 | Yes |
| 0.7 | 14.8 | 14.21 | 0.765 | Yes |
| 0.8 | 14.6 | 14.01 | 0.755 | Yes |
| 0.9 | 14.6 | 13.98 | 0.753 | Yes |
| 1.0 | 14.7 | 13.95 | 0.752 | Yes |

## 5.2 Config B (Solar 10m2 + HP)

Kathmandu (Solar 10m2):

| r | Time (h) | W_comp (kWh) | SEC (kWh/kg) | Converged |
| --- | --- | --- | --- | --- |
| 0.0 | 13.8 | 9.09 | 0.488 | Yes |
| 0.1 | 31.6 | 6.47 | 0.398 | Yes |
| 0.2 | 31.4 | 6.26 | 0.386 | Yes |
| 0.3 | 31.1 | 6.37 | 0.392 | Yes |
| 0.5 | 30.1 | 6.90 | 0.416 | Yes |
| 0.7 | 29.0 | 7.84 | 0.460 | Yes |
| 0.9 | 14.9 | 8.49 | 0.469 | Yes |
| 1.0 | 15.2 | 8.62 | 0.477 | Yes |

Biratnagar (Solar 10m2):

| r | Time (h) | W_comp (kWh) | SEC (kWh/kg) | Converged |
| --- | --- | --- | --- | --- |
| 0.0 | 14.4 | 5.18 | 0.287 | Yes |
| 0.1 | 27.8 | 9.50 | 0.548 | Yes |
| 0.2 | 27.7 | 9.62 | 0.553 | Yes |
| 0.5 | 26.9 | 10.91 | 0.618 | Yes |
| 0.7 | 14.8 | 7.76 | 0.431 | Yes |
| 0.9 | 14.6 | 7.59 | 0.423 | Yes |
| 1.0 | 14.7 | 7.60 | 0.423 | Yes |

Key finding: Config B achieves 100% convergence (22/22 runs) across all r values. Solar preheating eliminates the valley of death. Best SEC: KTM r=0.2 (0.386), BTN r=0.0 (0.287).

# 6. Fixed Energy / Fixed Time Comparison

## 6.1 At Fixed Energy (10 kWh electrical)

How much water is removed when the same electrical energy is consumed?

| r | Time to 10 kWh | Water (kg) | MR |
| --- | --- | --- | --- |
| 0.0 | 10.0 h | 18.80 | 0.032 |
| 0.1 | 39.4 h | 17.23 | 0.089 |
| 0.5 | 39.2 h | 17.34 | 0.075 |
| 0.8 | 11.8 h | 18.91 | 0.022 |
| 0.9 | 11.4 h | 18.98 | 0.022 |
| 1.0 | 11.6 h | 18.98 | 0.022 |

At r=0.1-0.7, the HP runs at very low power (~0.3 kW vs ~1.0 kW). It takes 39 hours to consume 10 kWh because Q_cond is throttled by the first law.

## 6.2 At Fixed Time (14 hours)

How much energy is consumed and water removed after 14 hours?

| r | W_total (kWh) | Water (kg) | MR | T_cham (C) |
| --- | --- | --- | --- | --- |
| 0.0 | 13.86 | 19.34 | 0.005 | 45.0 |
| 0.1 | 3.79 | 9.70 | 0.483 | 13.5 |
| 0.5 | 4.30 | 11.08 | 0.409 | 14.2 |
| 0.7 | 4.51 | 12.17 | 0.351 | 15.0 |
| 0.9 | 12.15 | 19.28 | 0.007 | 45.0 |
| 1.0 | 12.03 | 19.26 | 0.007 | 45.0 |

At 14h, r=0.1 has used only 3.79 kWh (27% of r=0.0) and removed only 50% of the water. The HP is effectively "starved" by the first-law constraint. The chamber temperature is 13.5 C instead of 45 C.

# 7. Key Physics Summary

1. The minimum thermal energy to evaporate 19.3 kg of water is fixed: Q_latent = m_w * h_fg = 19.3 * 2400/3600 = 12.9 kWh. This does not change with r.

2. What changes with r is HOW MUCH of the condenser heat goes to useful drying vs sensible losses (exhaust waste at r=0, evaporator re-cooling at r=1).

3. The HP first law (Q_cond = Q_evap + W_comp) creates a fundamental coupling in closed-loop mode that does not exist in open-loop mode.

4. In cold climates, intermediate r (0.1-0.7) mixes too much cold ambient air, starving the evaporator and creating a negative feedback loop.

5. Solar preheating (Config B) breaks this feedback by adding thermal energy between the evaporator and condenser, independent of the HP first law.

6. Literature support: Braun & Bansal (ORNL, 2022) showed vented and unvented HP dryers have fundamentally different Carnot limits. A 2019 Energies study found 39% COP degradation in cold-climate HP dryers with the same mechanism we observe.