# SAHPD Component Sizing

Reference: Config A (HP-only), Kathmandu, r = 0 — the coldest ambient of the three simulated sites, which drives peak HP duty. Every other configuration (B, C, D, E) has lower electrical demand, so this sizing is the worst-case envelope for purchasing decisions. HRX duty is taken from Config D1 (Kathmandu, eps_HRX = 0.70).

Data source: outputs/config_A/kathmandu/baseline_r0.0.csv and outputs/config_D1/kathmandu/hrx_eps0.70.csv (post 2026-04-08 iterative-evap bugfix, regenerated 2026-04-21).

## 1. Component sizing table

| # | Component | Quantity | Value from simulation | Design pick (with margin) |
| --- | --- | --- | --- | --- |
| 1 | Heat pump / reversible split AC | Peak Q_cond (heating duty) | 3.72 kW | ≥ 4.5 kW heating capacity (≈ 1.5 ton / 18,000 BTU/h) |
|  |  | Mean Q_cond | 3.53 kW |  |
|  |  | Peak W_comp (electric) | 1.07 kW | 230 V, 10 A single-phase circuit |
|  |  | Mean W_comp | 0.975 kW |  |
|  |  | Peak Q_evap (cooling) | 2.71 kW | ≈ 1 ton cooling equivalent |
|  |  | Refrigerant T_cond (fixed) | 55 °C | Within standard split AC envelope |
|  |  | Refrigerant T_evap target | 5 °C |  |
|  |  | Mean COP (simulation) | ~3.6 | Inverter, 5-star preferred |
| 2 | Supply air temperature | T_to_chamber | 45 °C (constant) | Set-point at controller |
| 3 | Airflow (blower / inline fan) | Mass flow m_da | 0.098 kg/s |  |
|  |  | Volumetric flow (ρ = 0.943) | ~374 m³/h | ≥ 450 m³/h axial or centrifugal inline fan |
|  |  | Total static pressure | 210 Pa | Fan curve ≥ 250 Pa at design CFM |
|  |  | Fan electric power | ~25 W continuous | Fan efficiency η ≥ 0.55 |
| 4 | Drying chamber | Inner cross-section | 0.79 m × 0.12 m (air gap) | 10 trays stacked, 6 cm pitch |
|  |  | Tray area (each) | 0.625 m² (≈ 0.79 × 0.79 m) | Perforated food-grade mesh |
|  |  | Chamber volume (trays + ducts) | ~0.8 m³ | Insulate walls (50 mm polyurethane) |
| 5 | Exhaust duct | Peak T_exhaust | 44.7 °C | Ø 150 mm flexible duct, short run |
|  |  | Supply humidity ratio ω | 0.0074 | Dry supply, within expected envelope |
| 6 | Heat-recovery exchanger (HRX) | Peak Q_HRX | 2.58 kW | Counter-flow air-to-air plate, ε ≥ 0.70 |
|  |  | Mean Q_HRX | 1.81 kW | Rated ≥ 3 kW @ 400 m³/h each side |
|  |  | ΔT hot side | 3–26 K (mean 18 K) | Equal supply + exhaust ducting |
|  |  | Face volumetric flow per side | ~400 m³/h |  |

## 2. Purchasing checklist (AC unit)

Nominal cooling 18,000 BTU/h AND heating capacity ≥ 4.5 kW at 7 °C outdoor.

Reversible heat-pump (4-way valve); NOT a cooling-only unit.

DC inverter compressor (capacity modulation).

Refrigerant R32 preferred over R410A (lower GWP, single-component, matches modelling).

Ask whether evaporator and condenser can be mounted together inside the dryer loop with a custom fan (critical for research instrumentation).

## 3. Nepal-market options (1.5-ton reversible inverter split)

All widely sold in Kathmandu. Confirm current nameplate heating capacity and availability with the retailer before purchase.

| Model | Notes | Where to check |
| --- | --- | --- |
| Midea MSAGC-18HRFN1 1.5-ton 5-star | 18,000 BTU/h cool, 20,000 BTU/h heat, full-DC inverter — closest match | Trade Nepal, Electrosewa |
| Stellar SA-15IS1HPA/HE 1.5-ton | Built for Nepali winters, labelled cooling + heating | Himelectronics (Stellar) |
| LG 1.5-ton Dual Inverter (wall mount) | Common 1.5-ton inverter; confirm SKU supports heating mode | AC Ghar, Daraz |
| Gree 1.5-ton wall mount | Regularly available; verify reversible model | AC Ghar |
| CG 1.5-ton DC Inverter wall-mount | Local brand, serviceable parts | AC Ghar |
| Samsung AR18TSHZRWKNRC | 5-in-1 convertible, DC inverter, reversible | Trade Nepal |

## 4. Heat-recovery exchanger (HRX) sourcing

Counter-flow air-to-air plate HRX units are not commonly stocked in the Nepali market. Two realistic sourcing paths:

Import a residential HRV core (sensible-only, NOT ERV) sized 300–500 m³/h at ≥ 70 % effectiveness. Suppliers: ERI Corporation (Italy), Heatex (Sweden), Holtop (China), Airwoods (China), Zeco Aircon (India). Zeco is the closest option logistically.

Fabricate locally: a workshop in Balaju can build a 0.5 × 0.5 × 0.3 m counter-flow box from brazed aluminium or corrugated polypropylene plates. Target ≥ 2 m² plate area to hit ε = 0.70 at 400 m³/h. Validate effectiveness with a thermocouple traverse before use.

Avoid ERVs with enthalpy wheels: moisture transfer across the plate would break the sensible-only assumption in the Config D/E simulations and confound the comparison.

## 5. Instrumentation to budget for

PID thermostat with supply-air sensor (override the AC's stock room sensor).

Four T/RH sensors minimum (SHT35 or equivalent): ambient, chamber inlet, exhaust, evaporator inlet.

Two clamp-on kWh meters: one for compressor, one for fan, to validate SEC.

Pressure taps + manometer across evaporator and condenser coils (verify the 80 Pa each assumed by the fan-power model).