3. Mathematical model

All ten configurations share the same component-level model. Only the connectivity (described in §2) and a small number of configuration-specific control rules differ. Refrigerant properties are evaluated through CoolProp; moist-air properties from a Tetens-based psychrometric library; the drying kinetics from the parametric M1 model fitted on 13 thin-layer apple-drying curves (§3.4).

3.1 Vapour-compression cycle

The heat pump uses R134a as the working fluid. Each time step solves the four state points of a single-stage cycle with fixed superheat ΔTsh = 5 K at the evaporator outlet and fixed sub-cooling ΔTsc = 5 K at the condenser outlet. Compressor isentropic efficiency ηis = 0.75 and mechanical efficiency ηm = 0.95 are held constant.

State 1 (evaporator outlet, superheated vapour):

T₁ = Tevap + ΔTsh, P₁ = Psat(Tevap), h₁ = h(T₁, P₁) [Eq. 3.1]

State 2s (isentropic compression endpoint):

P2s = Psat(Tcond), s2s = s₁, h2s = h(P2s, s_2s) [Eq. 3.2]

State 2 (actual compressor outlet):

h₂ = h₁ + (h2s − h₁) / ηis [Eq. 3.3]

State 3 (condenser outlet, sub-cooled liquid):

T₃ = Tcond − ΔTsc, h₃ = h(T₃, P_cond) [Eq. 3.4]

State 4 (expansion-valve outlet, two-phase):

h₄ = h₃ (isenthalpic), x₄ = (h₄ − hf) / (hg − h_f) [Eq. 3.5]

The refrigerant mass flow is sized to the air-side condenser duty Qcondtarget:

mref = Qcond_target / (h₂ − h₃) [Eq. 3.6]

Energy flows and COP follow:

Qevap = mref (h₁ − h₄), Wcomp = mref (h₂ − h₁) / ηm, COP = Qcond / W_comp [Eq. 3.7]

Operating envelope: −5 °C ≤ Tevap ≤ 20 °C, 30 °C ≤ Tcond ≤ 70 °C, pressure ratio ≤ 10. Cases that fall outside are flagged but not clipped, so unphysical operating points are visible in the results.

For sizing, the condenser pinch is fixed at +10 K above the air-outlet target and the evaporator approach at −10 K below the heat-source temperature. Tevap ≥ Tcond is treated as a hard fault and a 1 K minimum lift is enforced so CoolProp does not return a degenerate cycle.

3.2 Solar air collector

The flat-plate solar air collector is modelled with the Hottel-Whillier-Bliss steady-state form. Useful gain per time step is

Quseful = Ac · FR · [ηo · Kθ · G − UL · (Tin − Tamb)] [Eq. 3.8]

with collector area Ac (baseline 10 m²), optical efficiency ηo = 0.75 (transmittance-absorptance product), incidence-angle modifier Kθ = 1.0 (assumes a south-tilted reference orientation), and overall loss coefficient UL = 5 W m⁻² K⁻¹. The heat-removal factor F_R follows the standard ε-NTU form for an air collector,

FR = (Cmin / UA) · [1 − exp(−UA · F′ / C_min)] [Eq. 3.9]

with collector efficiency factor F′ = 0.90 and capacity rate Cmin = ṁair c_p,air. The outlet air temperature is

Tout = Tin + Quseful / (ṁair c_p,air) [Eq. 3.10]

Stagnation (ṁair = 0 or G < 10 W m⁻²) collapses Eq. 3.8 to zero useful gain. Absorber-plate temperature is tracked with first-order thermal inertia (Ccollector = 10 kJ K⁻¹) so that abrupt irradiance steps do not produce instantaneous outlet-temperature jumps:

Tabs(t+Δt) = (1 − α) Tabs(t) + α Tabs,ss, α = Δt / (τ + Δt), τ = Ccollector / (Ac UL / 1000) [Eq. 3.11]

A stagnation cap of T_abs ≤ 150 °C is enforced as a numerical safeguard.

3.3 Counter-flow heat-recovery exchanger (HRX)

Configurations D1, D2, D3, E1, E2, and E3 include a flat-plate counter-flow air-to-air HRX between the chamber exhaust and the ambient inlet. The HRX is modelled as a single-effectiveness device with ε_HRX = 0.70 (consistent with commercial polymer-plate units of comparable size). For air streams of equal mass flow,

Tamb,heated = Tamb + εHRX (Texhaust − T_amb) [Eq. 3.12]

Texh,cooled = Texhaust − εHRX (Texhaust − T_amb) [Eq. 3.13]

with the cold-side outlet humidity ratio held equal to the cold-side inlet (no mass transfer across the HRX plates is modelled; condensate that forms on the hot side is removed but does not cross to the cold side).

3.4 Drying kinetics (M1 parametric Arrhenius model)

The drying-chamber moisture content evolves through a first-order kinetic law,

dX/dt = −Keff(T, RH, v, d) · (X − Xeq) [Eq. 3.14]

discretised on each time step as

X(t+Δt) = X(t) − Keff (X(t) − Xeq) Δt, X(t+Δt) ≥ X_eq [Eq. 3.15]

The effective rate constant K_eff is the M1 parametric form fitted by single-stage non-linear least squares to thirteen PAVA-cleaned thin-layer apple-drying MR(t) curves drawn from the project's experimental dataset:

Keff(T, RH, v, d) = Kref · exp[(Ea/R)(1/Tref − 1/T)] · exp(−αRH · RH/100) · (v / vref)^γv · (dref / d)^δ_d [Eq. 3.16]

with reference state Tref = 50 °C, vref = 1.1 m s⁻¹, d_ref = 6 mm, and fitted parameters

K_ref = 2.097 × 10⁻⁴ s⁻¹

Ea/R = 3738 K (Ea = 31.08 kJ mol⁻¹, profile 95 % CI [28.56, 33.60] kJ mol⁻¹)

α_RH = 1.965

γ_v = 0.401

δ_d = 0.589

Fit residual RMSE on MR is 0.04685 across all thirteen curves (nobs = 386). The activation-energy CI overlaps the published apple-specific convective range cited in §1.2 (14.47–22.62 kJ mol⁻¹) at its lower end; the upper end of our CI exceeds that range and is consistent with the broader 12–83 kJ mol⁻¹ envelope reported by Erbay & Icier (2010) across 41 food products. A leave-one-curve-out cross-validation (LOCO-CV, n = 13) returned mean RMSEMR = 0.0528 for M1, 0.0404 for an Arrhenius+Midilli alternative (M2), and 0.0685 for a piecewise-linear ElasticNet baseline (M3). M1 and M2 differ within their bootstrap confidence interval; both significantly outperform M3. M1 is used as the operational kinetic model and M2 is used as the sensitivity bracket in §5.

3.5 GAB sorption isotherm

Equilibrium moisture content Xeq at chamber temperature T and relative humidity RH (water activity aw = RH) follows the three-parameter GAB form,

Xeq = (Xm C K aw) / [(1 − K aw)(1 − K aw + C K aw)] [Eq. 3.17]

with temperature-dependent parameters

Xm(T) = X{m,0} exp(ΔHxm / R T), C(T) = C0 exp(ΔHC / R T), K(T) = K0 exp(ΔH_K / R T) [Eq. 3.18]

The constants (X{m,0} = 3.141 × 10⁻³ kg kg⁻¹ db, ΔHxm = 8 057 J mol⁻¹, C0 = 4.923 × 10⁻³, ΔHC = 17 241 J mol⁻¹, K0 = 0.9904, ΔHK ≈ 0 J mol⁻¹) were taken from a pooled dataset for apple desorption (Kaymak-Ertekin & Gedik 2004; Maroulis et al. 1988; Mbarek & Mihoubi 2019), giving an RMSE of 0.005 kg kg⁻¹ db across 28 experimental points spanning 30–60 °C. To prevent a singularity as a_w → 1/K, the water activity is clamped at 0.95 within the simulation.

3.6 Psychrometrics and moist-air enthalpy

Saturation vapour pressure follows the Tetens correlation (over liquid water for T ≥ 0 °C, over ice otherwise). Humidity ratio, relative humidity, and dew-point temperature are computed at the local atmospheric pressure of each site (Biratnagar 100 460 Pa; Kathmandu 86 120 Pa; Taplejung 81 000 Pa), so the psychrometric chart shifts correctly with altitude.

Moist-air specific enthalpy is

h = c{p,da} T + ω (h{fg,0} + c_{p,v} T) [Eq. 3.19]

with c{p,da} = 1.006 kJ kg⁻¹ K⁻¹, c{p,v} = 1.86 kJ kg⁻¹ K⁻¹, and h_{fg,0} = 2 501 kJ kg⁻¹. Adiabatic mix nodes (Mix(a + b) in §2) are solved by mass-and-enthalpy balance:

ṁmix hmix = ṁa ha + ṁb hb, ṁmix ωmix = ṁa ωa + ṁb ωb [Eq. 3.20]

A constant-enthalpy humidification with a small liquid-water enthalpy correction (hout = hin + Δω · 4.186 · TinC) is used inside the chamber so the energy released when liquid water enters the air stream is accounted for; this correction was added after the 2026-04-09 first-law audit revealed that ignoring it produced a Qcond − (Qevap + W_comp) imbalance of order 10⁻³.

The chamber-outlet RH is bounded by RH_out,max (configurable, default 0.95) so that the air is never asked to pick up more moisture than it can carry; the kinetic step (§3.4) and the air-capacity step are taken as a minimum and the simulation reports both.

3.7 Configuration-specific control overlays

Three configurations apply additional control rules on top of the shared component model:

Iterative evaporator-supply sizing (D2, E2, E3). The ambient supplement that is mixed with the recovered exhaust before the evaporator is solved by a fixed-point iteration so that Qevap matches Qcond − W_comp under the first-law constraint at the current operating point; convergence tolerance is 10⁻³ on mass-flow ratio.

Solar-priority HP control (E3 only). If the post-collector air alone reaches Tset = 45 °C (i.e. QHRX + Qsolar already covers the chamber duty), the heat pump is switched off for that time step and the collector is allowed to finish the temperature lift. Otherwise the heat pump runs at a variable Tcond chosen so that the post-collector stream exits at exactly T_set, and the collector tops up the residual lift.

VPD-triggered exhaust bypass (all ten configs). As described in §2.5, the bypass valve opens when the chamber-outlet vapour-pressure deficit drops below 0.05 kPa, and closes after a 3× hysteresis margin (0.15 kPa) and a 600 s dwell timer. The same threshold and timing are used uniformly across the ten topologies.

3.8 First-law enforcement and validation

The condenser-side air-heating duty is taken as the binding constraint on every time step:

Qcondtarget = ṁair (hair,outset − hair,in) [Eq. 3.21]

with hair,in evaluated at the actual condenser inlet (which depends on the topology) and hair,outset evaluated at Tset with the inlet humidity ratio. The compressor speed (and hence mref) is sized to deliver Qcondtarget exactly. Qevap is then computed from Eq. 3.7 and matched to the evaporator-side air capacity by either (i) the parallel ambient draw (A r=0, B r=0, D1, E1), (ii) the dynamic ambient supplement (D2), or (iii) the iterative supplement (E2, E3). At every time step the first-law residual

εFL = Qcond − (Qevap + Wcomp) [Eq. 3.22]

is logged. The 2026-04-09 model-validation pass (Configs A, B, D1, D2, E1, E2 across all three sites) returned |εFL| < 10⁻⁶ kW on every time step, alongside |Σ dmw − mwcum| < 10⁻⁶ kg for the water mass balance and |ωcalc − ωpsychro| < 4 × 10⁻⁶ for the psychrometric consistency check. COP values were 3.5–4.8 with Carnot efficiency 0.61–0.62 across configurations, consistent with η_is = 0.75 and the operating envelope of §3.1, and no frost or impossible-cycle flags were raised.