# Kinetics Explained

M1 drying-kinetics model: data, fit, and runtime evaluation in the SAHPD RQ1 simulator

# 1. Overview

The kinetics module has two distinct phases. A one-time fit happens once per Python process, taking 13 thin-layer experimental drying curves and condensing them into five physical parameters. A per-step evaluation then plugs the live air state at each tray into those cached parameters to compute the rate constant K, which drives the first-order moisture update. The fit runs once. The evaluation runs ~10^5 times per simulation.

# 2. The 13 experimental curves

Each row of outputs/phase2/phase2_targets.csv lists one thin-layer drying run at a fixed operating point (T, v, thickness, RH). Together they span four physical axes:

| dataset | T (°C) | v (m/s) | d (mm) | RH_mid (%) | axis |
| --- | --- | --- | --- | --- | --- |
| T_40_v1p1 | 40 | 1.1 | 6 | 42.5 | T sweep |
| T_45_v1p1 | 45 | 1.1 | 6 | 42.5 | T sweep |
| T_50_v1p1_tempsweep | 50 | 1.1 | 6 | 42.5 | T sweep |
| T50_v0p60 | 50 | 0.6 | 6 | 42.5 | v sweep |
| T50_v0p85 | 50 | 0.85 | 6 | 42.5 | v sweep |
| T50_v1p1_velsweep | 50 | 1.1 | 6 | 42.5 | v sweep |
| T50_v1p1_t_4mm | 50 | 1.1 | 4 | 42.5 | d sweep |
| T50_v1p1_t_6mm | 50 | 1.1 | 6 | 42.5 | d sweep |
| T50_v1p1_t_8mm | 50 | 1.1 | 8 | 42.5 | d sweep |
| T50_v1p1_t_10mm | 50 | 1.1 | 10 | 42.5 | d sweep |
| T50V1p1_RH_25-28% | 50 | 1.1 | 6 | 26.5 | RH sweep |
| T50V1p1_RH_35-38% | 50 | 1.1 | 6 | 36.5 | RH sweep |
| T50V1p1_RH_40-45% | 50 | 1.1 | 6 | 42.5 | RH sweep |

Reference state (T_ref = 50 °C, v_ref = 1.1 m/s, d_ref = 6 mm) is the intersection point that appears in every sub-sweep. Total observations across the 13 curves: 1,573 rows of (time_min, X_db).

A raw row looks like:

time_min, X_db
1.92,     6.497
5.20,     6.407
9.77,     6.326
13.70,    6.236

# 3. PAVA isotonic smoothing

For each curve the simulator first computes MR_raw(t) = X_db(t) / X_db(0) and clips it to [0, 1.1]. Sensor noise (dust, weighing wobble) can produce small up-ticks where MR locally rises. The first-order model requires MR to be monotone decreasing, so the Pool-Adjacent-Violators Algorithm (PAVA) pools any non-monotonic blip with its neighbours and replaces them with their weighted average until the sequence is monotone. The averages are preserved; the noise is removed.

Implementation: kinetics.py:_pava_monotone (lines 119–136).

# 4. The M1 model

M1 is a five-parameter, first-order Arrhenius form with explicit RH, velocity, and thickness corrections:

K(T, RH, v, d) = K_ref · exp((Ea/R) · (1/T_ref − 1/T))
                       · exp(−α_RH · RH/100)
                       · (v / v_ref)^γ_v
                       · (d_ref / d)^δ_d

MR(t) = exp(−K · t_seconds)

Five unknowns: K_ref, Ea/R, α_RH, γ_v, δ_d. Everything else (T, v, d, RH, t) is measured. The reference constants T_ref = 50 °C, v_ref = 1.1 m/s, d_ref = 6 mm are hard-coded in kinetics.py:111–113.

Concrete row example. Take T=45 °C, v=1.1, d=6, RH=42.5% at t = 5.20 min:

v/v_ref = 1 → velocity term vanishes
d_ref/d = 1 → thickness term vanishes

K_45 = K_ref · exp((Ea/R) · (1/323.15 − 1/318.15)) · exp(−α_RH · 0.425)
     = K_ref · exp(−4.86e-5 · (Ea/R))            · exp(−0.425·α_RH)

MR_model = exp(−K_45 · 312)        # 5.20 min = 312 s
MR_obs   = 6.407 / 6.497 = 0.986

residual = MR_model − MR_obs       (one number per row)

Five unknowns, one residual per measurement, 1,573 measurements across all 13 curves.

# 5. The curve fit (nonlinear least squares)

scipy.optimize.least_squares with method 'trf' (trust-region reflective). The five parameters are stacked into a vector p = (ln K_ref, Ea/R, α_RH, γ_v, δ_d). The initial guess is _M1_NOM = (ln 1.9e-4, 2711, 1.75, 0.44, 0.66) and the parameters live inside the box [_M1_LO, _M1_HI] defined in kinetics.py:114–116.

Each iteration:

1. Pick current p.
2. For every observation across all 13 curves, compute r_i(p) = MR_model_i(p) − MR_obs_i.
3. Concatenate all residuals into one vector R(p) of length 1,573.
4. Compute the Jacobian J = ∂R/∂p (numerical by default).
5. Solve the Gauss-Newton step δp = (JᵀJ)⁻¹ JᵀR, project onto the box, scale by a trust-region radius.
6. Apply, recompute R(p), grow or shrink the trust region depending on whether the step helped.
7. Stop when xtol and ftol (both 1e-12) say further moves do not reduce ‖R‖² meaningfully.

Critical detail: each curve does not get its own K. The same five-parameter formula must explain every curve simultaneously, which is what forces the parameters to take physical meaning. Fitting one curve at a time would give 13 independent K values; this fit gives one function K(T, RH, v, d) that reproduces all 13.

# 6. What the fit discovered

| Parameter | Value | Meaning |
| --- | --- | --- |
| K_ref | 2.097 × 10⁻⁴ s⁻¹ | Rate at ref state (50 °C, 1.1 m/s, 6 mm, RH=0) |
| Ea/R | 3,738 K | Ea = 31.08 kJ/mol; T sensitivity |
| α_RH | 1.965 | Drying ×0.14 when RH=1.0 vs RH=0 |
| γ_v | 0.401 | Drying ×(v/1.1)^0.4 with velocity |
| δ_d | 0.589 | Drying ×(6/d)^0.59 (thinner = faster) |
| RMSE_MR | 0.04685 | Mean prediction error across 1,573 points |

Physical reading of each parameter:

• Characteristic time at the reference state: τ = 1/K_ref ≈ 79 min for a thin layer.
• +10 °C (40 → 50): K speeds up by exp(3738 · (1/313.15 − 1/323.15)) = exp(0.369) ≈ 1.45×.
• RH 20% → 80%: K drops by exp(−1.965 · 0.6) = 0.308 → 3.2× slower.
• Doubling velocity: 2^0.401 ≈ 1.32× faster (modest; above 0.5 m/s the boundary layer is already thin).
• Halving thickness (6 → 3 mm): 2^0.589 ≈ 1.5× faster (shorter internal diffusion path).
• RMSE_MR = 0.047 over a 0–1 scale = 5% mean error; acceptable for five parameters explaining 1,573 measurements across four physical axes.

# 7. Confidence in the fit

The fit also reports σ² = SS_res / (n_obs − n_par) = 2.20 × 10⁻³. From the profile likelihood:

• 95% CI for Ea: [28.56, 33.60] kJ/mol — overlaps the published apple-drying range (19.96 – 30.93 kJ/mol from Aktaş, Mishra, et al.). The values are physically plausible, not curve-fitting artefacts.
• Leave-one-curve-out cross-validation: RMSE_MR = 0.0528 on held-out curves vs 0.0469 on the training set. ~13 % degradation — no overfitting collapse.

# 8. How the simulator uses K, every step

At each tray, every 60 s of sim time, with the current inlet air state (T_air, RH_air) and the cached parameters:

ln K = ln(2.097e-4)
     + 3738 · (1/323.15 − 1/T_K)        # T term
     − 1.965 · RH_frac                  # RH term
     + 0.401 · ln(1.1 / 1.1)            # = 0 (v matches v_ref)
     + 0.589 · ln(6 / 6)                # = 0 (d matches d_ref)
K  = max(K_min, exp(ln K))

ΔX = K · (X − X_eq) · 60                # X_eq = 0 since GAB is off
X_new = max(X − ΔX, X_eq)
dm_w_kinetic = m_p_dry · ΔX             # kg water removed by kinetics

Worked examples at the two extremes seen in the SAHPD chamber:

Tray 0 inlet, T=45 °C, RH=0.30:

K = 2.097e-4 · exp(3738·(1/323.15 − 1/318.15)) · exp(−1.965·0.30)
  = 2.097e-4 · 0.834 · 0.555
  = 9.7 × 10⁻⁵ s⁻¹

At X = 6.5, dX/dt ≈ 6.3 × 10⁻⁴ kg/(kg·s)
Per 60-s step: ΔX ≈ 0.038

Tray 9 inlet, T=29 °C, RH=1.0 (the saturated front in the 5 cm geometry):

K = 2.097e-4 · exp(3738·(1/323.15 − 1/302.15)) · exp(−1.965·1.0)
  = 2.097e-4 · 0.405 · 0.140
  = 1.19 × 10⁻⁵ s⁻¹    (8× weaker than tray 0)

Kinetic ΔX would be 0.0046 per step, but the air-capacity gate (compute_dm_w_air_capacity) returns 0 because the air is already at RH=1.0 and cannot accept any more moisture. The tray freezes until upstream trays dry enough that fresher air reaches it. That is the drying-front behaviour seen in the 5 cm / 0.9 m/s smoke run.

# 9. The min(kinetic, air-capacity) gate

Per-step water removed is min(kinetic, air-capacity). The air-capacity gate (kinetics.py:compute_dm_w_air_capacity) bisects on Δω until the resulting outlet RH equals RH_out_max_frac (default ≈ 0.85). It returns m_da · Δω_max · dt. When the incoming air is already saturated, Δω_max = 0 and the kinetic K becomes irrelevant. This is the physical reason the propagating drying front exists in deep beds.

# 10. Variable map

| Variable | Lives in | Set when | Used by |
| --- | --- | --- | --- |
| _PARAMETRIC_PARAMS | kinetics.py module global | First sim call | Every later keff_from_state |
| _M1_T_REF_C=50, _M1_V_REF=1.1, _M1_D_REF=6 | kinetics.py:111–113 | Source constant | Both fit and evaluation |
| cfg.v_ms, cfg.thickness_mm | KineticsConfig | At sim startup | keff_from_state for v/d terms |
| cfg.RH_out_max_frac | KineticsConfig | At sim startup | Air-capacity gate (≈0.85) |
| X_eq_local per tray | runtime loop | Per step | First-order update (=0 since GAB removed) |

# 11. End-to-end summary

13 raw curves  →  PAVA cleaning  →  five-parameter NLS over all 1,573 points  →  cached parameters in module global  →  per-step K(T, RH) lookup at each tray  →  first-order moisture update  →  air-capacity gate  →  tray ΔX. Fit runs once per Python process. Evaluation runs ~10⁵ times per simulation but is just exp/log arithmetic on the cached parameters, so it is essentially free.