# Results and Discussion

This chapter presents the simulation results for ten SAHPD configurations of increasing complexity. Config A establishes the heat-pump-only baseline. Config B adds a flat-plate solar collector in series. Config C explores alternative solar placements that reveal fundamental design principles. An evaporator temperature investigation and VPD-based bypass strategy optimise closed-loop performance. Config D introduces heat recovery, and Config E combines heat recovery with solar integration to achieve the lowest energy consumption. Each section builds on the insights of the previous, tracing a path from the simplest dryer to the most efficient system. Section 7 synthesises all findings into a unified ranking and practical recommendations, and Section 8 documents the assumptions and limitations that bound the conclusions.

# 1  Config A: Heat-Pump-Only Baseline

## 1.1  System Description and Air Paths

Config A is the baseline heat-pump convective dryer (HPCD) with no solar assistance or heat recovery. It operates in one of two modes depending on the recirculation ratio r.

Open-loop mode (r = 0)

[DIAGRAM: Config A open-loop air path]

Figure 1: Config A open-loop (r = 0) schematic. Process air and evaporator air are separate streams.

Process air path: Ambient air enters the condenser, is heated from T_amb to T_set (45 °C), passes through the drying chamber where it picks up moisture, and is exhausted. The air is not dehumidified — it enters the chamber at ambient humidity ratio.

Evaporator heat source: A separate ambient air stream (or the outdoor environment) provides the low-temperature heat source. The evaporator saturation temperature is T_evap = T_amb − 10 K, adapting to local conditions every timestep.

Temperature lift: The compressor lifts from T_evap = T_amb − 10 °C to T_cond = T_set + 10 = 55 °C. The lift is 65 − T_amb [°C], so warmer ambient air means a smaller lift and higher COP.

Closed-loop mode (r > 0)

[DIAGRAM: Config A closed-loop air path]

Figure 2: Config A closed-loop (r > 0) schematic. Recirculated exhaust is mixed with fresh air, dehumidified at the evaporator, then reheated at the condenser.

In closed-loop mode, a fraction r of the chamber exhaust is recirculated and mixed with fresh ambient air before entering the evaporator:

Mixing: ω_mix = r·ω_exh + (1−r)·ω_amb; h_mix = r·h_exh + (1−r)·h_amb.

Evaporator (dehumidification): The mixed air passes over the coil at T_coil = T_evap + 3 K, where T_evap = 5 °C (fixed). Effectiveness ε_evap = 0.85 determines the outlet temperature. If the outlet falls below the mixed air’s dew point, moisture condenses and the air exits saturated.

Condenser (reheating): The cold, dehumidified air is heated to T_set = 45 °C. The first law is enforced: Q_cond = Q_evap + W_comp.

A key difference: in open-loop, T_evap adapts to ambient (T_amb − 10 K). In closed-loop, T_evap is fixed at 5 °C. This distinction drives the performance crossover discussed in Section 1.3.

## 1.2  Open-Loop Baseline (r = 0)

| Location | Elev. (m) | T_amb mean (°C) | SEC (kWh/kg) | Time (h) | COP mean | W_comp (kWh) |
| --- | --- | --- | --- | --- | --- | --- |
| KTM | 1 350 | 9.8 | 0.717 | 13.8 | 3.63 | 13.5 |
| BTN | 72 | 18.8 | 0.543 | 14.4 | 4.43 | 10.1 |
| TPJ | 1 820 | 12.0 | 0.566 | 14.0 | 3.92 | 10.6 |

Table 1: Config A open-loop (r = 0) annual-TMY performance.

SEC ranges from 0.543 kWh/kg at Biratnagar to 0.717 kWh/kg at Kathmandu, a difference of 24.3%. Two factors explain the gap:

Temperature lift: At Biratnagar (T_amb ≈ 19 °C), the lift is ≈46 K, yielding COP = 4.43. At Kathmandu (T_amb ≈ 10 °C), the lift is ≈55 K and COP = 3.63.

Atmospheric pressure: Biratnagar at 72 m operates at ≈100.5 kPa; Kathmandu at 1 350 m at ≈86.1 kPa. Lower pressure reduces air density and affects psychrometric state.

Drying time is nearly identical (13.8–14.4 h) because the chamber inlet is T_set = 45 °C at all locations. The difference between locations is purely in electricity consumption.

| Location | Annual | Autumn | Winter | Spring |
| --- | --- | --- | --- | --- |
| KTM | 0.717 | 0.500 | 0.588 | 0.487 |
| BTN | 0.543 | 0.435 | 0.427 | 0.367 |
| TPJ | 0.566 | 0.457 | 0.622 | 0.451 |

Table 2: Config A (r = 0) SEC (kWh/kg) by season.

Seasonal variation is substantial. At Biratnagar, spring SEC (0.367) is 14.0% lower than winter (0.427), driven by the higher spring ambient temperature.

## 1.3  Effect of Recirculation

| Location | r | SEC (kWh/kg) | Time (h) | COP mean | T_evap mean (°C) |
| --- | --- | --- | --- | --- | --- |
| KTM | 0.0 | 0.717 | 13.8 | 3.63 | -0.2 |
| KTM | 0.9 | 0.669 | 14.9 | 4.04 | 5.0 |
| KTM | 1.0 | 0.674 | 15.2 | 4.04 | 5.0 |
| BTN | 0.0 | 0.543 | 14.4 | 4.43 | 8.8 |
| BTN | 0.9 | 0.753 | 14.6 | 4.04 | 5.0 |
| BTN | 1.0 | 0.752 | 14.7 | 4.04 | 5.0 |
| TPJ | 0.0 | 0.566 | 14.0 | 3.92 | 2.0 |
| TPJ | 0.9 | 0.646 | 15.2 | 4.04 | 5.0 |
| TPJ | 1.0 | 0.647 | 15.4 | 4.04 | 5.0 |

Table 3: Config A annual-TMY performance at r = 0, 0.9, and 1.0.

The recirculation crossover

Recirculation improves SEC at Kathmandu but worsens it at Biratnagar. The mechanism is rooted in the evaporator temperature:

Open-loop (r = 0): T_evap = T_amb − 10 K. At Biratnagar (T_amb ≈ 19 °C), T_evap ≈ 9 °C. At Kathmandu (T_amb ≈ 10 °C), T_evap ≈ 0 °C.

Closed-loop (r > 0): T_evap is fixed at 5 °C regardless of ambient conditions. COP ≈ 4.04.

The crossover occurs at T_amb ≈ 15 °C, where the open-loop T_evap (T_amb − 10 = 5 °C) equals the fixed closed-loop value. Below this threshold, recirculation raises T_evap (e.g. from 0 °C to 5 °C at Kathmandu) and improves COP. Above it, recirculation lowers T_evap (e.g. from 9 °C to 5 °C at Biratnagar) and degrades COP.

A second reinforcing effect is the condenser inlet temperature. At r = 0, air enters the condenser at T_amb. At r = 0.9, the evaporator cools mixed air to T_after_evap ≈ 12 °C. At Biratnagar, this means the condenser receives 12 °C instead of 19 °C — 7 K of additional reheating. At Kathmandu, the condenser receives 12 °C instead of 10 °C — slightly warmer.

At Kathmandu, r = 0.9 reduces annual SEC by 6.7% (from 0.717 to 0.669 kWh/kg). At Biratnagar, r = 0.9 increases SEC from 0.543 to 0.753.

The difference between r = 0.9 and r = 1.0 is negligible (≤1%), indicating that the system’s energy balance is dominated by the internal heat pump cycle rather than the small fresh-air fraction.

# 2  Config B: Solar-Assisted HP (Series)

## 2.1  System Description

[DIAGRAM: Config B air path]

Figure 3: Config B series air path. Ambient air passes through the solar collector, then the condenser, before entering the chamber.

Config B adds a flat-plate solar collector (A_c = 10 m²) in series with the heat pump. The collector is modelled using the Hottel–Whillier–Bliss equation with an optical efficiency τα = 0.75 and an overall heat loss coefficient U_L = 5.0 W/m²K. These values represent a typical single-glazed flat-plate air collector (Duffie & Beckman, 2013; τα = 0.75–0.80, U_L = 4–6 W/m²K for single glass). The heat removal factor F_R is computed dynamically at each timestep using the NTU–effectiveness method, which accounts for the actual air mass flow rate and collector area.

Open-loop (r = 0): Ambient → Solar → Condenser → Chamber. The solar collector pre-heats ambient air; the condenser provides the remaining lift to T_set. At night, the system degrades gracefully to Config A behaviour.

Closed-loop (r > 0): Mix → Evaporator → Solar → Condenser → Chamber. The evaporator dehumidifies the recirculated air, then the solar collector reheats it before the condenser provides the final lift.

## 2.2  Open-Loop Results (r = 0, A_c = 10 m²)

| Location | SEC (kWh/kg) | Time (h) | COP | Solar frac. | W_comp (kWh) | ΔSEC vs A |
| --- | --- | --- | --- | --- | --- | --- |
| KTM | 0.488 | 13.8 | 3.63 | 0.63 | 9.1 | 32.0% |
| BTN | 0.287 | 14.4 | 4.43 | 0.80 | 5.2 | 47.1% |
| TPJ | 0.344 | 14.0 | 3.92 | 0.72 | 6.3 | 39.2% |

Table 4: Config B (r = 0, A_c = 10 m²) annual-TMY performance.

Solar integration reduces SEC by 32.0% at Kathmandu and 47.1% at Biratnagar. Drying time is unchanged (≈14 h) because the chamber inlet remains at T_set = 45 °C. The benefit is purely in reduced compressor work.

| Location | Annual | Autumn | Winter | Spring |
| --- | --- | --- | --- | --- |
| KTM | 0.488 | 0.289 | 0.295 | 0.182 |
| BTN | 0.287 | 0.267 | 0.240 | 0.128 |
| TPJ | 0.344 | 0.243 | 0.379 | 0.212 |

Table 5: Config B (r = 0, A_c = 10 m²) SEC by season.

## 2.3  Effect of Recirculation on Config B

| Location | r | SEC (kWh/kg) | Time (h) | COP | Solar frac. |
| --- | --- | --- | --- | --- | --- |
| KTM | 0.0 | 0.488 | 13.8 | 3.63 | 0.63 |
| KTM | 0.9 | 0.469 | 14.9 | 4.04 | 0.63 |
| KTM | 1.0 | 0.477 | 15.2 | 4.04 | 0.62 |
| BTN | 0.0 | 0.287 | 14.4 | 4.43 | 0.80 |
| BTN | 0.9 | 0.423 | 14.6 | 4.04 | 0.76 |
| BTN | 1.0 | 0.423 | 14.7 | 4.04 | 0.76 |
| TPJ | 0.0 | 0.344 | 14.0 | 3.92 | 0.72 |
| TPJ | 0.9 | 0.416 | 15.2 | 4.04 | 0.69 |
| TPJ | 1.0 | 0.418 | 15.4 | 4.04 | 0.68 |

Table 6: Config B (A_c = 10 m²) at r = 0, 0.9, and 1.0.

Recirculation degrades Config B at all locations. At Biratnagar, r = 0.9 increases SEC from 0.287 to 0.423 kWh/kg. The penalty is more severe than in Config A because of the solar–evaporator interaction:

The evaporator cools the mixed air from ~35 °C to ~12 °C. The solar collector then receives this cold air and must reheat it, largely compensating for the cooling the evaporator just performed.

The compressor does extra work to cool air that the solar collector then undoes. The solar fraction decreases because more compressor energy is consumed.

For solar-assisted HP dryers in series, open-loop operation is strongly preferred. This finding raises the question: can the solar collector be placed differently to avoid this interaction?

# 3  Config C: Solar Cascade Configurations

Config B demonstrated that a solar collector in series with the condenser reduces SEC by 32–47%. A natural question is whether rearranging the solar collector’s position can improve performance further. Config C explores two cascade arrangements where the solar collector interacts with the evaporator side of the cycle.

## 3.1  C1 and C2: Two Cascade Arrangements

Config C1: Mix before solar

[DIAGRAM: Config C1 air path]

Figure 4: Config C1 air path. Recirculated exhaust mixes with ambient, then passes through the solar collector, evaporator, and condenser.

r > 0: [r×Exhaust + (1−r)×Ambient] → Solar → Evaporator → Condenser → Chamber

r = 0: Ambient → Solar → Evaporator → Condenser → Chamber

The solar collector heats the mixed air, which then enters the evaporator. The evaporator must cool this warm, solar-heated air back down to the coil surface temperature — undoing the solar gain.

Config C2: Mix after solar

[DIAGRAM: Config C2 air path]

Figure 5: Config C2 air path. Fresh ambient air passes through the solar collector, then mixes with exhaust before the evaporator.

r > 0: Ambient → Solar → [Mix + r×Exhaust] → Evaporator → Condenser → Chamber

r = 0: Ambient → Solar → Evaporator → Condenser → Chamber

The solar collector always sees fresh ambient air (cold inlet → high collector efficiency). At r = 0, both C1 and C2 reduce to the same path. In both cases, the evaporator sits between the solar collector and condenser — a placement that has profound consequences.

## 3.2  Config C1 Results: The Cost of Wrong Ordering

| Config | Location | A_c (m²) | SEC (kWh/kg) | Time (h) | COP | Solar frac. |
| --- | --- | --- | --- | --- | --- | --- |
| A (r=0) | KTM | — | 0.717 | 13.8 | 3.63 | — |
| B (r=0) | KTM | 10 | 0.488 | 13.8 | 3.63 | 0.63 |
| C1 (r=0) | KTM | 10 | 0.718 | 31.7 | 3.79 | 0.70 |
| A (r=0) | BTN | — | 0.543 | 14.4 | 4.43 | — |
| B (r=0) | BTN | 10 | 0.287 | 14.4 | 4.43 | 0.80 |
| C1 (r=0) | BTN | 10 | 0.894 | 28.4 | 4.02 | 0.63 |
| A (r=0) | TPJ | — | 0.566 | 14.0 | 3.92 | — |
| B (r=0) | TPJ | 10 | 0.344 | 14.0 | 3.92 | 0.72 |
| C1 (r=0) | TPJ | 10 | 0.637 | 31.7 | 3.92 | 0.71 |

Table 7: Config C1 (r = 0, A_c = 10 m²) vs Config A and B.

Config C1 produces alarming results. At Kathmandu with 10 m², SEC = 0.718 — nearly identical to Config A (0.717) despite a solar fraction of 0.70. Worse, drying time explodes to 31.7 h, more than double the baseline.

The explanation is straightforward: the solar collector heats air that the evaporator immediately cools back down. At night, with no solar gain, the evaporator still cools ambient air unnecessarily before the condenser reheats it. The solar energy is largely wasted, and the evaporator’s cooling step extends drying time by forcing the condenser to work harder.

## 3.3  Config C2 Results: Solar-Boosted COP

| Config | Location | SEC (kWh/kg) | Time (h) | COP | Solar frac. | ΔSEC vs A |
| --- | --- | --- | --- | --- | --- | --- |
| C2 (r=0) | KTM | 0.561 | 13.8 | 5.05 | 0.59 | 21.8% |
| B (r=0) | KTM | 0.488 | 13.8 | 3.63 | 0.63 | 32.0% |
| C2 (r=0) | BTN | 0.381 | 14.4 | 7.76 | 0.76 | 29.7% |
| B (r=0) | BTN | 0.287 | 14.4 | 4.43 | 0.80 | 47.1% |
| C2 (r=0) | TPJ | 0.424 | 14.0 | 6.21 | 0.67 | 25.1% |
| B (r=0) | TPJ | 0.344 | 14.0 | 3.92 | 0.72 | 39.2% |

Table 8: Config C2 vs B (r = 0, A_c = 10 m², annual TMY).

C2 performs far better than C1, with normal drying times (13.8 h) and notably high COP (5.05 at KTM, 7.76 at BTN). The solar collector pre-heats the evaporator source air, raising T_evap and reducing the temperature lift.

However, C2 still does not match Config B. At Kathmandu, B achieves 0.488 versus C2’s 0.561. The reason is fundamental: C2 improves COP but does not reduce Q_cond. The condenser still heats air from the evaporator outlet to 45 °C. In Config B, the solar collector directly reduces Q_cond by pre-heating the condenser inlet. Since W_comp depends on both Q_cond and COP, reducing Q_cond (Config B) is more effective than only increasing COP (Config C2).

## 3.4  Lessons: Where to Put the Solar Collector

The ranking is consistent across all locations: B > C2 > C1. Three design principles emerge:

Solar energy is most effective on the condenser side. Config B’s series arrangement outperforms the cascade arrangements by 13–25% in SEC.

Component ordering matters critically. Placing the evaporator after the solar collector (C1/C2) wastes solar energy on air that must be cooled for dehumidification.

COP improvement alone is insufficient. C2 achieves COP values 2–3× higher than B, yet has worse SEC because Q_cond is unchanged.

These principles motivate the Config D family, which introduces heat recovery to reduce the condenser load — the same mechanism that makes Config B effective, but using waste heat from the exhaust rather than solar energy. Heat recovery works at night and in all weather, addressing Config B’s dependence on irradiance.

# 4  Evaporator Temperature and Bypass Optimisation

Before introducing the more advanced D and E configurations, this section addresses two optimisation questions arising from the Config A analysis: (1) Can the fixed evaporator temperature be improved? (2) Can the late-drying inefficiency be mitigated by bypassing the evaporator?

## 4.1  The Late-Drying Inefficiency

Analysis of the simulation time-series for Config A (r = 0.9) reveals that dehumidification effectiveness declines sharply as drying progresses:

| Drying phase | ω_mix (g/kg) | T_dp (°C) | T_after_evap (°C) | Δω removed (g/kg) |
| --- | --- | --- | --- | --- |
| Early (1 h) | 15.6 | 20.8 | 10.4 | 7.6 |
| Mid (7 h) | 11.1 | 15.5 | 12.4 | 2.1 |
| Late (13 h) | 9.6 | 13.3 | 13.1 | 0.15 |

Table 9: Evaporator dehumidification over the drying cycle (Config A, r = 0.9, Biratnagar).

By hour 13, the mixed air’s dew point (13.3 °C) nearly equals the evaporator outlet (13.1 °C). The evaporator is still cooling air from ~35 °C to ~13 °C — consuming compressor energy — but removing almost no moisture. The condenser must then reheat from 13 °C to 45 °C: 22 K of wasted cooling and reheating.

## 4.2  Adaptive Evaporator Temperature: A Negative Result

An adaptive strategy was investigated that sets T_evap relative to the condensation onset temperature — the coil temperature at which the evaporator outlet just reaches the dew point:

T_evap,coil,onset = T_mix − (T_mix − T_dp,mix) / ε_evap

The evaporator is set Δ K below onset: T_evap = T_onset − Δ. A small Δ provides minimal subcooling; a large Δ approaches the aggressive dehumidification of the fixed strategy. A sweep from Δ = 2 to 20 K was conducted for Config A at r = 0.9.

| Δ (K) | KTM SEC | KTM time (h) | BTN SEC | BTN time (h) |
| --- | --- | --- | --- | --- |
| Fixed 5°C | 0.669 | 14.9 | 0.753 | 14.6 |
| 2 | 0.680 | 22.4 | 0.678 | 20.3 |
| 4 | 0.700 | 16.9 | 0.747 | 16.1 |
| 6 | 0.731 | 14.7 | 0.799 | 14.2 |
| 10 | 0.806 | 13.7 | 0.891 | 13.3 |
| 20 | 0.892 | 13.2 | 0.996 | 12.8 |

Table 10: Onset-tracking adaptive T_evap sweep (Config A, r = 0.9). Fixed baseline included for comparison.

At Kathmandu, the fixed baseline wins outright. At Biratnagar, Δ = 2 K provides a 10% SEC improvement but extends drying time from 14.6 h to 20.3 h — a 39% increase. For practical daily-batch operations, this trade-off is unacceptable.

Why adaptive T_evap fails

The drying kinetics model includes an RH-dependent suppression term with exponent α_RH = 1.75. Higher chamber RH directly slows the drying rate. When the evaporator is raised above 5 °C (less dehumidification), chamber RH increases, the drying rate decreases, and extended fan and compressor hours offset any COP improvement.

This is specific to systems with RH-dependent kinetics. In a temperature-limited system, reducing subcooling would improve COP without affecting drying time. For apple slices, with their strong humidity sensitivity, aggressive dehumidification at T_evap = 5 °C is near-optimal.

## 4.3  Condenser-Direct Bypass: A Better Approach

Rather than weakening dehumidification (raising T_evap), the bypass strategy eliminates the cooling–reheating cycle entirely when dehumidification is no longer needed.

[DIAGRAM: Config A condenser-direct bypass]

Figure 6: Condenser-direct bypass. Left: normal evaporator path. Right: bypass — warm exhaust routes directly to condenser; evaporator runs on ambient as heat source only.

The condenser penalty fraction estimates how much the evaporator improves drying potential:

cond_penalty = (VPD_post_evap − VPD_exhaust) / VPD_post_evap

When this drops below a threshold, the system switches to bypass: warm exhaust (~43 °C) goes directly to the condenser (Q_cond ≈ 0.2 kW vs ~2 kW in normal mode). Humidity accumulates in the loop; when the penalty exceeds 3× the threshold, the system reverts to normal dehumidification. A physics-based dwell time prevents chattering: the minimum time between mode switches is computed from the humidity accumulation rate (τ = Δpenalty / |d(penalty)/dt|), clamped between 300 s (compressor protection) and 7 200 s (prevent lock-in). This ensures the system waits long enough for humidity to meaningfully change before switching again.

| Threshold | KTM SEC | KTM time (h) | BTN SEC | BTN time (h) |
| --- | --- | --- | --- | --- |
| OFF | 0.669 | 14.9 | 0.753 | 14.6 |
| 0.02 | 0.650 | 14.9 | 0.753 | 14.6 |
| 0.05 | 0.536 | 15.6 | 0.617 | 15.3 |
| 0.10 | 0.365 | 19.1 | 0.395 | 19.8 |
| 0.15 | 0.331 | 30.9 | 0.336 | 28.8 |
| 0.20 | 0.349 | 48.5 | 0.303 | 30.9 |

Table 11: Config A (r = 0.9) condenser-direct bypass threshold sweep.

The trade-off is clear. At threshold = 0.05, SEC drops by 18–20% with only ~1 h of additional drying time. At 0.10, the reduction reaches 45–48% but drying extends to ~19 h. Beyond 0.15, drying time doubles and SEC gains plateau.

The mechanism is intuitive: during bypass, compressor work drops from ~2 kW to ~0.2 kW (heating from 43 °C to 45 °C instead of 12 °C to 45 °C). However, no dehumidification occurs, so chamber RH rises and drying slows. The system oscillates between bypass (low energy, slow drying) and normal (high energy, fast drying), with the threshold controlling the duty cycle.

For daily-batch operations (target ≤16 h), threshold = 0.05 provides the best balance. This condenser-direct bypass concept is extended to D and E configurations as an exhaust bypass strategy in subsequent sections.

# 5  Config D: Heat Recovery Exchanger

The Config C analysis established that reducing the condenser load is the most effective use of supplementary energy. Config B achieves this with solar pre-heating, but its performance depends on irradiance and degrades at night. Config D takes a different approach: recovering waste heat from the chamber exhaust using a counter-flow heat recovery exchanger (HRX). This works continuously — day and night, in all weather — because it exploits the temperature difference between the warm exhaust and cool ambient air.

## 5.1  System Description and Variants

All D configurations use a counter-flow plate HRX with effectiveness ε_HRX = 0.70. The HRX transfers sensible heat from the warm exhaust stream (hot side) to the incoming ambient air (cold side). The outlet temperatures are:

T_amb,heated = T_amb + ε · (T_exhaust − T_amb)

T_exh,cooled = T_exhaust − ε · (T_exhaust − T_amb)

If the cooled exhaust falls below its dew point, condensation occurs on the exhaust side, recovering latent heat as well. The ambient side experiences only sensible heating (no moisture addition). All D configs operate in open-loop (r = 0).

Config D1: Ambient preheating, separate evaporator

[DIAGRAM: Config D1 air path]

Figure 7: Config D1 air path. Ambient air is preheated by the HRX, then heated to T_set by the condenser. Exhaust is cooled by the HRX and expelled. Evaporator draws from a separate ambient stream.

Condenser stream: Ambient → HRX (cold side) → Condenser → Chamber

Exhaust stream: Chamber → HRX (hot side) → Expelled

Evaporator: Separate ambient air (T_evap = T_amb − 10 K)

The HRX pre-heats the ambient air before the condenser. If the exhaust is at 40 °C and ambient at 10 °C, the HRX delivers air at 10 + 0.70 × 30 = 31 °C to the condenser. The condenser only needs to provide the remaining 14 K lift to 45 °C, compared to 35 K without heat recovery.

Config D2: Dynamic ambient compensation at evaporator

[DIAGRAM: Config D2 air path]

Figure 8: Config D2 air path. Same condenser path as D1. The evaporator draws from cooled exhaust (post-HRX), supplemented with ambient air when the exhaust alone cannot supply Q_evap.

D2 uses the same condenser path as D1, but routes the cooled exhaust (post-HRX) to the evaporator as a heat source. Since the cooled exhaust is still warmer than raw ambient (T_exh,cooled ≈ 19 °C vs T_amb ≈ 10 °C at Kathmandu), the evaporator operates at a higher T_evap and better COP. When the exhaust stream alone cannot supply Q_evap, ambient air is dynamically mixed in to make up the deficit. This creates a circular dependency between the air mix and the HP operating point, resolved by a fixed-point iteration described in Section 6.1 under Config E2.

Config D3: Swapped routing (humidity risk)

D3 reverses the stream assignment: exhaust goes through the HRX and then to the condenser; ambient goes through the HRX and then to the evaporator. This gives the evaporator a warm source (HRX-heated ambient) for high COP, but sends partially dehumidified exhaust air to the chamber — introducing a humidity penalty.

## 5.2  D1 and D2 Results

| Config | Location | SEC (kWh/kg) | Time (h) | COP | ΔSEC vs A(r=0) |
| --- | --- | --- | --- | --- | --- |
| D1 | KTM | 0.365 | 13.8 | 3.63 | 49.1% |
| D2 | KTM | 0.354 | 13.8 | 3.95 | 50.6% |
| D1 | BTN | 0.293 | 14.4 | 4.43 | 45.9% |
| D2 | BTN | 0.282 | 14.4 | 5.00 | 48.1% |
| D1 | TPJ | 0.310 | 14.0 | 3.92 | 45.2% |
| D2 | TPJ | 0.299 | 14.0 | 4.39 | 47.2% |

Table 12: Config D1 and D2 annual-TMY performance vs Config A (r = 0).

Heat recovery delivers dramatic SEC reductions. D1 achieves 0.365 kWh/kg at Kathmandu (49.1% below Config A) and 0.293 at Biratnagar (45.9% below Config A). Drying time is unchanged (≈14 h) because the chamber still receives air at T_set = 45 °C.

D2 provides a further improvement over D1: 0.354 at Kathmandu (COP = 3.95) vs D1’s 0.365 (COP = 3.63). The advantage comes from the warmer evaporator source: at Kathmandu, D2’s evaporator sees cooled exhaust at ≈19 °C (T_evap ≈ 9 °C) instead of D1’s raw ambient at ≈10 °C (T_evap ≈ 0 °C). The reduced temperature lift improves COP by 9%.

Compared to Config B (solar series, 10 m²), the D configs offer a meaningful trade-off. Config B achieves lower SEC at high-irradiance locations (BTN) but depends on weather. The D configs provide consistent performance regardless of solar conditions, making them attractive for operations that span night-time or monsoon seasons.

## 5.3  Config D3: The Humidity Penalty

| Config | Location | SEC (kWh/kg) | Time (h) | COP |
| --- | --- | --- | --- | --- |
| D3 | KTM | 0.477 | 17.3 | 5.91 |
| D3 | BTN | 0.460 | 20.4 | 6.67 |
| D3 | TPJ | 0.439 | 18.9 | 6.15 |

Table 13: Config D3 annual-TMY performance.

D3 achieves the highest COP among D variants (5.91 at KTM, 6.67 at BTN) because the evaporator sees warm HRX-heated ambient air. However, SEC is substantially worse (0.477 and 0.460 kWh/kg) and drying time extends to 17.3–20.4 h.

The root cause is that D3 sends the HRX-cooled exhaust — which is still humid — to the condenser and then the chamber. The chamber receives air at 45 °C but with higher humidity ratio than in D1/D2 (which send dry ambient air). The RH-dependent drying kinetics (α_RH = 1.75) directly penalise this: higher chamber RH slows drying, extending the batch time and increasing total energy consumption despite the better COP.

This reinforces the lesson from the adaptive T_evap investigation: for products with strong humidity sensitivity, the humidity of the chamber inlet air matters as much as — or more than — the thermodynamic efficiency of the heat pump cycle. D3 is not recommended for humidity-sensitive products.

## 5.4  VPD Exhaust Bypass for D Configs

The condenser-direct bypass developed in Section 4.3 for Config A is adapted here as an exhaust bypass for the open-loop D configs. The trigger metric is VPD utilisation — the fraction of the inlet air’s drying potential consumed by the chamber:

utilisation = 1 − VPD_exhaust / VPD_inlet

[DIAGRAM: Config D exhaust bypass]

Figure 9: Exhaust bypass for D configs. When VPD utilisation drops below the threshold, warm exhaust routes directly to condenser, bypassing the HRX.

When utilisation falls below the threshold (air passes through the chamber nearly unchanged in late drying), warm exhaust (≈40 °C) bypasses the HRX and routes directly to the condenser. The condenser provides only ≈5 K of reheating instead of the full lift from HRX-heated ambient. When utilisation rises above 3× the threshold, the system reverts to normal HRX operation. A fixed 600 s minimum dwell time prevents oscillation.

| Config | Location | SEC (no VPD) | SEC (VPD 0.05) | ΔSEC | Time (no VPD) | Time (VPD) |
| --- | --- | --- | --- | --- | --- | --- |
| D1 | KTM | 0.365 | 0.287 | 21.4% | 13.8 | 15.1 |
| D2 | KTM | 0.354 | 0.288 | 18.4% | 13.8 | 15.1 |
| D1 | BTN | 0.293 | 0.238 | 19.0% | 14.4 | 15.8 |
| D2 | BTN | 0.282 | 0.238 | 15.3% | 14.4 | 15.8 |
| D1 | TPJ | 0.310 | 0.248 | 20.1% | 14.0 | 15.2 |
| D2 | TPJ | 0.299 | 0.250 | 16.2% | 14.0 | 15.2 |

Table 14: D1 and D2 with VPD exhaust bypass (threshold = 0.05).

At the recommended threshold of 0.05, D1 achieves 19–21% SEC reduction and D2 achieves 15–18%, both with only 1–2 h of additional drying time. D2’s smaller percentage gain reflects the fact that its baseline SEC is already lower (better COP from the warmer evaporator source), leaving less room for bypass savings. The combined effect of heat recovery plus VPD bypass brings D1 to 0.238–0.287 kWh/kg and D2 to 0.238–0.288 kWh/kg — comparable to Config B’s solar-assisted performance but without any dependence on solar irradiance.

# 6  Config E: Combined HRX + Solar

Config E combines the two most effective strategies identified so far: heat recovery (Config D) and solar pre-heating on the condenser side (Config B). By stacking both, the condenser load is reduced from two independent sources — waste exhaust heat and solar irradiance — achieving the lowest SEC values in this study.

## 6.1  System Description

Config E1: HRX + Solar, separate evaporator

[DIAGRAM: Config E1 air path]

Figure 10: Config E1 air path. Ambient air is preheated by the HRX, then by the solar collector, then by the condenser. Evaporator draws from separate ambient.

Condenser stream: Ambient → HRX → Solar → Condenser → Chamber

Evaporator: Separate ambient air (as in D1)

E1 extends D1 by inserting the solar collector between the HRX and the condenser. The condenser receives air that has been preheated by both the HRX and the sun. On a clear day when the HRX delivers air at 31 °C and the solar collector adds another 10 K, the condenser only needs to lift from 41 °C to 45 °C — a 4 K lift requiring minimal compressor work.

Config E2: HRX + Solar, dynamic evaporator

[DIAGRAM: Config E2 air path]

Figure 11: Config E2 air path. Same condenser path as E1. Evaporator draws from cooled exhaust with dynamic ambient supplement (as in D2).

E2 uses the same condenser path as E1 but routes the cooled exhaust (post-HRX) to the evaporator with dynamic ambient supplement, as in D2. The warmer evaporator source improves COP further. When exhaust alone cannot supply Q_evap, ambient air is mixed in to make up the deficit. However, changing the air mix changes the evaporator source temperature, which in turn changes the HP operating point and therefore Q_evap itself — creating a circular dependency that must be resolved iteratively.

Iterative evaporator sizing algorithm

The evaporator coil temperature is set at T_evap = T_evap_source − 10 K (the approach temperature), so the available heat extraction is:

Q_avail = (ṁ_da + ṁ_amb,extra) · c_p · ε_evap · 10 K

where ṁ_da is the main dry-air mass flow and ṁ_amb,extra is the supplementary ambient air. When the exhaust stream alone (ṁ_amb,extra = 0) provides less heat than Q_evap required by the HP cycle, the algorithm proceeds as follows:

Step 1: Size the HP at the current T_evap_source to obtain Q_evap,needed.

Step 2: Compute Q_avail from the current air mix. If Q_avail ≥ Q_evap,needed, converged — stop.

Step 3: Calculate the total air mass needed: ṁ_total = Q_evap,needed / (c_p · ε_evap · 10). Set ṁ_amb,extra = ṁ_total − ṁ_da.

Step 4: Update the mixed source temperature: T_evap_source = (ṁ_da · T_exh,cooled + ṁ_amb,extra · T_amb) / (ṁ_da + ṁ_amb,extra).

Step 5: If |T_new − T_old| < 0.05 K, converged. Otherwise return to Step 1 (max 5 iterations).

In practice, convergence is reached within 2–3 iterations. The algorithm ensures that the evaporator always has a physically realisable heat source: exhaust alone when it suffices (higher COP), or a blend with ambient when the exhaust is too cold or the flow too small. This same algorithm is used in Config D2 (Section 5).

Config E3: Solar-priority control

[DIAGRAM: Config E3 air path]

Figure 12: Config E3 air path. Ambient → HRX → Condenser (variable) → Solar → Chamber. HP provides partial lift; solar finishes.

E3 rearranges the solar collector to after the condenser: Ambient → HRX → Condenser → Solar → Chamber. The HP provides a partial temperature lift (variable T_cond), and the solar collector supplies the remaining gain to T_set. When solar irradiance alone (from the HRX output temperature) can reach T_set, the HP is switched off entirely. This solar-priority control maximises solar utilisation but means the HP operates at a variable and sometimes higher T_cond when solar contribution is low.

## 6.2  E1 and E2 Results (A_c = 10 m²)

| Config | Location | SEC (kWh/kg) | Time (h) | COP | Solar frac. | ΔSEC vs A(r=0) |
| --- | --- | --- | --- | --- | --- | --- |
| E1 | KTM | 0.220 | 13.8 | 3.63 | 0.71 | 69.3% |
| E2 | KTM | 0.197 | 13.8 | 4.24 | 0.73 | 72.5% |
| E1 | BTN | 0.141 | 14.4 | 4.43 | 0.87 | 74.0% |
| E2 | BTN | 0.129 | 14.4 | 5.09 | 0.88 | 76.2% |
| E1 | TPJ | 0.178 | 14.0 | 3.92 | 0.78 | 68.6% |
| E2 | TPJ | 0.163 | 14.0 | 4.54 | 0.80 | 71.1% |

Table 15: Config E1 and E2 (A_c = 10 m²) annual-TMY performance.

The combined HRX + solar approach produces the lowest SEC values seen so far. E2 at Biratnagar achieves 0.129 kWh/kg — 76.2% below the Config A baseline — with a solar fraction of 0.88 and COP of 5.09. At Kathmandu, E2 achieves 0.197 kWh/kg.

E2 consistently outperforms E1 by 8.5% at BTN, following the same D2 > D1 pattern: the warmer evaporator source (cooled exhaust) improves COP.

## 6.3  Config E3: Solar-Priority Control

| Config | Location | SEC (kWh/kg) | Time (h) | COP | Solar frac. |
| --- | --- | --- | --- | --- | --- |
| E3 | KTM | 0.217 | 13.8 | 4.98 | 0.67 |
| E3 | BTN | 0.137 | 14.4 | 6.36 | 0.87 |
| E3 | TPJ | 0.176 | 14.0 | 5.60 | 0.77 |

Table 16: Config E3 (A_c = 10 m²) annual-TMY performance.

E3 achieves SEC = 0.137 at BTN and 0.217 at KTM, with higher COP (6.36 and 4.98) than E1 or E2. The higher COP reflects the solar-priority control: when the HP runs, it targets a lower T_cond (partial lift), and when solar is abundant the HP is off entirely.

However, E3 does not beat E2 in SEC at Biratnagar (0.137 vs 0.129). The reason is solar clipping: when the solar collector is placed after the condenser, it receives already-warm air (≈40 °C), and the collector’s efficiency drops because thermal losses scale with (T_in − T_amb). In E1/E2, the solar collector receives HRX-heated air at ≈30 °C — a colder inlet that preserves collector efficiency.

## 6.4  Solar Collector Area Sweep (E2)

To determine the practical collector size, a sweep from 2 to 20 m² was conducted for E2.

| A_c (m²) | KTM SEC | KTM solar frac. | BTN SEC | BTN solar frac. |
| --- | --- | --- | --- | --- |
| 2 | 0.291 | 0.30 | 0.214 | 0.51 |
| 4 | 0.260 | 0.48 | 0.171 | 0.71 |
| 6 | 0.234 | 0.60 | 0.147 | 0.81 |
| 8 | 0.211 | 0.68 | 0.136 | 0.85 |
| 10 | 0.197 | 0.73 | 0.129 | 0.88 |
| 15 | 0.176 | 0.80 | 0.120 | 0.92 |
| 20 | 0.168 | 0.84 | 0.116 | 0.93 |

Table 17: E2 SEC and solar fraction vs collector area (annual TMY).

Diminishing returns are evident. At Biratnagar, increasing A_c from 6 to 10 m² reduces SEC by only 0.018 kWh/kg (from 0.147 to 0.129), while the first 6 m² reduces SEC by 0.067 (from 0.214 to 0.147). At Kathmandu, the marginal benefit of each additional m² is larger due to lower baseline irradiance but still flattens beyond 10 m².

For practical installations, 6–10 m² provides the best cost–performance balance. Larger collectors yield diminishing SEC reductions while increasing capital cost and roof area requirements.

## 6.5  VPD Exhaust Bypass for E Configs

The same VPD exhaust bypass strategy from Section 5.4 is applied to E1 and E2 (threshold = 0.05). E3 is excluded because its solar-priority control already manages HP operation dynamically.

| Config | Location | SEC (no VPD) | SEC (VPD 0.05) | ΔSEC | Time (VPD) |
| --- | --- | --- | --- | --- | --- |
| E1 | KTM | 0.220 | 0.152 | 30.8% | 15.1 |
| E2 | KTM | 0.197 | 0.144 | 26.9% | 15.1 |
| E1 | BTN | 0.141 | 0.101 | 28.5% | 15.8 |
| E2 | BTN | 0.129 | 0.097 | 24.7% | 15.8 |
| E1 | TPJ | 0.178 | 0.133 | 25.0% | 15.2 |
| E2 | TPJ | 0.163 | 0.129 | 21.1% | 15.2 |

Table 18: E1 and E2 (10 m²) with VPD exhaust bypass (threshold = 0.05).

VPD bypass delivers 25–31% SEC improvement for E1 and 21–27% for E2, with only 1–2 h extra drying time. The best result is E2 + VPD at Biratnagar: SEC = 0.097 kWh/kg (15.8 h drying time). At Kathmandu, E2 + VPD achieves 0.144 kWh/kg.

The optimal VPD threshold for E configs is 0.05 — lower than the 0.10–0.15 that minimises SEC for D configs (Section 5.4 sweep). This is because the solar collector already reduces the HP’s energy share substantially; aggressive bypass pushes drying time up without proportional SEC gains. At 0.05, the system briefly enters bypass during late drying, captures the easy compressor savings, and returns to normal before the drying rate deteriorates significantly.

These results represent the culmination of the configuration hierarchy: starting from Config A’s 0.543–0.717 kWh/kg, progressive addition of solar (B), heat recovery (D), their combination (E), and controls optimisation (VPD bypass) achieves 0.097–0.144 kWh/kg — an 80–87% reduction in specific energy consumption.

# 7  Grand Synthesis and Recommendations

Sections 1–6 analysed each configuration on its own terms. This section places all of them on a single axis so the trade-offs become directly visible, then converts the findings into practical recommendations.

## 7.1  Complete Performance Comparison

Table below ranks every configuration analysed in this study by specific energy consumption at the two production-scale locations (Kathmandu and Biratnagar). Each row uses the best operational setup explored for that configuration: A at r = 0 (open-loop), B/C/E at 10 m² collector, and VPD variants at threshold = 0.05.

| Rank | Configuration | KTM SEC | BTN SEC | Avg SEC | KTM time | BTN time | Δ vs A (KTM) | Δ vs A (BTN) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | E2 + VPD (10 m²) | 0.144 | 0.097 | 0.121 | 15.1 | 15.8 | 79.9% | 82.1% |
| 2 | E1 + VPD (10 m²) | 0.152 | 0.101 | 0.126 | 15.1 | 15.8 | 78.8% | 81.4% |
| 3 | E2 (10 m²) | 0.197 | 0.129 | 0.163 | 13.8 | 14.4 | 72.5% | 76.2% |
| 4 | E3 (10 m²) | 0.217 | 0.137 | 0.177 | 13.8 | 14.4 | 69.8% | 74.8% |
| 5 | E1 (10 m²) | 0.220 | 0.141 | 0.180 | 13.8 | 14.4 | 69.3% | 74.0% |
| 6 | D1 + VPD | 0.287 | 0.238 | 0.262 | 15.1 | 15.8 | 59.9% | 56.2% |
| 7 | D2 + VPD | 0.288 | 0.238 | 0.263 | 15.1 | 15.8 | 59.7% | 56.0% |
| 8 | D2 | 0.354 | 0.282 | 0.318 | 13.8 | 14.4 | 50.6% | 48.1% |
| 9 | D1 | 0.365 | 0.293 | 0.329 | 13.8 | 14.4 | 49.1% | 45.9% |
| 10 | B (10 m²) | 0.488 | 0.287 | 0.387 | 13.8 | 14.4 | 32.0% | 47.1% |
| 11 | D3 | 0.477 | 0.460 | 0.469 | 17.3 | 20.4 | 33.4% | 15.1% |
| 12 | C2 (10 m²) | 0.561 | 0.381 | 0.471 | 13.8 | 14.4 | 21.8% | 29.7% |
| 13 | A (r = 0) | 0.717 | 0.543 | 0.630 | 13.8 | 14.4 | 0.0% | 0.0% |
| 14 | A (r = 0.9) | 0.669 | 0.753 | 0.711 | 14.9 | 14.6 | 6.7% | -38.8% |
| 15 | C1 (10 m²) | 0.718 | 0.894 | 0.806 | 31.7 | 28.4 | -0.2% | -64.7% |

Table 19: Grand comparison of all configurations, ranked from best to worst by average SEC (annual TMY, KTM + BTN).

## 7.2  Ranking Narrative

The spread between worst and best is striking. The baseline Config A (r = 0) consumes 0.717–0.543 kWh/kg depending on climate, while the best configuration — E2 + VPD (10 m²) — achieves 0.144–0.097 kWh/kg, a 79.9%–82.1% reduction.

Tier 1 — Best performers (SEC < 0.20 avg):

The top tier is dominated by Config E variants. E2 + VPD leads the ranking because it combines all three gains: (i) heat recovery reduces condenser load; (ii) solar preheating supplies a large fraction of the remaining load; (iii) VPD exhaust bypass cuts compressor work during late drying when the chamber air is no longer carrying moisture efficiently. E1 and E3 follow close behind.

Tier 2 — Solid performers (SEC 0.20–0.35 avg):

Config D variants (with and without VPD) and some E variants at lower solar-fraction sites sit here. D1/D2 + VPD is notable: it matches solar-assisted Config B at Kathmandu without any solar hardware — heat recovery alone delivers roughly the same magnitude of savings as a 10 m² collector at a moderate-irradiance site.

Tier 3 — Mediocre performers (SEC 0.35–0.55 avg):

Config B (solar + HP series), D3 (swapped HRX), and C2 sit here. B is climate-dependent — excellent at BTN, average at KTM. D3 loses its evaporator-side COP advantage to the humidity penalty at the chamber inlet. C2 works but is strictly inferior to B.

Tier 4 — Worst performers:

Config A and C1 are at the bottom. A is the minimum-capital reference; C1 fails entirely at r = 0 (drying stalls at low T_evap because the condenser cannot reach T_set from cool ambient alone). C1 is a cautionary tale rather than a candidate system.

## 7.3  Practical Recommendations

Configuration choice depends on site conditions, operational schedule, and capital budget. The following recommendations distil the study findings into concrete guidance.

For high-irradiance sites (Biratnagar-like):

E2 + VPD (10 m²): lowest SEC (0.097 kWh/kg), strong solar fraction (0.91), compact footprint. Preferred when roof area ≥ 10 m² is available.

E2 (10 m²) without VPD: if control complexity is a concern; 0.129 kWh/kg, simpler operation.

B (10 m²): if HRX retrofit is not feasible; 0.287 kWh/kg, simplest solar-assisted layout.

For moderate-irradiance sites (Kathmandu-like):

E2 + VPD (10 m²): still the best choice; 0.144 kWh/kg, SF ≈ 0.78.

D2 + VPD: if solar is unavailable or unreliable; 0.288 kWh/kg, fully weather-independent.

D1 + VPD: marginally higher SEC than D2 but simpler evaporator plumbing (no exhaust routing to evap).

For continuous / night operation:

D1 + VPD or D2 + VPD: solar contribution drops to zero at night; HRX provides constant, climate-independent energy recovery. Operating at night with E-series configurations collapses performance to D-series levels.

For minimum-capital retrofits:

VPD bypass on Config A (r = 0.9): 7% SEC reduction at zero hardware cost — purely a controls change.

HRX alone (D1): the biggest single energy saving from one piece of hardware (≈50% SEC reduction vs A at KTM). If only one retrofit is possible, this is it.

## 7.4  Key Design Takeaways

1. Heat recovery outperforms solar in capital-efficiency.

At Kathmandu, a 10 m² solar collector (Config B) brings SEC from 0.717 to 0.488. An HRX (Config D1) brings the same baseline down to 0.365 — a larger improvement from a simpler, cheaper device. Where both are feasible they are additive; where only one is, the HRX is the better first buy.

2. Solar placement matters as much as solar area.

C1, C2, B, and E all use the same 10 m² collector but achieve SEC from 0.894 down to 0.097 depending on where the collector sits in the air loop. Placing the collector on the cold-side (condenser inlet) preserves collector efficiency and maximises useful gain. Placing it on already-warm air (E3 after condenser, C1 before evaporator) is wasteful or counter-productive.

3. Humidity at the chamber inlet dominates kinetics.

The Midilli α_RH = 1.75 humidity exponent means that any design choice raising chamber inlet RH has a direct, nonlinear drying-time penalty. This is why D3 (high COP but humid inlet) loses to D1/D2, and why the adaptive-T_evap investigation (Section 4) produced a negative result. For humidity-sensitive products, dehumidification is not a luxury — it is the primary driver of drying rate.

4. VPD bypass is nearly-free optimisation.

Exhaust bypass at threshold = 0.05 delivers 15–31% SEC reduction across all compatible configurations with <10% additional drying time, and requires only a single damper and a humidity sensor. It is the cheapest efficiency intervention in the study.

5. COP alone is an incomplete metric.

C2 achieves COP = 7.76 at BTN (highest in the study) but SEC = 0.381 kWh/kg — worse than E1’s 0.141 kWh/kg with COP = 4.43. High COP wasted on redundant heating (because the solar collector has already raised the air to T_set) does not translate to low SEC. System-level SEC, not component-level COP, is the correct optimisation target.

# 8  Limitations and Assumptions

The results presented in this study are subject to several simplifying assumptions. Understanding their scope is essential for interpreting the SEC numbers and for framing follow-up experimental validation work.

## 8.1  Weather and Climate Data

Single TMY (typical meteorological year) dataset per location. Year-to-year variability in irradiance, temperature, and humidity is not captured. Real-world SEC for a specific batch may differ depending on that year’s weather.

Hourly resolution interpolated to the simulation timestep. Sub-hourly cloud transients (minutes-scale intermittency) are smoothed. The Hottel–Whillier–Bliss quasi-steady model assumes the collector reaches steady state within each timestep; in reality, thermal inertia of the absorber and header damps short fluctuations.

Only three Nepal locations analysed (KTM, BTN, TPJ). Generalisation to sites with substantially different climates (e.g., coastal humid tropics, arid continental) requires re-running with local TMY data.

## 8.2  Thermal and Flow Simplifications

No duct or chamber wall losses. All heat delivered by the condenser is assumed to reach the product. In practice, insulation quality, duct runs, and chamber wall thermal mass would absorb 5–15% of delivered heat.

Constant fan power. The fan is modelled as a fixed consumption term; real fans have load-dependent efficiency and flow-rate coupling to duct pressure drop, which scales with air path complexity (higher in D/E with HRX).

No startup transients or thermal mass of the dryer structure. Each timestep is treated as steady-state. For batch drying of several hours, startup transients are a minor (<5%) contribution but not zero.

HRX modelled as a fixed-effectiveness (ε = 0.70) counter-flow exchanger. Fouling, condensation-induced performance degradation, and frost on the cold side are not modelled.

No condensation-to-liquid mass tracking on the HRX cold stream. When exhaust air cools below its dew point in the HRX, latent heat release is accounted for energetically, but the dropout water is not tracked as a separate output.

## 8.3  Product and Kinetics

Fixed apple slice geometry (6 mm thickness). Thinner slices would dry faster with different K/Ea values; thicker slices would be diffusion-limited and require a shrinkage model. Cross-product generalisation (e.g., to cardamom, chilli, ginger) requires re-fitting the Midilli kinetic parameters.

Uniform tray loading assumed. No spatial gradients within trays or between trays are modelled; all ten trays are treated as receiving identical air conditions.

Midilli kinetics (K_ref = 1.63 × 10⁻⁴ /s, Ea/R = 2711 K, α_RH = 1.75) fitted from phase-2 experimental data with R² = 0.90. The 10% unexplained variance propagates into the drying-time predictions.

No product quality metrics (colour, vitamin retention, rehydration capacity) are simulated. SEC optimisation is purely an energy-efficiency exercise; thermal damage from prolonged drying or humid chambers is not penalised beyond the kinetic slowdown.

## 8.4  Heat Pump Cycle

Fixed compressor isentropic efficiency (η_is = 0.75). Real scroll/rotary compressors have efficiency curves that vary with pressure ratio; at high T_cond or low T_evap, η_is typically drops to 0.65–0.70.

R134a refrigerant assumed throughout. Other refrigerants (R290, R744, R1234yf) would shift optimal operating envelopes and might change configuration rankings at the extremes of the operating window.

Variable-speed compressor with infinite turndown assumed. Real systems have minimum stable speeds and may cycle on/off, incurring cycling losses that are not captured here.

Fixed evaporator and condenser effectiveness (ε_evap, ε_cond) with a constant approach temperature. Real coil behaviour depends on air-side fouling, refrigerant charge, and partial condensation at the evaporator.

## 8.5  What the Model Does Not Address

Capital cost is not quantified. The rankings above assume hardware exists; practical decisions must weigh the capital cost of a 10 m² solar collector or an HRX against the SEC reduction achieved and the electricity tariff.

Maintenance burden is not modelled. HRX and solar collectors require periodic cleaning; compressor service intervals scale with operating hours.

Control system realism. The VPD bypass, solar-priority control (E3), and dynamic evaporator mixing (D2/E2) are modelled as idealised state-machines; real deployments would face sensor noise, actuation delays, and mode-chattering that the 600 s hysteresis dwell partially but not fully mitigates.

These limitations do not change the qualitative conclusions of the study — the performance hierarchy E > D > B > C > A is robust to all of them. They do, however, mean the absolute SEC numbers should be interpreted as best-case engineering estimates. Validation against a physical prototype is the logical next step and would primarily refine: (a) HRX effectiveness under realistic condensation, (b) duct losses for the larger E-series air paths, and (c) compressor efficiency curves across the operating envelope.