# Results and Discussion

This chapter presents the simulation results for ten SAHPD configurations across three contrasting Nepali climates: Kathmandu (highland basin), Biratnagar (tropical lowland) and Taplejung (mid-altitude hills). The sections are sequenced as a design narrative. Config A establishes the heat-pump-only baseline. Config B introduces flat-plate solar in series. Config C explores alternative solar placements that expose an ordering principle. Section 4 tackles the late-drying inefficiency uncovered in the recirculation analysis. Config D adds a waste-heat recovery exchanger (HRX) that works day and night, and Config E combines HRX with solar to reach the lowest specific energy consumption observed in the study. Section 7 ranks every configuration on a single axis and converts the findings into site-specific recommendations, and Section 8 documents the modelling assumptions that bound these conclusions.

Every number in every table is taken directly from the simulation outputs. Model inputs (collector optical and loss coefficients, evaporator and condenser effectiveness, HRX effectiveness, compressor isentropic efficiency, and the fitted Midilli drying kinetics) are documented once at the point of first use and repeated in the limitations section (Section 8). No literature values are cited in the results narrative unless they are also inputs to the code.

# 1  Config A: Heat-Pump-Only Baseline

## 1.1  System Description and Air Paths

Config A is the reference convective heat-pump dryer with no solar assistance and no heat recovery. It therefore isolates the performance ceiling of a vapour-compression cycle driving a ten-tray chamber, and provides the benchmark against which every subsequent configuration is measured. The system operates in one of two regimes depending on the recirculation ratio r.

Open-loop mode (r = 0)

[ DIAGRAM: Config A open-loop air path ]

Figure 1. Config A open-loop (r = 0) schematic. Process air and evaporator air are separate streams.

Ambient air enters the condenser, is heated to the set-point T_set = 45 °C, passes through the drying chamber where it picks up moisture, and is exhausted. No dehumidification takes place in the process loop; chamber-inlet humidity equals ambient humidity. A separate ambient stream feeds the evaporator, and the evaporator saturation temperature adapts to local conditions every timestep as T_evap = T_amb − 10 K (a constant 10 K approach between the source air and the refrigerant). The compressor therefore lifts the refrigerant from T_evap to T_cond = T_set + 10 = 55 °C, a total lift of 65 − T_amb K. Warmer ambient air shrinks the lift and, all else equal, raises the cycle COP.

Closed-loop mode (r > 0)

[ DIAGRAM: Config A closed-loop air path ]

Figure 2. Config A closed-loop (r > 0) schematic. Recirculated exhaust mixes with fresh ambient air, is dehumidified at the evaporator, then reheated at the condenser before re-entering the chamber.

A fraction r of the chamber exhaust is mixed with fresh ambient air upstream of the evaporator on enthalpy and humidity ratio (ω_mix = r·ω_exh + (1−r)·ω_amb; h_mix similarly). The mixed stream is cooled against a coil held at T_coil = T_evap + 3 K, and with effectiveness ε_evap = 0.85 this determines the evaporator outlet state. If the outlet falls below the mixed stream's dew point, moisture condenses and the air exits saturated. The cold, dehumidified stream is then reheated to T_set by the condenser under a first-law closure (Q_cond = Q_evap + W_comp).

A single modelling choice drives almost every closed-loop result in this chapter. In open-loop mode T_evap tracks the ambient (T_amb − 10 K), whereas in closed-loop it is pinned at 5 °C regardless of ambient. This is a deliberate simplification that reflects how most commercially available vapour-compression dehumidifiers are specified. The asymmetry produces the recirculation crossover analysed in Section 1.3.

## 1.2  Open-Loop Baseline (r = 0)

Table 1. Config A open-loop (r = 0) annual-TMY performance across the three locations.

| Location | T_amb (°C) | Time (h) | SEC (kWh/kg) | W_comp (kWh) | W_fan (kWh) | Q_cond (kWh) | COP |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Kathmandu | 15.7 | 13.85 | 0.717 | 13.52 | 0.34 | 49.0 | 3.63 |
| Biratnagar | 24.4 | 14.45 | 0.543 | 10.13 | 0.36 | 44.4 | 4.43 |
| Taplejung | 16.8 | 13.97 | 0.566 | 10.61 | 0.34 | 41.6 | 3.92 |

Annual-TMY SEC ranges from 0.543 kWh/kg at Biratnagar to 0.717 kWh/kg at Kathmandu, a 32 % climate-driven spread for the same physical hardware and the same product load. Taplejung sits close to Kathmandu at 0.566 kWh/kg. Two effects account for this spread. First is the temperature lift. Biratnagar's 24.4 °C mean ambient gives a 40.6 K lift and COP = 4.43, whereas Kathmandu's 15.7 °C mean ambient produces a 49.3 K lift and COP = 3.63. The second is site pressure. Biratnagar (72 m, about 100.5 kPa) has a denser air mass flow per unit volumetric flow than Kathmandu (1 350 m, about 86.1 kPa) or Taplejung (1 790 m, about 81.8 kPa), which slightly improves the volumetric heat-transfer coefficient at the coils.

Drying time on the annual TMY is within 0.6 h across the three sites (13.85 to 14.45 h). This near-constancy is expected. The chamber-inlet temperature is held at 45 °C at every timestep, and inlet humidity ratio (ω_amb) is low enough at all three sites that the α_RH = 1.75 suppression term in the Midilli kinetics does not significantly decelerate the rate. The differences between sites are therefore channelled almost entirely into compressor work, which is the metric that SEC exposes.

Table 2. Config A (r = 0) seasonal breakdown. Annual rows correspond to the TMY file; autumn/winter/spring use the seasonal splits.

| Location | Season | T_amb (°C) | Time (h) | SEC (kWh/kg) | W_comp (kWh) | Q_cond (kWh) | COP |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Kathmandu | Annual TMY | 15.7 | 13.85 | 0.717 | 13.52 | 49.0 | 3.63 |
| Kathmandu | Autumn (Oct–Nov) | 15.1 | 16.93 | 0.500 | 9.25 | 42.3 | 4.59 |
| Kathmandu | Winter (Dec–Jan) | 9.6 | 13.40 | 0.588 | 11.05 | 42.8 | 3.89 |
| Kathmandu | Spring (Mar–Apr) | 15.5 | 13.37 | 0.487 | 9.10 | 38.0 | 4.21 |
| Biratnagar | Annual TMY | 24.4 | 14.45 | 0.543 | 10.13 | 44.4 | 4.43 |
| Biratnagar | Autumn (Oct–Nov) | 23.4 | 20.03 | 0.435 | 7.88 | 42.9 | 5.48 |
| Biratnagar | Winter (Dec–Jan) | 17.7 | 15.45 | 0.427 | 7.86 | 38.9 | 4.99 |
| Biratnagar | Spring (Mar–Apr) | 26.2 | 13.53 | 0.367 | 6.76 | 33.5 | 5.04 |
| Taplejung | Annual TMY | 16.8 | 13.97 | 0.566 | 10.61 | 41.6 | 3.92 |
| Taplejung | Autumn (Oct–Nov) | 16.0 | 17.07 | 0.457 | 8.40 | 39.1 | 4.67 |
| Taplejung | Winter (Dec–Jan) | 10.3 | 14.80 | 0.622 | 11.67 | 45.1 | 3.87 |
| Taplejung | Spring (Mar–Apr) | 17.2 | 14.02 | 0.451 | 8.38 | 36.0 | 4.32 |

The seasonal breakdown contains two findings that the annual figures obscure. First, drying time is not a pure function of chamber-inlet temperature. At Biratnagar, autumn takes 20.0 h versus 13.5 h in spring despite near-identical chamber conditions, because the 23.4 °C autumn ambient carries considerably more water than the 26.2 °C spring ambient. The humidity-sensitivity exponent α_RH = 1.75 converts this ω difference into a direct drying-rate penalty, and the same mechanism resurfaces in Sections 4 and 5 when evaluating evaporator-bypass strategies. Second, the best SEC at every site is observed in spring (0.487 at KTM, 0.367 at BTN, 0.451 at TPJ), because spring offers the most favourable combination of warm ambient (high COP) and low water content (short drying time). Autumn and winter each fail one of these criteria.

## 1.3  Effect of Recirculation

Table 3. Config A recirculation sweep at annual TMY. ΔSEC is measured against each site's own r = 0 baseline.

| Location | r | Time (h) | SEC (kWh/kg) | W_comp (kWh) | Q_cond (kWh) | COP | ΔSEC vs r=0 (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Kathmandu | 0.0 | 13.85 | 0.717 | 13.52 | 49.0 | 3.63 | +0.0 |
| Kathmandu | 0.9 | 14.92 | 0.669 | 12.35 | 49.9 | 4.04 | -6.7 |
| Kathmandu | 1.0 | 15.22 | 0.674 | 12.45 | 50.3 | 4.04 | -5.9 |
| Biratnagar | 0.0 | 14.45 | 0.543 | 10.13 | 44.4 | 4.43 | +0.0 |
| Biratnagar | 0.9 | 14.63 | 0.753 | 13.98 | 56.5 | 4.04 | +38.8 |
| Biratnagar | 1.0 | 14.73 | 0.752 | 13.95 | 56.4 | 4.04 | +38.6 |
| Taplejung | 0.0 | 13.97 | 0.566 | 10.61 | 41.6 | 3.92 | +0.0 |
| Taplejung | 0.9 | 15.23 | 0.646 | 11.91 | 48.1 | 4.04 | +14.1 |
| Taplejung | 1.0 | 15.40 | 0.647 | 11.92 | 48.2 | 4.04 | +14.3 |

Recirculation is not a universally beneficial knob. At Kathmandu it trims SEC by 6.7 % at r = 0.9; at Biratnagar it inflates SEC by 38.7 %; at Taplejung it sits in between at +14.1 %. The sign of the effect is set by the evaporator temperature, which, as noted in Section 1.1, is ambient-adaptive in open-loop and fixed at 5 °C in closed-loop. The open-loop T_evap equals the closed-loop T_evap precisely when T_amb = 15 °C, and this threshold emerges cleanly in the data. Kathmandu's 15.7 °C annual mean sits only slightly above the crossover, so switching to the fixed 5 °C T_evap raises the evaporator operating point and improves COP. Biratnagar's 24.4 °C mean sits 9 K above the crossover, so recirculation depresses T_evap and degrades COP. Taplejung's 16.8 °C mean sits just above, producing a moderate penalty.

A second, reinforcing effect operates through the condenser inlet. At r = 0 the condenser receives air at T_amb, whereas at r = 0.9 it receives the evaporator outlet at roughly 12 °C. For Biratnagar this means the condenser must now lift 33 K instead of 21 K (a 55 % increase in Q_cond) even though the lift per unit mass is smaller. For Kathmandu the penalty is muted because 12 °C is only a few kelvin below the 15.7 °C ambient. The W_comp column in Table 3 confirms this: at Biratnagar, compressor work jumps from 10.1 kWh (r = 0) to 14.0 kWh (r = 0.9) despite an identical product load.

Moving from r = 0.9 to r = 1.0 changes SEC by at most 1 % at every site. Once recirculation dominates, the residual sensitivity to the fresh-air fraction is minor because the energy balance is dominated by the internal cycle. Practically, r = 0.9 is the relevant closed-loop operating point for the subsequent sections and is used throughout the optimisation analyses in Section 4.

# 2  Config B: Solar + Heat-Pump in Series

## 2.1  System Description and Air Paths

Config B adds a single flat-plate solar collector in series with the heat-pump condenser. The collector always sits immediately upstream of the condenser on the process loop so that any free useful gain is applied first and the compressor only has to make up the deficit to T_set. Baseline collector area is 10 m²; a 5 m² area is also evaluated at Kathmandu to size the sensitivity to collector area.

Open-loop mode (r = 0)

[ DIAGRAM: Config B open-loop air path ]

Figure 3. Config B open-loop schematic. Ambient air is preheated in the solar collector, topped up to T_set at the condenser, dries the product, and is exhausted. The evaporator draws a separate ambient stream (T_evap = T_amb − 10 K).

Ambient air flows through the collector, picks up solar useful gain (computed with the Hottel, Whillier, Bliss first-order loss model using τα = 0.75 and U_L = 5.0 W/m²K, cast in NTU form), and enters the condenser at T_after_solar. When T_after_solar is already above T_set the condenser switches off and the inlet is clipped to T_set by ambient bypass (no compressor work incurred). Otherwise the condenser provides the residual lift up to T_set = 45 °C. The evaporator runs on a parallel ambient stream so the compressor COP tracks ambient in the same way as Config A open-loop.

Closed-loop mode (r > 0)

[ DIAGRAM: Config B closed-loop air path ]

Figure 4. Config B closed-loop schematic. Recirculated exhaust mixes with ambient, is dehumidified at the evaporator, then picks up solar gain before final condenser heating.

A fraction r of the chamber exhaust is mixed with fresh ambient air on enthalpy and humidity ratio, passed through the evaporator (fixed coil at T_evap = 5 °C, ε_evap = 0.85) where dehumidification occurs, then routed through the solar collector, and finally through the condenser to reach T_set. This ordering has a specific physical consequence: the solar collector sees cold, partially saturated air leaving the evaporator rather than warmer ambient, which amplifies the useful gain because collector losses scale with (T_absorber − T_amb). The evaporator is the only component with a hard operating-point pin in Config B; every other temperature in the process loop is self-consistent with the first-law closure Q_cond = Q_evap + W_comp.

## 2.2  Annual Primary Case (r = 0, 10 m²)

Table 4. Config B annual-TMY performance (r = 0, 10 m² collector) across the three locations. ΔSEC is the reduction relative to each site's own Config A (r = 0) annual result.

| Location | T_amb (°C) | Time (h) | SEC (kWh/kg) | W_comp (kWh) | Q_solar (kWh) | Q_cond (kWh) | Solar frac | COP | ΔSEC vs A (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Kathmandu | 15.7 | 13.85 | 0.488 | 9.09 | 15.9 | 32.9 | 0.627 | 3.63 | 32.0 |
| Biratnagar | 24.4 | 14.45 | 0.287 | 5.18 | 22.8 | 22.6 | 0.804 | 4.43 | 47.1 |
| Taplejung | 16.8 | 13.97 | 0.344 | 6.32 | 16.8 | 24.5 | 0.716 | 3.92 | 39.2 |

At 10 m² collector area the annual-TMY SEC drops to 0.488 kWh/kg at Kathmandu, 0.287 at Biratnagar, and 0.344 at Taplejung. Compared against the Config A open-loop baseline of the same site, these represent reductions of 32 %, 47 % and 39 % respectively. The driver is visible in the Q_solar column: 15.9 kWh delivered to Kathmandu, 22.8 kWh to Biratnagar, and 16.8 kWh to Taplejung over the drying run. The Biratnagar solar fraction reaches 0.80 (80 % of the total energy input to the air comes from the collector), Taplejung 0.72, and Kathmandu 0.63.

The ranking of the three sites reverses between Config A and Config B. In Config A, Biratnagar led because its warm ambient produced the highest COP; the climate penalty at Kathmandu was attributed almost entirely to the larger compressor lift. In Config B, Biratnagar leads by an even wider absolute margin (0.287 vs 0.488 at KTM) because the two advantages compound: the same warm ambient raises COP, and the same tropical climate also delivers more annual GHI. Taplejung's intermediate position reflects a tradeoff, its high altitude gives more direct-beam irradiance per year but its cooler air depresses COP.

Drying time stays close to the Config A range (13.85 to 14.45 h, within 0.7 h of the baseline) because the chamber-inlet temperature is still held at 45 °C at every timestep. Solar therefore does not shorten the drying run; it shifts the energy bill from the compressor to the collector.

## 2.3  Seasonal Variation

Table 5. Config B seasonal breakdown at r = 0, 10 m² collector.

| Location | Season | T_amb (°C) | Time (h) | SEC (kWh/kg) | W_comp (kWh) | Q_solar (kWh) | Solar frac | COP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Kathmandu | Annual TMY | 15.7 | 13.85 | 0.488 | 9.09 | 15.9 | 0.627 | 3.63 |
| Kathmandu | Autumn (Oct–Nov) | 15.1 | 16.93 | 0.289 | 5.17 | 24.5 | 0.814 | 4.53 |
| Kathmandu | Winter (Dec–Jan) | 9.6 | 13.40 | 0.295 | 5.38 | 22.6 | 0.798 | 3.90 |
| Kathmandu | Spring (Mar–Apr) | 15.5 | 13.37 | 0.182 | 3.20 | 33.2 | 0.904 | 4.15 |
| Biratnagar | Annual TMY | 24.4 | 14.45 | 0.287 | 5.18 | 22.8 | 0.804 | 4.43 |
| Biratnagar | Autumn (Oct–Nov) | 23.4 | 20.03 | 0.267 | 4.65 | 23.1 | 0.818 | 5.45 |
| Biratnagar | Winter (Dec–Jan) | 17.7 | 15.45 | 0.240 | 4.25 | 19.8 | 0.811 | 4.98 |
| Biratnagar | Spring (Mar–Apr) | 26.2 | 13.53 | 0.128 | 2.14 | 33.9 | 0.932 | 5.02 |
| Taplejung | Annual TMY | 16.8 | 13.97 | 0.344 | 6.32 | 16.8 | 0.716 | 3.92 |
| Taplejung | Autumn (Oct–Nov) | 16.0 | 17.07 | 0.243 | 4.27 | 27.9 | 0.856 | 4.63 |
| Taplejung | Winter (Dec–Jan) | 10.3 | 14.80 | 0.379 | 6.96 | 18.9 | 0.721 | 3.87 |
| Taplejung | Spring (Mar–Apr) | 17.2 | 14.02 | 0.212 | 3.75 | 28.7 | 0.875 | 4.25 |

Spring delivers the lowest SEC at every site (KTM 0.182, BTN 0.128, TPJ 0.212 kWh/kg), outperforming the annual figure by a factor of two to three. Solar fraction climbs above 0.90 at Kathmandu and Biratnagar in spring because clear-sky days combine with long photoperiods, so the collector alone covers nearly all of the heating load. Autumn at Biratnagar is the notable counter-example. Drying time stretches to 20.0 h despite a 23.4 °C ambient and a healthy 0.82 solar fraction, because post-monsoon humidity inflates ω and the α_RH = 1.75 Midilli term slows the drying rate. SEC still remains low (0.267 kWh/kg) because the extended run hours are mostly during daylight when the collector is active.

Winter behaviour differs across sites. Kathmandu and Biratnagar winters finish in about 13 to 15 h at SEC ≈ 0.24 to 0.30 kWh/kg because short days are compensated by low ambient humidity (fast kinetics). Taplejung winter is the slowest and most expensive at 0.379 kWh/kg, driven by the combination of low ambient (depressed COP) and shorter photoperiod at altitude. Solar fraction at TPJ in winter is 0.72, several points below KTM winter (0.80) at the same collector area.

## 2.4  Effect of Recirculation

Table 6. Config B recirculation sweep at annual TMY, 10 m² collector. ΔSEC is measured against each site's own r = 0 baseline.

| Location | r | Time (h) | SEC (kWh/kg) | W_comp (kWh) | Q_solar (kWh) | Solar frac | COP | ΔSEC vs r=0 (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Kathmandu | 0.0 | 13.85 | 0.488 | 9.09 | 15.9 | 0.627 | 3.63 | +0.0 |
| Kathmandu | 0.3 | 31.05 | 0.392 | 6.37 | 30.6 | 0.802 | 3.80 | -19.7 |
| Kathmandu | 0.5 | 30.13 | 0.416 | 6.90 | 27.3 | 0.772 | 3.84 | -14.7 |
| Kathmandu | 0.7 | 28.98 | 0.460 | 7.84 | 23.0 | 0.720 | 3.90 | -5.7 |
| Kathmandu | 0.8 | 16.00 | 0.487 | 8.78 | 15.4 | 0.622 | 4.04 | -0.2 |
| Kathmandu | 0.9 | 14.92 | 0.469 | 8.49 | 15.3 | 0.628 | 4.04 | -3.8 |
| Kathmandu | 1.0 | 15.22 | 0.477 | 8.62 | 15.2 | 0.622 | 4.04 | -2.3 |
| Biratnagar | 0.0 | 14.45 | 0.287 | 5.18 | 22.8 | 0.804 | 4.43 | +0.0 |
| Biratnagar | 0.3 | 27.50 | 0.568 | 9.92 | 30.0 | 0.732 | 4.04 | +98.0 |
| Biratnagar | 0.5 | 26.88 | 0.618 | 10.91 | 28.1 | 0.702 | 4.04 | +115.4 |
| Biratnagar | 0.7 | 14.77 | 0.431 | 7.76 | 25.6 | 0.755 | 4.04 | +50.4 |
| Biratnagar | 0.8 | 14.55 | 0.423 | 7.60 | 25.5 | 0.758 | 4.04 | +47.4 |
| Biratnagar | 0.9 | 14.63 | 0.423 | 7.59 | 25.4 | 0.757 | 4.04 | +47.3 |
| Biratnagar | 1.0 | 14.73 | 0.423 | 7.60 | 25.3 | 0.756 | 4.04 | +47.5 |

Recirculation behaves very differently in Config B than in Config A, and the most striking feature of the sweep is not the SEC value itself but the drying time. At both Kathmandu and Biratnagar, moderate recirculation pushes drying from a one-day run of roughly 14 to 15 h into an extended two-day run of 27 to 31 h, and then collapses back to a one-day run once r exceeds a site-specific threshold. The threshold itself is physically meaningful. At Biratnagar the transition occurs between r = 0.6 (26.4 h) and r = 0.7 (14.8 h); at Kathmandu it shifts higher, to between r = 0.7 (29.0 h) and r = 0.8 (16.0 h). The transition marks the recirculation ratio above which the evaporator removes enough water per pass to keep the chamber inlet dry enough to sustain drying through the night, when the collector is idle.

The cycle interaction that causes this bifurcation is absent from Config A. When r increases, the chamber exhaust mixes with ambient air on ω and h, and the evaporator is forced to dehumidify the warmer, moister mix. Because the coil is pinned at 5 °C, the evaporator outlet is always close to 8 °C regardless of inlet, and very cold air enters the solar collector. The large (T_absorber − T_air_in) then shifts the collector operating point towards higher absorber temperature, which through the U_L loss term reduces collector efficiency. At Kathmandu the ambient dew point is low enough that the evaporator only lightly dehumidifies, and the compressor savings dominate at moderate r (SEC drops to 0.386 kWh/kg at r = 0.2, a 21 % improvement on the r = 0 baseline). At Biratnagar the much higher ambient ω means the evaporator removes large amounts of water, the evaporator outlet is near-saturated at 8 °C, the collector loses much of its useful gain, and drying still stretches into a second day where nighttime humidity further slows the rate. SEC therefore inflates by +47 to +133 % across the Biratnagar sweep, peaking near r = 0.6.

Once the sweep crosses the one-day threshold (r ≥ 0.7 at Biratnagar, r ≥ 0.8 at Kathmandu), SEC settles to a narrow band: 0.47 to 0.49 kWh/kg at KTM, 0.42 kWh/kg at BTN. These values are close to the open-loop Config B SEC at Kathmandu but still 47 % above the open-loop Biratnagar baseline, because the evaporator is still doing latent work that the climate does not reward with faster drying. The condenser still reheats the air to 45 °C, and the drying kinetics are only weakly responsive to inlet ω in this regime, so the latent load is effectively wasted. This is the late-drying inefficiency that motivates the VPD-bypass strategy analysed in Section 4.

## 2.5  Preview of the VPD Bypass Benefit

Table 7. Config B VPD-bypass benefit at r = 0.9, annual TMY, 10 m² collector. Baseline is the plain closed-loop run at the same r; VPD is the oscillating condenser-direct bypass strategy analysed in Section 4.

| Location | Baseline t (h) | VPD t (h) | Baseline SEC | VPD SEC | Baseline W_comp | VPD W_comp | Baseline solar frac | VPD solar frac | ΔSEC (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Kathmandu | 14.92 | 17.68 | 0.469 | 0.343 | 8.49 | 5.94 | 0.628 | 0.660 | 27.0 |
| Biratnagar | 14.63 | 17.65 | 0.423 | 0.333 | 7.59 | 5.74 | 0.757 | 0.777 | 21.2 |
| Taplejung | 15.23 | 15.82 | 0.416 | 0.285 | 7.45 | 4.90 | 0.687 | 0.762 | 31.5 |

Letting the controller skip the evaporator whenever the exhaust vapour-pressure deficit is already comfortably above the drying threshold recovers most of the recirculation penalty. At r = 0.9 annual TMY, VPD-bypass cuts SEC by 27 % at Kathmandu, 21 % at Biratnagar, and 32 % at Taplejung. The full mechanism is developed in Section 4 together with the analogous Config D/E results, but the preview is useful here because it shows that the upward slope of the Biratnagar recirculation curve in Table 6 is not an intrinsic limitation of solar-assisted closed-loop drying; it is an artefact of running the evaporator unconditionally.

## 2.6  Collector Area Sweep (Kathmandu)

Table 8. Config B collector-area sensitivity at Kathmandu, r = 0, annual TMY.

| Area (m²) | Time (h) | SEC (kWh/kg) | W_comp (kWh) | Q_solar (kWh) | Solar frac | COP |
| --- | --- | --- | --- | --- | --- | --- |
| 5 | 13.85 | 0.589 | 11.05 | 8.8 | 0.437 | 3.63 |
| 10 | 13.85 | 0.488 | 9.09 | 15.9 | 0.627 | 3.63 |

Doubling collector area from 5 to 10 m² at Kathmandu reduces SEC from 0.589 to 0.488 kWh/kg, a 17 % improvement. Q_solar delivered rises from 8.8 to 15.9 kWh (+80 %) but the SEC improvement is much less than proportional because drying time is fixed by the kinetics at the same chamber conditions. Extra area primarily shifts energy from the compressor to the collector and raises the solar fraction from 0.44 to 0.63; beyond 10 m², further increases would hit diminishing returns unless paired with storage or a recirculation strategy that keeps useful load on the collector during off-peak periods (a point revisited in the thermal-storage discussion in Section 8).