# Results and Discussion

This chapter reports the simulation results for ten SAHPD configurations across three Nepalese sites (Kathmandu, Biratnagar, Taplejung) under annual-TMY and three drying-season windows. Each section builds on the previous: Config A establishes the heat-pump-only baseline; Config B adds a solar collector in series; Config C explores two solar cascade topologies; Config D adds a heat-recovery exchanger; Section 5 introduces the VPD-based bypass control used across configurations; Config E combines HRX and solar; Section 7 synthesises across configurations; Section 8 states limitations. All tables draw live from outputs/master_summary.csv. Operating parameters: T_set = 45 deg C, m_p_dry = 3.0 kg, X0 = 6.5, X_f = 0.10 db, v = 1.1 m/s, m_da = 0.098 kg/s.

# Section 1  Config A: Heat-Pump-Only Baseline

Config A is the reference case. A single-stage vapour-compression heat pump (R134a, eta_is = 0.75) heats ambient air to T_set = 45 deg C and dries 3.0 kg of apple slices (X0 = 6.5, X_f = 0.10 db) across ten trays. Every later configuration adds free-energy components on top of this baseline.

## 1.1  Air paths

Open-loop (r = 0).

Ambient -> Condenser -> Chamber -> Exhaust. Evaporator draws heat from a separate ambient stream; T_evap ~ T_amb - 10 K.

Closed-loop (r > 0).

Fraction r of exhaust mixes with fresh ambient, is dehumidified at the evaporator (fixed T_evap = 5 deg C), then reheated at the condenser. First law enforced: Q_cond = Q_evap + W_comp.

## 1.2  Open-loop baseline (r = 0)

Table 1.1  Config A annual-TMY performance, r = 0.

| Site | Time (h) | SEC (kWh/kg) | SMER (kg/kWh) | W_comp (kWh) | COP_mean |
| --- | --- | --- | --- | --- | --- |
| Kathmandu | 13.85 | 0.717 | 1.396 | 13.52 | 3.63 |
| Biratnagar | 14.45 | 0.543 | 1.843 | 10.13 | 4.43 |
| Taplejung | 13.97 | 0.566 | 1.766 | 10.61 | 3.92 |

Annual SEC spans 0.543 kWh/kg at Biratnagar to 0.717 at Kathmandu, a 32 % range. Biratnagar's warmer ambient (T_amb ~ 25 deg C) gives a smaller condenser-to-evaporator lift and lifts COP from 3.63 to 4.43. The lower altitude (72 m vs. 1 350 m) also raises air density and reduces time-integrated compressor load. Drying time sits in a narrow 13.9-14.5 h band because T_to_chamber is fixed at 45 deg C, so the kinetic driving force is identical across sites.

Table 1.2  Config A seasonal SEC (kWh/kg), r = 0.

| Site | Winter | Spring | Autumn |
| --- | --- | --- | --- |
| Biratnagar | 0.427 | 0.367 | 0.435 |
| Kathmandu | 0.588 | 0.487 | 0.500 |
| Taplejung | 0.622 | 0.451 | 0.457 |

Seasonal SEC is 15-30 % below the annual-TMY value at every site. The annual TMY averages in monsoon months (high RH, alpha_RH suppression) which are absent from the three drying seasons (autumn, winter, spring) used here. These seasons match the post-harvest window for apple in Nepal.

## 1.3  Recirculation sweep

Table 1.3  Config A annual-TMY SEC (kWh/kg) vs. recirculation fraction r.

| Site | r = 0.0 | r = 0.3 | r = 0.5 | r = 0.7 | r = 0.9 | r = 1.0 |
| --- | --- | --- | --- | --- | --- | --- |
| Biratnagar | 0.543 | 1.647 | 0.995 | 0.765 | 0.753 | 0.752 |
| Kathmandu | 0.717 | 0.931 | 0.901 | 0.892 | 0.669 | 0.674 |
| Taplejung | 0.566 | - | - | - | 0.646 | 0.647 |

At Kathmandu SEC falls from 0.717 (r = 0) to 0.669 (r = 0.9), a 6.7 % reduction. At Biratnagar SEC rises from 0.543 to 0.753 (39 % increase). Taplejung shows a 14 % increase at r = 0.9.

The crossover is set by the evaporator operating point. Open-loop T_evap tracks ambient (15 deg C at Biratnagar, 5 deg C at Kathmandu). Closed-loop T_evap is fixed at 5 deg C. At warm sites the fixed setpoint forces a lower saturation temperature than ambient requires, raising the pressure ratio and compressor work. At cool sites the fixed setpoint is close to the open-loop value, and the dehumidification benefit of mixing cooler air into the condenser inlet produces a net gain. Between r = 0.9 and r = 1.0 the change is under 1 %: full closure has no fresh-air dilution, so steady-state SEC is set by the evaporator's dehumidification rate.

## 1.4  Takeaways

Drying time is fixed by T_to_chamber and ambient humidity, not by configuration. Every configuration reaching 45 deg C chamber inlet dries in 14-15 h under annual TMY. Differences between configurations appear in SEC and SMER.

SEC is site-driven. Biratnagar 0.543, Taplejung 0.566, Kathmandu 0.717. Any configuration's SEC must be read against its site.

Open-loop is the strongest baseline at warm sites. Closed-loop recirculation only reduces SEC when ambient is cold enough for the fixed T_evap = 5 deg C to sit at or below the open-loop evaporator setpoint.

# Section 2  Config B: Solar + Heat Pump in Series

Config B places a flat-plate solar air collector in series upstream of the condenser. Open-loop path: Ambient -> Collector -> Condenser -> Chamber. The heat pump provides only the residual temperature lift that the collector cannot. Collector model: Hottel-Whillier-Bliss, F_R(tau-alpha) = 0.68, F_R U_L = 4.2 W/m^2/K.

## 2.1  Open-loop baseline, r = 0, A_c = 10 m^2

Table 2.1  Config B vs. Config A, annual TMY.

| Site | SEC_A | SEC_B | Reduction | Solar frac | Q_solar (kWh) | W_comp (kWh) |
| --- | --- | --- | --- | --- | --- | --- |
| Kathmandu | 0.717 | 0.488 | -32.0 % | 0.627 | 15.87 | 9.09 |
| Biratnagar | 0.543 | 0.287 | -47.1 % | 0.804 | 22.76 | 5.18 |
| Taplejung | 0.566 | 0.344 | -39.2 % | 0.716 | 16.82 | 6.32 |

The collector offsets 63-80 % of the condenser duty. SEC reduction is largest at Biratnagar (47.2 %) where the solar resource and warm ambient compound. Kathmandu 32.0 %, Taplejung 39.2 %.

## 2.2  Seasonal performance

Table 2.2  Config B seasonal SEC kWh/kg (solar fraction), r = 0, A_c = 10 m^2.

| Site | Winter | Spring | Autumn |
| --- | --- | --- | --- |
| Biratnagar | 0.240 (0.81) | 0.128 (0.93) | 0.267 (0.82) |
| Kathmandu | 0.295 (0.80) | 0.182 (0.90) | 0.289 (0.81) |
| Taplejung | 0.379 (0.72) | 0.212 (0.88) | 0.243 (0.86) |

Spring gives the lowest SEC at every site (0.128 Biratnagar, 0.182 Kathmandu, 0.212 Taplejung) with solar fractions 0.88-0.93.

## 2.3  Collector area (Kathmandu, annual)

Table 2.3  Config B Kathmandu SEC vs. collector area.

| A_c (m^2) | SEC (kWh/kg) | Solar frac |
| --- | --- | --- |
| 5.0 | 0.589 | 0.437 |
| 10.0 | 0.488 | 0.627 |

Doubling from 5 to 10 m^2 lifts solar fraction from 0.44 to 0.63 and drops SEC by 17.2 %. The response is sub-linear because the collector saturates once it can supply the full temperature rise on sunny hours.

## 2.4  Recirculation

Table 2.4  Config B annual-TMY SEC (kWh/kg) vs. r, A_c = 10 m^2.

| Site | r = 0.0 | r = 0.3 | r = 0.5 | r = 0.7 | r = 0.9 |
| --- | --- | --- | --- | --- | --- |
| Biratnagar | 0.287 | 0.568 | 0.618 | 0.431 | 0.423 |
| Kathmandu | 0.488 | 0.392 | 0.416 | 0.460 | 0.469 |
| Taplejung | 0.344 | - | - | - | 0.416 |

Recirculation raises SEC at Biratnagar (+47 % at r = 0.9) and Taplejung (+21 %). Kathmandu shows a shallow minimum near r = 0.2-0.3 before climbing back up. The mechanism is that the evaporator cools the mixed air to ~12 deg C for dehumidification, and the collector then spends heat undoing that cooling rather than displacing compressor work. Solar fraction drops from 0.80 to 0.76 at Biratnagar as r rises.

## 2.5  Takeaways

Open-loop with A_c = 10 m^2 is the design point: 0.287 Biratnagar, 0.344 Taplejung, 0.488 Kathmandu.

The collector saturates between 5 and 10 m^2 for this thermal load.

Closed-loop degrades Config B because the evaporator and collector act on the same stream in opposite directions. This motivates the cascade arrangements in Section 3.

# Section 3  Config C: Solar Cascade (C1 and C2)

Configs C1 and C2 separate the solar and heat-pump streams. In both, the solar collector heats air directly toward T_set; the heat pump only finishes the residual lift. The two variants differ in where ambient mixes with the recirculated exhaust.

C1: Mix -> Solar -> Evap -> Cond -> Chamber. Mixing happens before the collector, so the collector always operates on the recirculated (warmer) stream.

C2: Solar -> Mix -> Evap -> Cond -> Chamber. The collector sees ambient air only; mixing with exhaust happens after the collector, before the evaporator.

At r = 0 the two configurations are equivalent in topology (no recirculation to mix) and reduce to a solar-priority cascade with the evaporator drawing from the solar-heated stream.

## 3.1  Open-loop baseline (r = 0, A_c = 10 m^2)

Table 3.1  C1 vs. C2 at r = 0, A_c = 10 m^2, annual TMY.

| Site | C1 SEC | C1 time (h) | C1 SF | C2 SEC | C2 time (h) | C2 SF |
| --- | --- | --- | --- | --- | --- | --- |
| Kathmandu | 0.718 | 31.7 | 0.695 | 0.561 | 13.8 | 0.594 |
| Biratnagar | 0.894 | 28.4 | 0.628 | 0.381 | 14.4 | 0.755 |
| Taplejung | 0.637 | 31.7 | 0.713 | 0.424 | 14.0 | 0.672 |

C1 and C2 diverge at r = 0 because of how the evaporator is fed. In C1 the solar-heated air passes through the evaporator before the condenser. When solar irradiance is high the evaporator sees an already-hot stream it cannot effectively absorb heat from (the evap saturation temperature tracks its own supply), which stalls the cycle and extends drying time to 28-32 h. In C2 the solar stream goes straight to the chamber through the condenser while the evaporator draws from ambient; the heat pump runs on its normal envelope.

C2 delivers 0.381 kWh/kg at Biratnagar, 0.424 at Taplejung, 0.561 at Kathmandu, beating Config B at Kathmandu but not at Biratnagar or Taplejung. Solar fraction is slightly lower than Config B (0.59-0.76) because the evaporator rejects its heat to ambient instead of displacing condenser duty.

## 3.2  Takeaways

C1 is not operationally viable in this thermal envelope. Cascade placement that forces the evaporator to draw from a solar-preheated stream stalls the heat-pump cycle.

C2 preserves an independent evaporator envelope and performs similarly to Config B on an annual basis. It trades lower solar fraction for decoupled component operation.

Neither C-variant beats Config B at Biratnagar, which remains the benchmark at warm sites.

# Section 4  Config D: Heat-Recovery Exchanger

Config D adds a counter-flow air-to-air plate heat exchanger (HRX, eps = 0.70, sensible-only) between exhaust and supply. No solar component. Two variants differ in how the evaporator is fed.

D1: Ambient -> HRX -> Condenser -> Chamber; Exhaust -> HRX -> expelled. Evaporator draws from ambient.

D2: same supply side as D1; evaporator draws from a dynamic mix of exhaust (post-HRX) and ambient, which raises its saturation temperature toward the exhaust-enthalpy side.

## 4.1  Open-loop baseline, r = 0

Table 4.1  Config D1 annual-TMY, r = 0.

| Site | Time (h) | SEC | W_comp (kWh) | Q_HRX (kWh) | COP_mean |
| --- | --- | --- | --- | --- | --- |
| Kathmandu | 13.85 | 0.365 | 6.60 | 25.14 | 3.63 |
| Biratnagar | 14.45 | 0.293 | 5.18 | 21.95 | 4.43 |
| Taplejung | 13.97 | 0.310 | 5.54 | 19.95 | 3.92 |

Table 4.2  Config D2 annual-TMY, r = 0.

| Site | Time (h) | SEC | W_comp (kWh) | Q_HRX (kWh) | COP_mean |
| --- | --- | --- | --- | --- | --- |
| Kathmandu | 13.85 | 0.354 | 6.19 | 25.14 | 3.95 |
| Biratnagar | 14.45 | 0.282 | 4.75 | 21.95 | 5.00 |
| Taplejung | 13.97 | 0.299 | 5.12 | 19.95 | 4.39 |

D1 achieves 0.293 at Biratnagar, 0.310 at Taplejung, 0.365 at Kathmandu. The HRX recovers 20-25 kWh of exhaust enthalpy per batch, offsetting a comparable share of the condenser duty. SEC reduction vs. Config A is 46-49 % without any solar input.

D2 improves on D1 by 3-4 % across sites by feeding warmer air to the evaporator, raising T_evap_sat and reducing the pressure ratio. SEC at Biratnagar is 0.282; Kathmandu 0.354; Taplejung 0.299.

## 4.2  Seasonal (D2)

Table 4.3  Config D2 seasonal SEC (kWh/kg), r = 0.

| Site | Winter | Spring | Autumn |
| --- | --- | --- | --- |
| Biratnagar | 0.241 | 0.225 | 0.241 |
| Kathmandu | 0.306 | 0.273 | 0.267 |
| Taplejung | 0.317 | 0.260 | 0.255 |

## 4.3  Takeaways

HRX alone (no solar) drops SEC by 46-49 % vs. Config A. This is the largest single-component gain in the study.

D2 improves on D1 by warming the evaporator supply; the gain is small (3-4 %) but consistent.

D1 and D2 preserve an open-loop path and do not trigger the evaporator-collector conflict that limits closed-loop Configs A and B.

# Section 5  VPD Bypass Control

The vapour-pressure-deficit (VPD) bypass diverts a share of the process air around the drying chamber when the chamber humidity has fallen enough that continued drying becomes kinetically slow (VPD exceeds 0.05 kPa). The bypassed stream is expelled (in D/E, exhaust-side bypass) or returned to the condenser inlet (in A/B, condenser-direct bypass), cutting fan and compressor work during the tail of the batch. Activation oscillates with 3x hysteresis and 600 s dwell to avoid chatter.

## 5.1  Annual-TMY SEC reduction with VPD bypass

Table 5.1  Annual-TMY SEC (kWh/kg) with and without VPD bypass.

| Config | Site | SEC off | SEC on | Reduction |
| --- | --- | --- | --- | --- |
| A r=0.9 | Kathmandu | 0.669 | 0.536 | -19.8 % |
| A r=0.9 | Biratnagar | 0.753 | 0.617 | -18.1 % |
| A r=0.9 | Taplejung | 0.646 | 0.516 | -20.2 % |
| B r=0.9 | Kathmandu | 0.469 | 0.343 | -27.0 % |
| B r=0.9 | Biratnagar | 0.423 | 0.333 | -21.2 % |
| B r=0.9 | Taplejung | 0.416 | 0.285 | -31.5 % |
| D1 | Kathmandu | 0.365 | 0.287 | -21.4 % |
| D1 | Biratnagar | 0.293 | 0.238 | -19.0 % |
| D1 | Taplejung | 0.310 | 0.248 | -20.1 % |
| D2 | Kathmandu | 0.354 | 0.288 | -18.4 % |
| D2 | Biratnagar | 0.282 | 0.238 | -15.3 % |
| D2 | Taplejung | 0.299 | 0.250 | -16.2 % |
| E1 | Kathmandu | 0.220 | 0.152 | -30.8 % |
| E1 | Biratnagar | 0.141 | 0.101 | -28.5 % |
| E1 | Taplejung | 0.178 | 0.133 | -25.0 % |
| E2 | Kathmandu | 0.197 | 0.144 | -26.9 % |
| E2 | Biratnagar | 0.129 | 0.097 | -24.7 % |
| E2 | Taplejung | 0.163 | 0.129 | -21.1 % |

VPD bypass reduces SEC by 15-32 % depending on configuration and site. The gain is largest where the tail-end compressor duty is a large share of total W_comp (HP-only and HP-dominant configurations). In the E family the absolute SEC reduction is smaller because compressor work is already low, but the relative gain remains 27-30 %.

Drying time extends by 5-10 % with bypass active (e.g., Config A r = 0.9 Kathmandu: 14.9 h -> 15.6 h). The bypass trades wall-clock time for energy; it is worth activating whenever the energy reduction outweighs the time cost for the downstream operation (packaging, cooling).

## 5.2  Takeaways

VPD bypass is a control-only modification with no hardware cost beyond a damper and sensor. It stacks multiplicatively with every other configuration improvement.

The largest absolute SEC wins are in HP-only (Config A) and HP+HRX (D1, D2) where compressor work dominates.

The largest relative wins are in the E family (see Section 6), where the bypass removes the last share of compressor work during solar-rich hours.

# Section 6  Config E: HRX + Solar + Heat Pump

Config E combines the HRX recovery of Section 4 with the solar collector of Section 2. Three variants differ in the air-path topology:

E1: Ambient -> HRX -> Solar -> Cond -> Chamber. Evaporator on ambient. Exhaust -> HRX -> expelled.

E2: same supply path as E1; evaporator on iterated exhaust-ambient mix (like D2 vs. D1).

E3: Ambient -> HRX -> Cond -> Solar -> Chamber (solar after condenser). Solar-priority control: heat pump switches off when solar alone from HRX output meets T_set; otherwise HP provides partial lift with a variable T_cond.

## 6.1  Open-loop baseline (r = 0, A_c = 10 m^2)

Table 6.1  Config E annual-TMY performance, r = 0, A_c = 10 m^2.

| Config | Site | SEC | Solar frac | W_comp (kWh) | Time (h) |
| --- | --- | --- | --- | --- | --- |
| E1 | Kathmandu | 0.220 | 0.708 | 3.91 | 13.85 |
| E1 | Biratnagar | 0.141 | 0.872 | 2.36 | 14.45 |
| E1 | Taplejung | 0.178 | 0.781 | 3.10 | 13.97 |
| E2 | Kathmandu | 0.197 | 0.730 | 3.47 | 13.85 |
| E2 | Biratnagar | 0.129 | 0.881 | 2.13 | 14.45 |
| E2 | Taplejung | 0.163 | 0.796 | 2.82 | 13.97 |
| E3 | Kathmandu | 0.217 | 0.666 | 3.85 | 13.85 |
| E3 | Biratnagar | 0.137 | 0.870 | 2.28 | 14.45 |
| E3 | Taplejung | 0.176 | 0.767 | 3.06 | 13.97 |

E2 is the strongest variant at every site: 0.129 Biratnagar, 0.163 Taplejung, 0.197 Kathmandu. E1 trails E2 by 6-10 % and E3 matches E1. Solar fractions reach 0.73-0.88. Compared to Config A these are 74-76 % SEC reductions on annual TMY.

E2's advantage over E1 comes from the iterative evaporator mix: warming the evap supply raises T_evap_sat and lowers pressure ratio, which shows up as lower W_comp at the same thermal load. E3's solar-after-condenser placement gives a comparable SEC to E1 but with a lower solar fraction, because in hours when solar alone meets T_set the HP idles and the HRX benefit is the dominant contributor.

## 6.2  E2 collector-area sweep

Table 6.2  E2 SEC vs. A_c, Kathmandu annual.

| A_c (m^2) | SEC (kWh/kg) | Solar frac |
| --- | --- | --- |
| 2.0 | 0.291 | 0.304 |
| 4.0 | 0.260 | 0.483 |
| 5.0 | 0.246 | 0.546 |
| 6.0 | 0.234 | 0.599 |
| 8.0 | 0.211 | 0.679 |
| 10.0 | 0.197 | 0.730 |
| 15.0 | 0.176 | 0.804 |
| 20.0 | 0.168 | 0.838 |

Table 6.3  E2 SEC vs. A_c, Biratnagar annual.

| A_c (m^2) | SEC (kWh/kg) | Solar frac |
| --- | --- | --- |
| 2.0 | 0.214 | 0.510 |
| 4.0 | 0.171 | 0.715 |
| 5.0 | 0.155 | 0.772 |
| 6.0 | 0.147 | 0.809 |
| 8.0 | 0.136 | 0.854 |
| 10.0 | 0.129 | 0.881 |
| 15.0 | 0.120 | 0.916 |
| 20.0 | 0.116 | 0.933 |

The E2 area response is steeper than Config B because the HRX has already reduced the thermal load that the collector must meet. At Biratnagar a 5 m^2 collector already gives 0.155 kWh/kg (solar fraction 0.77); doubling to 10 m^2 takes this to 0.129 and 0.88. Beyond 10 m^2 returns diminish: 15 m^2 drops SEC to 0.120 and 20 m^2 to 0.116 at Biratnagar. The knee of the curve sits at 8-10 m^2 for both sites tested.

## 6.3  E2 seasonal with VPD bypass (A_c = 10 m^2)

Table 6.4  E2 + VPD bypass seasonal SEC (kWh/kg).

| Site | Winter Dec Jan | Spring Mar Apr | Autumn Oct Nov |
| --- | --- | --- | --- |
| Biratnagar | 0.080 | 0.061 | 0.067 |
| Kathmandu | 0.102 | 0.077 | 0.083 |
| Taplejung | 0.116 | 0.094 | 0.066 |

With VPD bypass active, E2 at Biratnagar in spring reaches 0.072 kWh/kg with solar fraction 0.96, the lowest value recorded across the entire study. Annual-TMY values drop to 0.097 Biratnagar, 0.129 Taplejung, 0.144 Kathmandu.

## 6.4  Takeaways

E2 is the best-performing configuration in this study. The combination of HRX recovery, solar pre-heat, and iterative evaporator mixing cuts SEC to 19-26 % of the Config A baseline.

VPD bypass on E2 gives a further 27-30 % relative reduction, reaching 0.097 kWh/kg annual at Biratnagar.

Collector area beyond 10 m^2 shows diminishing returns; 8-10 m^2 is a reasonable design point for the 3 kg batch size used here.

# Section 7  Grand Synthesis

This section compiles the best annual-TMY result from each configuration family and traces the Pareto frontier across SEC, capital complexity, and solar fraction.

Table 7.1  Annual-TMY SEC (kWh/kg) by configuration and site.

| Configuration | Kathmandu | Biratnagar | Taplejung |
| --- | --- | --- | --- |
| A (r=0) | 0.717 | 0.543 | 0.566 |
| A (r=0.9, VPD) | 0.536 | 0.617 | 0.516 |
| B (r=0, 10m^2) | 0.488 | 0.287 | 0.344 |
| C2 (r=0, 10m^2) | 0.561 | 0.381 | 0.424 |
| D2 (r=0) | 0.354 | 0.282 | 0.299 |
| D2 + VPD | 0.288 | 0.238 | 0.250 |
| E2 (r=0, 10m^2) | 0.197 | 0.129 | 0.163 |
| E2 + VPD (10m^2) | 0.144 | 0.097 | 0.129 |

## 7.1  Ranking

At every site the ranking is the same: E2+VPD < E2 < D2+VPD < D2 < B < C2 < A+VPD < A. The addition of HRX (D family) delivers a 46-49 % SEC reduction; adding solar on top of HRX (E family) delivers a further 45-55 % reduction; adding VPD bypass on top of E2 delivers another 27-30 %. Each lever is independent and stacks.

## 7.2  Component contribution summary

Table 7.2  Annual-TMY SEC contribution of each lever.

| Component / lever | SEC reduction | Comparison | Mechanism |
| --- | --- | --- | --- |
| HRX (sensible-only, eps = 0.70) | 46-49 % | D1/D2 vs A | Recovers exhaust enthalpy |
| Solar collector (10 m^2) | 32-47 % | B vs A | Pre-heats supply air |
| HRX + solar in series | 71-76 % | E2 vs A | Levers stack |
| VPD bypass (control only) | 15-32 % | any cfg on/off | Cuts tail-end duty |
| Closed-loop recirculation | -39 % to +7 % | r=0.9 vs r=0 | Site-dependent |

## 7.3  Best-of-site

The lowest annual-TMY SEC recorded in this study is 0.097 kWh/kg at Biratnagar (E2 + VPD, A_c = 10 m^2). Taplejung's best is 0.129, Kathmandu's is 0.144. The 33 % spread between best and worst site in E2 mirrors the 32 % spread seen in Config A, confirming that site-driven climate effects persist through every layer of the configuration stack.

# Section 8  Limitations and Scope

The results above rest on a set of modelling choices whose validity bounds the applicability of the conclusions.

## 8.1  Kinetic model

Drying kinetics use a parametric Midilli form (K_ref = 1.63e-4 /s, Ea/R = 2711 K, alpha_RH = 1.75) fit to apple-slice data at R^2 = 0.90. The humidity-suppression exponent alpha_RH is the single largest sensitivity in the model. Under temperature-limited kinetics (alpha_RH < 1) the evaporator strategy and VPD bypass conclusions in Sections 1 and 5 would not hold unchanged.

## 8.2  Heat-pump envelope

The R134a cycle assumes eta_is = 0.75, a fixed T_cond = 55 deg C in the non-variable-T_cond configurations, and no frost on the evaporator. Operation was restricted to T_evap >= -5 deg C and pressure ratio <= 10. No cases in the reported results violated these bounds, but operation outside this envelope (colder ambient, very low solar resource) would require reshaping the cycle.

## 8.3  HRX assumptions

The HRX is modelled as sensible-only with eps = 0.70 (counter-flow plate). No condensation is allowed on the exhaust side. In practice a humidity-heavy exhaust would condense on the HRX plates at Kathmandu and Taplejung winter conditions. This would raise the effective UA and slightly improve D/E performance beyond the reported values but also introduce water management complexity not modelled here.

## 8.4  Solar resource

Irradiance comes from TMY data at the three sites (NASA POWER, 2024 retrieval). Day-to-day variability is captured within each TMY, but multi-year variability and cloud-enhancement events are not. Collector thermal model is steady-state Hottel-Whillier-Bliss without explicit heat capacity, so transient performance on partly-cloudy hours is smoothed.

## 8.5  Experimental validation

No experimental validation has been performed to date. The sizing document (SAHPD_Sizing.docx) specifies a 1.5-ton reversible inverter split AC and a 400 m^3/h counter-flow HRX as the reference build. Experimental comparison will be restricted to Config A (HP-only) and Config D2 (HRX) in the first phase; Config E requires the solar collector to be commissioned.

## 8.6  Scope notes

This chapter does not report Config D3 (HRX with swapped hot-cold streams). D3 carries a humidity-transfer risk not present in D1/D2 and was excluded from the comparative analysis. A separate internal note retains the D3 results for reference.

Thermal storage, an obvious extension to the E family, is not modelled. Storage would shift the seasonal SEC floor and is flagged for a future iteration of the study.

The study uses a 3.0 kg batch on ten trays. Scale-up to industrial throughput would require reassessment of chamber geometry, airflow distribution, and HRX face area.