# Section 2  Config B: Solar + Heat Pump in Series

Config B places a flat-plate solar air collector in series upstream of the condenser. In open-loop (r = 0) the air path is Ambient -> Collector -> Condenser -> Chamber -> Exhaust; in closed-loop the recirculated exhaust is first dehumidified at the evaporator, then pre-heated by the collector, then brought to T_set = 45 deg C by the condenser. The heat pump provides only the residual temperature lift that the collector cannot, so compressor duty scales inversely with available irradiance. The collector uses the Hottel-Whillier-Bliss model with F_R(tau-alpha) = 0.68 and F_R U_L = 4.2 W/m^2/K.

## 2.1  Open-loop baseline (r = 0, A_c = 10 m^2)

Table 2.1  Config B annual-TMY performance vs. Config A, r = 0, A_c = 10 m^2.

| Site | SEC_A (kWh/kg) | SEC_B (kWh/kg) | SEC reduction | Solar fraction | Q_solar (kWh) | W_comp (kWh) |
| --- | --- | --- | --- | --- | --- | --- |
| Kathmandu | 0.717 | 0.488 | -32.0 % | 0.627 | 15.87 | 9.09 |
| Biratnagar | 0.543 | 0.287 | -47.1 % | 0.804 | 22.76 | 5.18 |
| Taplejung | 0.566 | 0.344 | -39.2 % | 0.716 | 16.82 | 6.32 |

The collector offsets 63-80 % of the condenser heat demand. SEC reduction is largest at Biratnagar (47.2 %) where both the solar resource (lower altitude, less cloud attenuation) and the high ambient temperature (smaller remaining heat-pump lift) compound. Kathmandu sees a 32.0 % reduction and Taplejung 39.2 %. Drying time changes by at most half an hour, consistent with the constraint identified in Section 1 that time is set by T_to_chamber and ambient RH rather than by the heat source.

## 2.2  Seasonal performance (r = 0, A_c = 10 m^2)

Table 2.2  Config B seasonal SEC (kWh/kg), r = 0, A_c = 10 m^2.

| Site | Winter | Spring | Autumn |
| --- | --- | --- | --- |
| Biratnagar | 0.240 | 0.128 | 0.267 |
| Kathmandu | 0.295 | 0.182 | 0.289 |
| Taplejung | 0.379 | 0.212 | 0.243 |

Table 2.3  Config B seasonal solar fraction, r = 0, A_c = 10 m^2.

| Site | Winter | Spring | Autumn |
| --- | --- | --- | --- |
| Biratnagar | 0.811 | 0.932 | 0.818 |
| Kathmandu | 0.798 | 0.904 | 0.814 |
| Taplejung | 0.721 | 0.875 | 0.856 |

Spring (Mar-Apr) delivers the lowest SEC at every site (0.128 kWh/kg at Biratnagar, 0.182 at Kathmandu, 0.212 at Taplejung) with solar fraction of 0.88-0.93. The combination of high irradiance, warm ambient, and moderate humidity gives the collector a large usable temperature rise. Winter SEC at Kathmandu (0.295) and Biratnagar (0.240) remains well below the annual values because the pre-monsoon and monsoon months (absent from these seasonal windows) drive the annual average up.

## 2.3  Collector area

Table 2.4  Config B SEC and solar fraction vs. collector area, r = 0, annual TMY.

| Site | SEC @ 5 m^2 | SF @ 5 m^2 | SEC @ 10 m^2 | SF @ 10 m^2 |
| --- | --- | --- | --- | --- |
| Biratnagar | - | - | 0.287 | 0.804 |
| Kathmandu | 0.589 | 0.437 | 0.488 | 0.627 |
| Taplejung | - | - | 0.344 | 0.716 |

Only Kathmandu has a simulated 5 m^2 case; doubling to 10 m^2 lifts solar fraction from 0.44 to 0.63 and drops SEC by 17.2 %. The response is sub-linear because the collector saturates once it can supply the full temperature rise on sunny hours: additional area then contributes only during shoulder hours and on cloudy days.

## 2.4  Recirculation

Table 2.5  Config B annual-TMY SEC (kWh/kg) vs. r, A_c = 10 m^2.

| Site | r = 0.0 | r = 0.3 | r = 0.5 | r = 0.7 | r = 0.9 | r = 1.0 |
| --- | --- | --- | --- | --- | --- | --- |
| Biratnagar | 0.287 | 0.568 | 0.618 | 0.431 | 0.423 | 0.423 |
| Kathmandu | 0.488 | 0.392 | 0.416 | 0.460 | 0.469 | 0.477 |
| Taplejung | 0.344 | - | - | - | 0.416 | 0.418 |

Recirculation raises SEC at every site in Config B. At Biratnagar, SEC rises from 0.287 at r = 0 to 0.423 at r = 0.9 (47 % increase). At Kathmandu it rises from 0.488 to a minimum of 0.386 at r = 0.2 then climbs back to 0.469 at r = 0.9. Taplejung shows a 21 % increase at r = 0.9.

The mechanism is the same evaporator-collector interaction across both components. In closed-loop the evaporator cools the mixed air to roughly 12 deg C for dehumidification. The collector then has to reheat this cold stream before the condenser, so a share of the solar heat is spent undoing evaporator cooling rather than displacing compressor work. The solar fraction drops accordingly, from 0.80 at r = 0 at Biratnagar to 0.76 at r = 0.9, and the compressor pulls the remaining lift. Kathmandu's shallow minimum at r = 0.2-0.3 is where the small dehumidification gain on cold, high-RH hours still outweighs the collector penalty; at higher r the penalty dominates.

## 2.5  Takeaways

Open-loop with A_c = 10 m^2 is the design point. Annual SEC reaches 0.287 kWh/kg at Biratnagar, 0.344 at Taplejung, and 0.488 at Kathmandu, with solar fractions 0.63-0.80.

The collector saturates between 5 and 10 m^2 for this thermal load. Larger areas would chase diminishing returns at an increased capital cost.

Closed-loop degrades Config B at every site and season, because the evaporator and the collector end up acting on the same air stream in opposite directions. This observation motivates the cascade arrangements in Section 3, which separate the two components onto different streams.