# Results and Discussion

# Section 2: Config C (Solar Cascade Configurations)

This section examines two alternative solar collector placements that attempt to improve upon Config B’s series arrangement. Config C1 places the solar collector before the evaporator with pre-mixing, while Config C2 uses fresh ambient air through the solar collector before mixing with recirculated exhaust. The results demonstrate that solar placement in the air path has a profound effect on system performance and establish design principles for the more advanced configurations that follow.

## 2.1  Config C: Solar Cascade — Concept and Variants

Config B demonstrated that a solar collector in series with the condenser can reduce SEC by 32–47% compared to the HP-only baseline. A natural follow-up question is: can the solar collector be positioned differently to achieve even greater benefit? Config C explores this by placing the solar collector in a cascade arrangement with the evaporator, where solar energy assists the heat pump cycle rather than directly heating the process air.

Two variants were designed, differing in where recirculated exhaust enters the air path relative to the solar collector:

Config C1: Mix before solar

[DIAGRAM: Config C1 air path]

Figure 2.1a: Config C1 air path. Recirculated exhaust mixes with ambient air before passing through the solar collector, then the evaporator, and finally the condenser.

Air path (r > 0): [r×Exhaust + (1−r)×Ambient] → Solar → Evaporator → Condenser → Chamber

Air path (r = 0): Ambient → Solar → Evaporator → Condenser → Chamber

In C1, the solar collector receives the mixed air (which at r > 0 includes warm exhaust) and heats it further. This solar-heated air then enters the evaporator for dehumidification. The key issue is that the evaporator must cool this warm, solar-heated air back down to the coil surface temperature for moisture removal — undoing much of the solar gain.

Config C2: Mix after solar

[DIAGRAM: Config C2 air path]

Figure 2.1b: Config C2 air path. Ambient air passes through the solar collector first, then mixes with recirculated exhaust before entering the evaporator and condenser.

Air path (r > 0): Ambient → Solar → [Mix solar_out + r×Exhaust] → Evaporator → Condenser → Chamber

Air path (r = 0): Ambient → Solar → Evaporator → Condenser → Chamber

In C2, the solar collector always sees fresh ambient air (cold inlet → high collector efficiency). The solar-heated air is then mixed with recirculated exhaust. This arrangement preserves the solar collector’s efficiency advantage while still allowing recirculation.

At r = 0 (open-loop), both C1 and C2 reduce to the same air path: Ambient → Solar → Evaporator → Condenser → Chamber. However, in this path the evaporator sits between the solar collector and the condenser. In open-loop mode with dry ambient air, the evaporator provides no dehumidification benefit — it simply cools the solar-heated air, which the condenser must then reheat. This fundamental placement issue drives the results presented below.

## 2.2  Config C1: Mix Before Solar — Results

Table 2.1 presents Config C1 performance at r = 0 for two solar collector areas, alongside the Config A and B baselines.

| Config | Location | A_c (m²) | SEC (kWh/kg) | Time (h) | COP | Solar frac. |
| --- | --- | --- | --- | --- | --- | --- |
| A (r=0) | KTM | — | 0.717 | 13.8 | 3.63 | — |
| B (r=0) | KTM | 10 | 0.488 | 13.8 | 3.63 | 0.63 |
| C1 (r=0) | KTM | 5 | 0.980 | 55.9 | 3.77 | 0.60 |
| C1 (r=0) | KTM | 10 | 0.718 | 31.7 | 3.79 | 0.70 |
| A (r=0) | BTN | — | 0.543 | 14.4 | 4.43 | — |
| B (r=0) | BTN | 10 | 0.287 | 14.4 | 4.43 | 0.80 |
| C1 (r=0) | BTN | 5 | 0.900 | 29.9 | 4.03 | 0.52 |
| C1 (r=0) | BTN | 10 | 0.894 | 28.4 | 4.02 | 0.63 |
| A (r=0) | TPJ | — | 0.566 | 14.0 | 3.92 | — |
| B (r=0) | TPJ | 10 | 0.344 | 14.0 | 3.92 | 0.72 |
| C1 (r=0) | TPJ | 10 | 0.637 | 31.7 | 3.92 | 0.71 |

Table 2.1: Config C1 (r = 0) annual-TMY performance compared to Config A and B baselines.

Config C1 at r = 0 produces alarming results. At Kathmandu with 10 m², SEC is 0.718 kWh/kg — nearly identical to Config A (0.717) despite having a solar collector with a solar fraction of 0.70. More critically, the drying time explodes to 31.7 h, more than double the Config A baseline of 13.8 h.

At Biratnagar, the situation is even worse: C1 achieves SEC = 0.894 with a drying time of 28.4 h — compared to Config B’s 0.287 in 14.4 h with the same 10 m² collector.

Why C1 performs so poorly at r = 0

In open-loop C1, the air path is: Ambient → Solar → Evaporator → Condenser → Chamber. The solar collector heats the ambient air (e.g. from 10 °C to 30 °C during midday). This warm, solar-heated air then enters the evaporator, which cools it back to ~12 °C (the coil surface temperature). The solar gain is largely destroyed, and the condenser must reheat from 12 °C to 45 °C regardless of how much solar heating occurred.

Worse, the ambient air in open-loop mode is typically dry enough that no dehumidification occurs at the evaporator — the evaporator provides only sensible cooling. The net effect is that the solar collector heats the air, the evaporator cools it back down (wasting both solar and compressor energy), and the condenser provides the full temperature lift as if the solar collector were absent.

The extended drying time is explained by periods (especially at night) when the evaporator cools the air below the optimal drying temperature. At night, with no solar gain, the air enters the evaporator at ambient temperature and exits at ~12 °C. The condenser heats it to 45 °C as in Config A, but the COP is penalised by the unnecessary evaporator cooling step.

Effect of recirculation on C1

Adding recirculation (r = 0.7) to C1 at Kathmandu improves SEC to 0.689 kWh/kg with a drying time of 15.3 h. The improvement occurs because recirculated exhaust is warm and humid, giving the evaporator meaningful dehumidification work. However, C1 with r = 0.7 still offers no advantage over Config B at r = 0 (0.488 kWh/kg in 13.8 h), which achieves better SEC with a simpler air path.

## 2.3  Config C2: Mix After Solar — Results

Table 2.2 presents Config C2 performance at r = 0 compared to the same baselines.

| Config | Location | A_c (m²) | SEC (kWh/kg) | Time (h) | COP | Solar frac. |
| --- | --- | --- | --- | --- | --- | --- |
| B (r=0) | KTM | 10 | 0.488 | 13.8 | 3.63 | 0.63 |
| C2 (r=0) | KTM | 5 | 0.628 | 13.8 | 4.23 | 0.42 |
| C2 (r=0) | KTM | 10 | 0.561 | 13.8 | 5.05 | 0.59 |
| B (r=0) | BTN | 10 | 0.287 | 14.4 | 4.43 | 0.80 |
| C2 (r=0) | BTN | 5 | 0.452 | 14.4 | 5.55 | 0.59 |
| C2 (r=0) | BTN | 10 | 0.381 | 14.4 | 7.76 | 0.76 |
| B (r=0) | TPJ | 10 | 0.344 | 14.0 | 3.92 | 0.72 |
| C2 (r=0) | TPJ | 10 | 0.424 | 14.0 | 6.21 | 0.67 |

Table 2.2: Config C2 (r = 0) annual-TMY performance compared to Config B.

C2 performs dramatically better than C1. At Kathmandu with 10 m², SEC = 0.561 kWh/kg with a normal drying time of 13.8 h and a notably high COP of 5.05. At Biratnagar, SEC = 0.381 with COP = 7.76.

However, C2 still does not match Config B. At Kathmandu, B achieves SEC = 0.488 versus C2’s 0.561 (13.0% better). At Biratnagar, B achieves 0.287 versus C2’s 0.381 (24.8% better).

How C2’s cascade arrangement works

In C2 at r = 0, the air path is: Ambient → Solar → Evaporator → Condenser → Chamber. Like C1, the evaporator sits after the solar collector. However, C2’s higher COP values (5.05–7.76) reveal what happens: the solar collector pre-heats the air entering the evaporator, which raises the evaporator’s heat source temperature. In the open-loop HP sizing, T_evap = T_source − 10 K. When the source is solar-heated air at 30 °C instead of ambient at 10 °C, T_evap rises to ~20 °C, dramatically reducing the temperature lift and boosting COP.

The trade-off is that this arrangement does not reduce the condenser load directly — the condenser still heats air from the evaporator outlet (~24 °C) to T_set (45 °C). In Config B, the solar collector heats air on the condenser side, directly reducing the temperature lift the condenser must provide. This is why B achieves lower SEC despite C2’s higher COP: the total compressor work is determined by Q_cond / COP, and B reduces Q_cond while C2 only increases COP.

Seasonal performance

| Location | Annual | Autumn | Winter | Spring |
| --- | --- | --- | --- | --- |
| KTM | 0.561 | 0.346 | 0.393 | 0.242 |
| BTN | 0.381 | 0.327 | 0.313 | 0.178 |
| TPJ | 0.424 | 0.287 | 0.458 | 0.256 |

Table 2.3: Config C2 (r = 0, A_c = 10 m²) SEC (kWh/kg) by season.

C2 shows strong seasonal variation. At Biratnagar in spring, SEC drops to 0.178 kWh/kg with a solar fraction of 0.91 — the solar collector provides so much energy to the evaporator source that the compressor barely operates (COP = 101.4). At Kathmandu in spring, SEC = 0.242 with similarly high solar utilisation.

The extremely high COP values in spring (> 100) occur because the solar collector heats the evaporator source air so effectively that the temperature lift approaches zero for some timesteps. While these instantaneous COP values are physical artefacts of the near-zero lift, they reflect a genuine reduction in compressor work: the solar collector is doing most of the thermodynamic lifting.

## 2.4  B vs C1 vs C2: Why Solar Placement Matters

Table 2.4 provides a direct comparison of the three solar configurations at r = 0 with A_c = 10 m².

| Config | Location | SEC (kWh/kg) | Time (h) | COP | Solar frac. | vs Config A |
| --- | --- | --- | --- | --- | --- | --- |
| B | KTM | 0.488 | 13.8 | 3.63 | 0.63 | 32.0% |
| C1 | KTM | 0.718 | 31.7 | 3.79 | 0.70 | -0.2% |
| C2 | KTM | 0.561 | 13.8 | 5.05 | 0.59 | 21.8% |
| B | BTN | 0.287 | 14.4 | 4.43 | 0.80 | 47.1% |
| C1 | BTN | 0.894 | 28.4 | 4.02 | 0.63 | -64.7% |
| C2 | BTN | 0.381 | 14.4 | 7.76 | 0.76 | 29.7% |
| B | TPJ | 0.344 | 14.0 | 3.92 | 0.72 | 39.2% |
| C1 | TPJ | 0.637 | 31.7 | 3.92 | 0.71 | -12.5% |
| C2 | TPJ | 0.424 | 14.0 | 6.21 | 0.67 | 25.1% |

Table 2.4: Comparison of solar configurations (r = 0, A_c = 10 m², annual TMY). ‘vs Config A’ is SEC improvement.

The ranking is consistent across all locations: B > C2 > C1. The explanation lies in where the solar energy enters the thermodynamic cycle:

Config B (Amb → Solar → Cond → Chamber): Solar energy directly reduces the condenser load. The air arrives at the condenser already pre-heated, so Q_cond is smaller. The compressor does less work because Q_cond = Q_evap + W_comp, and a smaller Q_cond means less W_comp.

Config C2 (Amb → Solar → Evap → Cond → Chamber): Solar energy raises the evaporator source temperature, increasing COP but not reducing Q_cond. The compressor does less work per unit of heat transferred, but the total heat to be transferred remains the same.

Config C1 (Mix → Solar → Evap → Cond → Chamber): Solar energy heats air that the evaporator then cools. The solar gain is largely wasted, and the drying time increases dramatically at r = 0 because the evaporator cooling step delays the air’s arrival at the drying temperature.

This ranking has a simple physical interpretation: in a heat pump dryer, the most valuable use of free solar energy is to reduce the condenser load (Config B), because the condenser is the bottleneck for energy consumption. Using solar to boost COP (Config C2) is the next best option. Using solar to heat air that will be cooled immediately (Config C1) is counterproductive.

## 2.5  Lessons for System Design

The Config C results establish three design principles that guide the development of more advanced configurations in subsequent sections:

Solar energy is most effective on the condenser side. Config B’s series arrangement (Solar → Condenser) consistently outperforms the cascade arrangements (Solar → Evaporator) by 13–25% in SEC.

Component ordering in the air path critically affects performance. Placing the evaporator after the solar collector (C1/C2) wastes solar energy on air that must be cooled for dehumidification. This lesson applies equally to the design of heat recovery systems: the heat recovery exchanger should feed the condenser side, not the evaporator side.

COP improvement alone is insufficient. Config C2 achieves COP values 2–3× higher than Config B, yet has worse SEC because the condenser load remains unchanged. In a heat pump dryer, the optimisation target is total compressor work (W_comp = Q_cond / COP − Q_evap / COP), not COP in isolation.

These principles motivate the Config D family (Section 3), which introduces heat recovery to reduce the condenser load — the same mechanism that makes Config B effective, but using waste heat from the exhaust instead of solar energy. This approach works at night and in all weather conditions, addressing Config B’s dependence on solar irradiance.