# Results and Discussion

# Section 2: VPD Bypass Optimisation

This section investigates a controls-level strategy to reduce specific energy consumption (SEC) by bypassing the heat pump’s evaporator or the heat recovery exchanger during periods of low drying potential. The strategy is applied to Config A (closed-loop), D1/D2 (HRX + HP), and E1/E2 (HRX + Solar + HP). A systematic threshold sweep identifies the optimal operating point that balances SEC reduction against drying time increase.

## 2.1  The Late-Drying Inefficiency Problem

Section 1.7 demonstrated that the evaporator’s dehumidification effectiveness declines sharply as drying progresses. In late-stage drying, the product has low moisture content, the exhaust air is only marginally more humid than the inlet, and the evaporator continues cooling the air without meaningful moisture removal. This wastes compressor energy on cooling that must then be undone by the condenser.

The same problem affects the open-loop D and E configurations. In late drying, the chamber exhaust carries very little additional moisture. The heat recovery exchanger (HRX) pre-warms the inlet air using this exhaust, but the benefit diminishes when there is little temperature or humidity difference between streams. Meanwhile, the heat pump continues to heat ambient air from the HRX outlet to T_set, consuming energy that scales with drying time rather than moisture removal.

Two questions arise: (1) Can we detect when the air is no longer being productively used? (2) Can we reduce energy consumption during these periods without unacceptably extending drying time?

## 2.2  VPD Bypass Strategy — Mechanism

Two related bypass strategies were developed, each using a Vapour Pressure Deficit (VPD) criterion to detect when the drying air is under-utilised.

Condenser-direct bypass (Config A, r > 0)

[DIAGRAM: Config A condenser-direct bypass schematic]

Figure 2.1: Config A condenser-direct bypass. Left: normal evaporator path. Right: bypass mode — warm exhaust routes directly to condenser, evaporator runs on ambient as heat source only.

The condenser penalty fraction estimates how much the evaporator improves the air’s drying potential compared to simply recirculating the exhaust directly:

cond_penalty = (VPD_post_evap − VPD_exhaust) / VPD_post_evap

where VPD is evaluated at T_set for both streams. When this fraction falls below a threshold (i.e. the evaporator barely improves drying potential), the system switches to condenser-direct mode: warm exhaust (~43 °C) is routed directly to the condenser, which needs only ~2 K of reheating (Q_cond ≈ 0.2 kW vs ~2 kW in normal mode). The evaporator switches to drawing heat from ambient air (sensible heat source only, no dehumidification).

Humidity accumulates in the closed loop during bypass. When the condenser penalty rises above 3× the threshold, the system switches back to normal evaporator mode for dehumidification. A minimum dwell time of 300 s prevents rapid oscillation.

Exhaust bypass (Configs D/E, r = 0)

[DIAGRAM: Config D/E exhaust bypass schematic]

Figure 2.2: Config D exhaust bypass. Left: normal HRX path. Right: bypass mode — warm exhaust routes directly to condenser, bypassing the HRX.

For open-loop D and E configurations, the VPD utilisation metric measures the fraction of the inlet air’s drying potential that is consumed by the chamber:

utilisation = 1 − VPD_exhaust / VPD_inlet

High utilisation indicates the air is picking up significant moisture (early drying). Low utilisation means the air passes through nearly unchanged (late drying). When utilisation falls below the threshold, the warm exhaust (~40 °C) is routed directly to the condenser, bypassing the HRX. The condenser provides a small temperature lift (~5 K), dramatically reducing compressor work. When utilisation rises above 3× the threshold, the system reverts to normal HRX operation. Dwell time is 600 s.

## 2.3  Config A (r = 0.9): Condenser-Direct Bypass

A sweep of the condenser penalty threshold was conducted for Config A at r = 0.9 across both locations. Table 2.1 presents SEC and drying time as a function of threshold.

| Threshold | KTM SEC | KTM time (h) | KTM ΔSEC | BTN SEC | BTN time (h) | BTN ΔSEC |
| --- | --- | --- | --- | --- | --- | --- |
| OFF | 0.669 | 14.9 | — | 0.753 | 14.6 | — |
| 0.01 | 0.669 | 14.9 | 0% | 0.753 | 14.6 | 0% |
| 0.02 | 0.650 | 14.9 | −2.8% | 0.753 | 14.6 | 0% |
| 0.05 | 0.536 | 15.6 | −19.7% | 0.617 | 15.3 | −18.1% |
| 0.10 | 0.365 | 19.1 | −45.4% | 0.395 | 19.8 | −47.5% |
| 0.15 | 0.331 | 30.9 | −50.5% | 0.336 | 28.8 | −55.4% |
| 0.20 | 0.349 | 48.5 | −47.9% | 0.303 | 30.9 | −59.7% |
| 0.30 | 0.383 | 72.0 | −42.7% | 0.364 | 72.0 | −51.6% |

Table 2.1: Config A (r = 0.9) condenser-direct bypass sweep. ΔSEC is improvement relative to no bypass.

The results reveal a dramatic trade-off between SEC and drying time. At threshold = 0.10, SEC is reduced by 45–48% with a moderate drying time increase from ~15 h to ~19 h (+30%). At threshold = 0.15, SEC improves by 50–55% but drying time doubles to 29–31 h. Above 0.20, the system spends too long in bypass mode, humidity accumulates, and the simulation approaches the 72 h limit.

The mechanism is intuitive: during bypass, the compressor work drops from ~2 kW (heating air from 12 °C to 45 °C) to ~0.2 kW (heating from 43 °C to 45 °C). However, no dehumidification occurs, so chamber RH rises and the drying rate slows. The system oscillates between bypass (low energy, slow drying) and normal (high energy, fast drying) modes, with the threshold determining the duty cycle.

For a practical system where drying time is constrained to ~16 h (one operational day), threshold = 0.05 provides the best balance: 19–20% SEC reduction with only ~1 h of additional drying time.

## 2.4  Configs D1 and D2: Exhaust Bypass Optimisation

Tables 2.2 and 2.3 present the VPD utilisation threshold sweep for Configs D1 and D2 respectively.

| Threshold | KTM SEC | KTM time (h) | KTM ΔSEC | BTN SEC | BTN time (h) | BTN ΔSEC |
| --- | --- | --- | --- | --- | --- | --- |
| OFF | 0.365 | 13.8 | — | 0.293 | 14.4 | — |
| 0.01 | 0.331 | 14.0 | −9.3% | 0.264 | 14.6 | −10.2% |
| 0.02 | 0.314 | 14.2 | −14.1% | 0.253 | 14.8 | −13.7% |
| 0.05 | 0.287 | 15.1 | −21.4% | 0.238 | 15.8 | −19.1% |
| 0.10 | 0.268 | 17.2 | −26.7% | 0.225 | 18.0 | −23.2% |
| 0.15 | 0.257 | 20.5 | −29.6% | 0.221 | 22.3 | −24.6% |
| 0.20 | 0.258 | 25.3 | −29.3% | 0.229 | 30.3 | −22.0% |
| 0.30 | 0.337 | 72.0 | −7.8% | 0.298 | 72.0 | −* |

Table 2.2: Config D1 (HRX, ambient evaporator) VPD exhaust bypass sweep.

| Threshold | KTM SEC | KTM time (h) | KTM ΔSEC | BTN SEC | BTN time (h) | BTN ΔSEC |
| --- | --- | --- | --- | --- | --- | --- |
| OFF | 0.354 | 13.8 | — | 0.282 | 14.4 | — |
| 0.01 | 0.327 | 14.0 | −7.7% | 0.258 | 14.6 | −8.5% |
| 0.02 | 0.311 | 14.2 | −12.0% | 0.250 | 14.8 | −11.5% |
| 0.05 | 0.289 | 15.1 | −18.4% | 0.239 | 15.8 | −15.4% |
| 0.10 | 0.273 | 17.2 | −22.9% | 0.231 | 18.0 | −17.9% |
| 0.15 | 0.267 | 20.5 | −24.5% | 0.232 | 22.3 | −17.6% |
| 0.20 | 0.273 | 25.3 | −22.9% | 0.247 | 30.3 | −12.4% |
| 0.30 | 0.389 | 72.0 | +9.8% | 0.348 | 72.0 | +23.3% |

Table 2.3: Config D2 (HRX, dynamic ambient compensation evaporator) VPD exhaust bypass sweep.

Both D1 and D2 show substantial SEC improvement from exhaust bypass, with diminishing returns above threshold = 0.10. The pattern is consistent across locations: at threshold = 0.05, SEC improves by 15–21% with only 1–2 h of additional drying time (from ~14 h to ~15–16 h). At threshold = 0.10, improvement reaches 18–27% but drying time extends to 17–18 h.

D1 and D2 respond similarly to the bypass, with D2 showing slightly smaller relative gains. This is expected: D2’s dynamic ambient compensation at the evaporator already improves the heat pump’s COP (by using cooled exhaust rather than raw ambient at the evaporator), so the marginal benefit of bypassing is smaller.

At threshold ≥ 0.30, the simulation exceeds 72 h for both configs, indicating that the system spends nearly all its time in bypass mode and drying rate is insufficient to reach the target moisture content.

## 2.5  Configs E1 and E2: Exhaust Bypass Optimisation

Tables 2.4 and 2.5 present the VPD bypass sweep for the solar-assisted HRX configurations E1 and E2 (A_c = 10 m²).

| Threshold | KTM SEC | KTM time (h) | KTM ΔSEC | BTN SEC | BTN time (h) | BTN ΔSEC |
| --- | --- | --- | --- | --- | --- | --- |
| OFF | 0.220 | 13.8 | — | 0.141 | 14.4 | — |
| 0.01 | 0.186 | 14.0 | −15.5% | 0.111 | 14.6 | −21.2% |
| 0.02 | 0.169 | 14.2 | −23.2% | 0.103 | 14.8 | −26.9% |
| 0.05 | 0.152 | 15.1 | −30.9% | 0.101 | 15.8 | −28.5% |
| 0.10 | 0.155 | 17.2 | −29.7% | 0.106 | 18.0 | −25.0% |
| 0.15 | 0.151 | 20.5 | −31.4% | 0.110 | 22.3 | −21.7% |
| 0.20 | 0.150 | 25.3 | −31.6% | 0.114 | 30.3 | −19.4% |
| 0.30 | 0.235 | 72.0 | +6.5% | 0.184 | 72.0 | +30.8% |

Table 2.4: Config E1 (HRX + Solar, ambient evaporator, A_c = 10 m²) VPD exhaust bypass sweep.

| Threshold | KTM SEC | KTM time (h) | KTM ΔSEC | BTN SEC | BTN time (h) | BTN ΔSEC |
| --- | --- | --- | --- | --- | --- | --- |
| OFF | 0.197 | 13.8 | — | 0.129 | 14.4 | — |
| 0.01 | 0.170 | 14.0 | −13.7% | 0.105 | 14.6 | −18.6% |
| 0.02 | 0.157 | 14.2 | −20.3% | 0.099 | 14.8 | −23.6% |
| 0.05 | 0.144 | 15.1 | −26.9% | 0.097 | 15.8 | −24.7% |
| 0.10 | 0.147 | 17.2 | −25.2% | 0.102 | 18.0 | −20.7% |
| 0.15 | 0.146 | 20.5 | −25.9% | 0.108 | 22.3 | −16.5% |
| 0.20 | 0.147 | 25.3 | −25.6% | 0.112 | 30.3 | −13.6% |
| 0.30 | 0.232 | 72.0 | +17.8% | 0.182 | 72.0 | +41.4% |

Table 2.5: Config E2 (HRX + Solar, dynamic evaporator, A_c = 10 m²) VPD exhaust bypass sweep.

The E configurations show the sharpest knee in the SEC–time curve at threshold = 0.05. At this threshold:

E1: SEC reduces by 29–31% (KTM: 0.220 → 0.152; BTN: 0.141 → 0.101 kWh/kg) with only 1–2 h extra drying time.

E2: SEC reduces by 25–27% (KTM: 0.197 → 0.144; BTN: 0.129 → 0.097 kWh/kg) with only 1–2 h extra drying time.

Above threshold = 0.05, the E configs show diminishing returns with rapidly increasing drying time. This is because the solar collector already provides substantial free energy; during bypass, the system loses access to the HRX’s heat recovery while the solar benefit alone is insufficient to maintain the drying rate. The optimal threshold for E configs (0.05) is lower than for D configs (0.10–0.15) because the solar energy makes the system more sensitive to operational disruptions.

E2 at Biratnagar with threshold = 0.05 achieves SEC = 0.097 kWh/kg, which represents the lowest specific energy consumption across all configurations and conditions tested in this study. This is 86% lower than the Config A baseline (0.717 kWh/kg at r = 0) and 82% lower than Config A with recirculation (0.543 kWh/kg at r = 0).

## 2.6  Optimal Threshold Selection: Balancing SEC and Drying Time

Table 2.6 summarises the recommended thresholds, selected to achieve substantial SEC reduction while keeping drying time within practical limits (no more than ~2 h increase over the baseline ~14 h cycle).

| Config | Bypass type | Recommended threshold | KTM SEC (vs baseline) | BTN SEC (vs baseline) | Time increase |
| --- | --- | --- | --- | --- | --- |
| A (r=0.9) | Cond-direct | 0.05 | 0.536 (−20%) | 0.617 (−18%) | +1 h |
| D1 | Exhaust | 0.05 | 0.287 (−21%) | 0.238 (−19%) | +1–2 h |
| D2 | Exhaust | 0.05 | 0.289 (−18%) | 0.239 (−15%) | +1–2 h |
| E1 (10 m²) | Exhaust | 0.05 | 0.152 (−31%) | 0.101 (−29%) | +1–2 h |
| E2 (10 m²) | Exhaust | 0.05 | 0.144 (−27%) | 0.097 (−25%) | +1–2 h |

Table 2.6: Recommended VPD bypass thresholds balancing SEC and drying time.

A uniform threshold of 0.05 is recommended across all configurations. While D configs show marginally better SEC at 0.10–0.15, the drying time penalty (3–6 h additional) makes these impractical for daily batch operations where the dryer must complete a cycle within one working period.

For Config A, the threshold operates on a different metric (condenser penalty rather than VPD utilisation), but the 0.05 value provides a comparable balance. Higher thresholds (0.10–0.15) are available for applications where energy cost is critical and a longer drying period is acceptable.

## 2.7  Summary: VPD Bypass Impact on System Performance

Table 2.7 provides a comprehensive comparison of all configurations with and without VPD bypass at the recommended threshold of 0.05, alongside the Config A open-loop baseline.

| Config | VPD | KTM SEC | BTN SEC | KTM time (h) | BTN time (h) |
| --- | --- | --- | --- | --- | --- |
| A (r=0) | OFF | 0.717 | 0.543 | 13.8 | 14.4 |
| A (r=0.9) | OFF | 0.669 | 0.753 | 14.9 | 14.6 |
| A (r=0.9) | 0.05 | 0.536 | 0.617 | 15.6 | 15.3 |
| D1 | OFF | 0.365 | 0.293 | 13.8 | 14.4 |
| D1 | 0.05 | 0.287 | 0.238 | 15.1 | 15.8 |
| D2 | OFF | 0.354 | 0.282 | 13.8 | 14.4 |
| D2 | 0.05 | 0.289 | 0.239 | 15.1 | 15.8 |
| E1 (10 m²) | OFF | 0.220 | 0.141 | 13.8 | 14.4 |
| E1 (10 m²) | 0.05 | 0.152 | 0.101 | 15.1 | 15.8 |
| E2 (10 m²) | OFF | 0.197 | 0.129 | 13.8 | 14.4 |
| E2 (10 m²) | 0.05 | 0.144 | 0.097 | 15.1 | 15.8 |

Table 2.7: Summary of all configurations with and without VPD bypass (threshold = 0.05). SEC in kWh/kg.

Key findings from the VPD bypass optimisation:

VPD bypass is universally beneficial: every configuration tested shows SEC improvement at threshold = 0.05, with 15–31% reduction depending on configuration and location.

The benefit is greatest for E configs (25–31%) because the solar collector reduces the HP’s share of total energy, making the bypass’s compressor savings a larger fraction of the total.

Drying time increases by only 1–2 h at threshold = 0.05, from ~14 h to ~15–16 h — well within a single daily batch cycle.

The SEC–time trade-off is non-linear: doubling the threshold from 0.05 to 0.10 typically gains only 5–8% additional SEC improvement but adds 2–3 h of drying time. Tripling to 0.15 adds minimal SEC gain but extends drying by 5–7 h.

At aggressive thresholds (≥0.30), all configurations fail to complete drying within 72 h, confirming that the bypass must be used judiciously.

The best absolute performance is E2 at Biratnagar with VPD bypass: SEC = 0.097 kWh/kg. This represents a 13.5× reduction in specific energy from the worst-case scenario (Config A, r = 0.9, BTN, SEC = 0.753 kWh/kg with no bypass) and a 5.6× reduction from the simple open-loop baseline (Config A, r = 0, BTN, SEC = 0.543 kWh/kg).

The VPD bypass strategy is a controls-level optimisation that requires no additional hardware — only a humidity sensor at the chamber exhaust and control logic to actuate a damper. Combined with the hardware-level improvements of heat recovery (D configs) and solar integration (E configs), it enables SEC values below 0.10 kWh/kg at favourable locations, approaching the theoretical minimum for heat-pump-assisted convective drying.